SA WG2 Meeting #S2-165	S2-2409589

14 - 18 October, 2024, Hyderabad, India

3GPP TSG-CT WG4 Meeting #124	C4-243492

# Maastricht, Netherlands; 19th – 23rd August 2024

# Title:	LS on Configuration of Slice Usage Control Information

# Response to:

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	CT4

To:	SA2

Cc:	CT1, CT3

Contact Person:

Name:	Zhijun Li

Tel. Number:

E-mail Address:	li.zhijun3@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

To support network slice usage control, CT4 defined the configuration of slice deregistration inactivity timer and PDU Session inactivity timer in UE subscription data in UDM. Such slice related timer is access type independent, see clauses 6.1.6.2.8 and 6.1.6.2.38 of 3GPP TS 29.503.

CT4 also observed that clause 5.15.15.2 and 5.15.15.3 of 3GPP TS 23.501 specifies that the AMF starts the slice deregistration inactivity timer per S-NSSAI per access type. Furthermore, in 3GPP TS 23.501 clause 5.15.15.3 there is an NOTE saying that for MA-PDU Session the PDU Session inactivity timer is access type independent, while it is not clear for SA-PDU Session whether the PDU Session inactivity timer is access type independent or not.

CT4 discussed how slice usage control information configured in the UE subscription is used by the AMF to start per S-NSSAI per access type slice deregistration inactivity timer, and by SMF to start PDU Session inactivity timer and would like to ask SA2 to answer the following questions:

Q1: For SA-PDU Session, is the PDU Session inactivity timer independent of access type?

Q2: To support per S-NSSAI per access type slice usage control defined in 3GPP TS 23.501, does it need to configure different timer values to different access type in UDM?

Q3: If the answer of Q2 is no, what is the exact AMF/SMF behaviour of performing per S-NSSAI per access type network slice usage control (i.e. starting slice deregistration inactivity timer or PDU Session inactivity timer)? Does the AMF or SMF assign same timer value to 3GPP access and non-3GPP access?

2. Actions:

To SA2 group.

ACTION: 	CT4 kindly asks SA2 group to answer the above questions and if necessary provide clarification in their specifications accordingly.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm