SA WG2 Meeting #S2-165	S2-2409592

14 - 18 October, 2024, Hyderabad, India

3GPP TSG RAN WG1 Meeting #118		R1-2407364

Maastricht, Netherlands, 19th – 23rd August 2024

Title:	Reply LS on Clarification of requirements for Ambient IoT

Response to:	R1-2405796 / S2-2407231

Release:	Rel-19

Work Item:	FS_AmbientIoT, FS_Ambient_IoT_solutions

Source:	RAN1

To:	SA2

Cc:	SA1, SA3, RAN2, RAN3

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 has discussed the questions raised in the LS from SA2 regarding the feasibility of supporting non-volatile memory (NVM) in Ambient IoT (A-IoT) devices:

SA2 have discussed different aspects with respect to non-volatile memory: - whether or not an Ambient IoT device can incorporate a non-volatile memory in the device design i.e., include a non-volatile memory in the Bill-of-Materials (BoM). - whether an Ambient IoT device will be able to update its non-volatile memory based on its energy status and energy storage capabilities at some point after receiving a trigger from the Reader. SA2 has not reached any common understanding. While there is no consensus in SA2 whether it is in RAN1 scope to determine devices' level of capability to support non-volatile memory; SA2 would like to ask the following question to RAN1: Question 5: SA2 would like to ask RAN1 if RAN1 can provide feedback on the non-volatile memory aspects listed above.

RAN1 would like to provide the following answers to SA2’s questions:

Whether or not an Ambient IoT device can incorporate a non-volatile memory in the device design i.e., include a non-volatile memory in the Bill-of-Materials (BoM)

RAN1 is studying several different A-IoT device architectures, which all are assumed to include a memory block described as follows:

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

Therefore, it can be assumed that an A-IoT device can incorporate an NVM in the device design, i.e., include an NVM in the BoM.

Whether an Ambient IoT device will be able to update its non-volatile memory based on its energy status and energy storage capabilities at some point after receiving a trigger from the Reader

RAN1 would like to provide the following answer:

- An Ambient IoT device will be able to update its non-volatile memory at some point after receiving a trigger from the Reader. Writing may not always be possible at all times.

- The power consumption during writing to the NVM is higher than during reading.

- RAN1 thinks that frequent or recurring writing to NVM should be avoided.

- RAN1 has not discussed energy status and energy storage capabilities in relation to an Ambient IoT device ability to update its non-volatile memory.

# 2	Actions

To SA2:

ACTION: 	RAN1 respectfully asks SA2 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #118-bis	14th – 18th October 2024		Hefei, CN

TSG-RAN WG1 Meeting #119	18th – 22nd November 2024		Orlando, US