3GPP TSG-SA WG2 Meeting #165	S2-2409683

# Hyderabad, India, 14 - 18 October, 2024 		(revision of S2-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0571 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data collection for AI/ML based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R19 FS_AIML_CN has concluded KI#1 for LCS supporting Direct AI/ML based positioning in TR 23.700-84. As for data collection, the LMF is enhanced to perform data collection based on the request from NWDAF or the internal trigger. For the following EN in TS 23.273, a new subclause is elaborated for NWDAF-triggered data collection. The NWDAF interacts with LMF reusing the Ngmlc service. “Editor's note: The procedure of NWDAF containing MTLF collects input data for LMF-based AI/ML Positioning ML model training is FFS.” For the following EN, the AMF can select the UEs considering AOI and the UE privacy related to location data for ML model training, which is beyond the serices of the current Nafm_Eventexposure_Subscribe service. A new service for location data collection for ML model training is needed “Editor's note: Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS.” Considering the current LCS privacy check is initiated by AMF (i.e., NAS Location notification invoke), for the UE privacy check for location data collection, the similar mechanism is reused (i.e., AMF checks). Besides, current LCS privacy settings only care about whether UE is allowed to be located, i.e., location measurements are only used for calculating the its location result, but other purposes of location measurement is not covered. Thus, LCS privacy profile is not sufficient to support location data collection for AI positioning model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new subclause is introduced for NWDAF-triggered data collection for ML model training; - The detail of UE privacy check for data collection for ML model training is added (i.e., AMF checks). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data collection for LMF-based AI/ML positioning triggered by NWDAF and how to perform UE privacy check are not supported in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18, 5.18.1, 6.x(new), 6.x.1(new), 6.x.2(new), 6.x.3(new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.18	Support for UE positioning based on a ML Model at the LMF

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 5.18.1.

Editor's note:	The procedure of LMF collects input data from NG-RAN for ML model training is FFS.

LMF may also request a ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF. LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37]. LMF requests ML model training for LMF-based AI/ML Positioning with ML Model Filter Information, which may indicate the area of interest. NWDAF containing MTLF collects input data to perform the ML model training and model provision to LMF as described in clause 6.2A of TS 23.288 [37].

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

Editor's note:	The Performance monitoring procedures are FFS.

# * * * * Next change * * * *

5.18.1	Void

# * * * * Next change * * * *

6.x	Procedure of data collection for training LMF-based AI/ML positioning model

6.x.1	General

This clause specifies procedures for the data collection by LMF to train the ML model for LMF-based AI//ML positioning internally or in NWDAF containing MTLF. The data collection can be triggered by the LMF or the NWDAF containing MTLF.

Subclause 6.x.2 describes the procedure of LMF data collection for LMF-based AI/ML positioning based on the internal trigger.

Subclause 6.x.3 describes the procedure of LMF data collection for LMF-based AI/ML positioning based on NWDAF containing MTLF request.

# * * * * Next change * * * *

6.x.2	Data collection initiated by LMF to train the AI/ML based positioning model

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Editor's note:	The procedure to collect the input data for AI/ML based positioning from NG-RAN is FFS.

Figure 6.x.2-1: Data collection initiated by LMF to train the AI/ML based positioning model using UE measurements

1.	The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs that are willing to report location data within an area of interest using Namf_Location_AIPosData_Subscribe service operation. The service operation includes the area of interest, an indication that indicates the data is collected to train ML model for LMF-based AI/ML based positioning, data source type (e.g. RAN, UE, PRU).

Editor's note:	Whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS.

2.	For each UE located in the area of interest, the AMF sends the Nudm_SDM_Get request message towards the UDM to get the enhanced LCS privacy profile (see table 7.1-1) of the UE identified by its GPSI or SUPI.

3.	The UDM returns the enhanced LCS Privacy profile in the Nudm_SDM_Get response message. The AMF checks the privacy setting for data collection. If the result of the privacy check indicating that UE is not willing to report data for ML model training, steps 4-5 are skipped and AMF shall not include this UE identifier to the LMF. If the result of the privacy check indicating that UE is willing to report data for ML model training without notification, steps 4-5 are skipped and AMF can include this UE identifier to the LMF.

4.	For each UE located in the area of interest, if the result of privacy check indicating that the notification or notification with privacy verification is needed, a notification invoke message is sent to the UE, including an indication that indicates the data is collected for ML model training, and whether privacy verification is required.

5.	The UE notifies the UE user of the location data collection request and, if privacy verification was requested, waits for the user to grant or deny permission. The UE then returns a notification result to the AMF indicating whether permission is granted or denied for the current location data collection request, and optionally, a time period when UE is willing to report location data for ML model training. If the UE user does not respond after a predetermined time period, the AMF shall infer a "no response" condition. The AMF shall not include this UE identifier to the LMF, if either UE user denies permission or there is no response received but the "no response" condition indicating UE is not willing to report data for ML model training.

6.	The AMF send Namf_Location_AIPosData_Notify to LMF including a list of UE ID (i.e., GPSI or SUPI) that are willing to report location data for ML model training, and optionally the time period for each UE during which UE is willing to report location data.

Editor's note:	Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS.

7.	For each UE that provides the enhanced LCS privacy profile to data collection for model training and willing to report location data for ML model training, the LMF initiates a request for input data.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the permission for data collection for model training is revoked, the LMF stops to collect the input data from the UE to train the AI/ML based positioning model.

# * * * * Next change * * * *

6.x.3	Data collection initiated by NWDAF containing MTLF to train the AI/ML based positioning model

The figure 6.x.3-1 shows how LMF performs data collection for LMF-based AI/ML positioning triggered by NWDAF containing MTLF.

Editor's note:	The procedure to collect the input data for LMF-based AI/ML positioning from NG-RAN is FFS.

Figure 6.x.3-1: Data collection initiated by NWDAF to train the AI/ML based positioning model

0.	Based on the ML model provision subscription/request from the service consumer (e.g., LMF) or internal trigger, the NWDAF may determine to initiate the data collection to train the ML model for AI/ML based positioning.

1.	The NWDAF invokes the Ngmlc_Location_AIPosData_Subscribe service operation to the GMLC to collect data within the area of interest. The service operation includes the NWDAF ID, Notification Target Address (+Notification Correlation ID) pointing to the NWDAF itself, the area of interest, an indication that indicates the data is collected to train ML model for LMF-based AI/ML positioning, data source type (e.g. RAN, UE, PRU) and time window for data collection.

2.	The GMLC determines AMF based on the area of interest received in step 1. The GMLC invokes Namf_Location_AIPosData_Subscribe service operation to the AMF including the NWDAF ID, Notification Target Address (+Notification Correlation ID) and all other information received in step 1.

3.	The AMF selects one or more LMF(s) considering the area of interest, LMF Service Area, the LMF capabilities supporting AI positioning and data collection as defined in clause 5.1. The AMF may either query the NRF or based on AMF local configuration to discover and select a proper LMF.

4.	After receiving the request in step 2, the AMF determines the UEs for data collection considering the area of interest, LCS privacy profile and local configuration. In order to deliver to LMF a list of UE that are willing to report location data for ML model training, the AMF may perform LCS privacy check for the UE located in the area of interest as described for step 2-5 in clause 6.x.2.

The AMF invokes the Nlmf_Location_AIPosData_Subscribe service operation to the LMF(s) selected in step 3 to initiate data collection. The service operation includes an indication that data collection is needed for training ML model, a list of UE ID (i.e., GPSI or SUPI) that are willing to report location data for ML model training, and all other information received in step 2. It may also include the time period for each UE during which UE is willing to report location data.

Editor's note:	Whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS.

5A.	For each UE that provides the enhanced LCS privacy profile to data collection for model training and willing to report location data for ML model training, the LMF initiates a request for input data.

Editor’s Note:	The sources of data, specified data types and protocol used for model training, inference and model performance monitoring for AI/ML based positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs.

5B.		After data collection from UE, PRU or NG-RAN, the LMF may store the data in the ADRF, using the Nadrf_DataManagement_StorageRequest service operation as defined in TS 23.288[37]. If the data is already stored in the ADRF, the LMF determines the ADRF ID plus DataSetTag or ADRF ID plus Storage Transaction ID.

6.	The LMF invokes the Nlmf_Location_AIPosData_Notify service operation towards the NWDAF based on Notification Target Address or NWDAF ID received in step 4. The service operation includes either the requested data, or the ADRF ID plus DataSetTag or ADRF ID plus Storage Transaction ID if the LMF stores the requested positioning data in step 5B.

When the ADRF ID plus DataSetTag or ADRF ID plus Storage Transaction ID are received, the NWDAF could obtain the data by invoking Nadrf_DataManagement_RetrievalRequest as defined in TS 23.288[37].

# * * * * Next change * * * *

7.1	UDM

For each UE subscriber the UDM stores LCS related data as part of the Subscriber Data Management (SDM) service as defined in clause 5.2.3.3.1 of TS 23.502 [19].

The privacy profile data is defined in table 7.1-1 containing data for the privacy classes for which location of the target UE is permitted. For the meaning of each LCS privacy profile data type and included data, refer to clause 5.4.2.

Table 7.1-1: LCS privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Location Privacy Indication | M O | Indication of one of the following mutually exclusive global settings: - Location is disallowed. - Location is allowed (default). Time period when the Location Privacy Indication is valid. |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any LCS client or AF not in the external LCS client list or otherwise identified for the Call/session Unrelated Class, the following data may be present: - One of the following mutually exclusive options: - Location not allowed (default case) - Location allowed with notification. - Location with notification and privacy verification; location allowed if no response. - Location with notification and privacy verification; location restricted if no response. - Time period when positioning is allowed. - Geographical area where positioning is allowed. - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC. External LCS client list: a list of zero or more LCS clients, AFs and LCS Client groups with the following data for each entry: - One of the following mutually exclusive options: - Location allowed without notification (default case). - Location allowed with notification. - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed Service types list: a list of one or more service types for which the LCS client is allowed to locate the particular UE. The possible service types are defined in TS 22.071 [2]. The following data may be present for each service type in the list: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response. - Location with notification and privacy verification; location restricted if no response. - Time period when positioning is allowed. - Geographical area where positioning is allowed. - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC. |
| PLMN Operator Class | O | LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information. - O&M LCS client in the HPLMN. - O&M LCS client in the VPLMN. - LCS client recording anonymous location information. - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE. - NWDAF in the HPLMN (when the UE is currently being served by the HPLMN). - NWDAF in the VPLMN. |
| User Plane Connection between UE and LCS Client or AF | O | Indication of one of the following mutually exclusive global settings: - UE is allowed to report periodic or triggered location events via user plane to an LCS Client or AF - UE is not allowed to report periodic or triggered location events via user plane to an LCS Client or AF (default). |
| Event report expected area | O | Presents a geographical area generated by UE, which is used by GMLC to determine event report allowed area for the UE. |
| Area usage indication | O | Indication of one of the following mutually exclusive global settings on using event report allowed area: - Inside reporting (default). - Outside reporting. |
| GMLC address list | O | Addresses of GMLC located in local network(s) which are allowed to be used for the UE positioning. The GMLC address is used to identify the local network for verification of location service in PNI-NPN. |
| Location data collection class | M O O | For the location training data collector (i.e., LMF), the following data may be present: - One of the following mutually exclusive options: - UE is not willing to report location data for ML model training (default case). - UE is willing to report location data for ML model training without notification. - UE is willing to report location data for ML model training with notification. - notification and privacy verification needed; UE is willing to report location data for ML model training if no response. - notification and privacy verification needed; UE is not willing to report location data for ML model training if no response. - Time period when the UE is willing to report location data for ML model training. |

The Mobile Originating data is defined in table 7.1-2 containing the LCS MO-LR services that a UE can receive.

Table 7.1-2: LCS Mobile Originated data for a UE Subscriber

| MO-LR Data | Presence | UDM data |
| --- | --- | --- |
| Mobile Originated data | M | List of MO-LR services allowed for a UE subscriber: - Basic Self Location (UE can receive its own location). - Autonomous Self Location (UE can receive location assistance data). - Transfer to Third Party. |
| List of Assistance Data Types for MO-LR | O | A list of one or more types of location assistance data that may be provided to the UE in the MO-LR procedure. |

The LCS broadcasting data is defined in table 7.1-3 containing a list of assistance data types for which ciphering keys should be provided to the UE if requested by the UE.

Table 7.1-3: LCS broadcasting data for a UE subscriber

| Broadcasting Data | Presence | Description |
| --- | --- | --- |
| List of Assistance Data Types | O | A list of one or more types of location assistance data for which ciphering keys should be provided to the UE if requested by the UE when the assistance data is broadcast using ciphering. |

Other UE LCS subscriber data is defined in table 7.1-4 containing UE's subscription data related with LCS.

Table 7.1-4: UE LCS subscriber data

| UE LCS subscriber data | Presence | Description |
| --- | --- | --- |
| PRU indication | O | When present, it is used to indicate that the UE is allowed to serve as a PRU. |
| LPHAP indication | O | When present, it indicates a UE requires low power and high accuracy positioning. |
| LMF ID | O | An LMF ID which presents an LMF deployed in local network to support location service for the UE. |
| Indication of user plane positioning between UE and LMF | O | When present, it indicates that the UE is allowed to use the user plane positioning between UE and LMF. |

# * * * * End of changes * * * *