SA2 Meeting #165	S2-2409634

# Hyderabad, India, 14 - 18 October 2024

Source:	ZTE

Title:	Discussion and way forward of UE side data collection

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This discussion paper presents 4 different options of UE side training data collection with some observations. A related Reply LS is in S2-240XXXX.

Discussion

In LS in from RAN WG(RP-242389), the following 4 options are given for UE side data collection:

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

The followings are the agreed way forward of RAN#105.

RAN#105 way forward and requirements

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

Some initial observations are listed below:

Observation 1: Option 3 is out of SA2 scope, So no needs to discuss the feasibility of Option 3.The 		 controllability  of options is up to SA3.

Observation 2:  Option 1a is the simplest and fastest way for UE data collection. The OTT server can collect data from UE directly via UP at any time.

Observation 3: Option 2 uses NAS layer for UE training data collection, which may cause signalling congestion in control plan Network Functions(e.g. AMF) if the collection is frequent with large size of training data. However, this problem can be avoided by collecting UE training data in spare time(e.g. during night).

Observation 4: For the FFS/study if and how to handle non-standardized data (i.e. partial visibility), if we put the training data in the container for transfer then it is difficult for operator to decode and monitor whether the training data collected is legal. So the training data should be fully controllable by MNO.

Observation 5: It is not clear about the differences between option 1b and options 1a/2. If the Server for data collection refers to the AF inside CN, then it is same with option 2. If the Server for data collection refers to OTT server, then it is same with option 1a. Some clarifications from RAN2 are needed.

Observation 6:  The UP description of option 1b/2 is not clear. Dose it refer to use GTP-U header for training data collection or using DCAF(EVEX) framework?  Some clarifications from RAN2 are needed.

Observation 7: If we use PDU session for training data transfer, it is not clear about how to build or select appropriate PDU session with associated URSP rule, IP address, S-NSSAI.

Proposals of way forward

Proposal 1(The most preferred way): Using option 1a for UE training data collection.

Proposal 2(A compromised way):  Using option 2 for non-frequent and small size UE training data collection in spare time AND the training data is to be standardized and to be transferred without any container.