3GPP TSG-SA2 Meeting #165	S2-2409733

# 14 – 18 October, 2024, Hyderabad, IN

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1208 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the general inference procedure for vertical federated learning to resolve ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The AF initiate VFL case is added into the general inference procedure to resolve the fisrt EN. For both NWDAF and AF initiate VFL case, it should be AnLF as a VFL consumer. The logic is quite same, using one procedure to capture all the case will make the logic clearer. The Editor's note: How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS. is removed since clause 6.2H.2.1 describes the VFL discovery. The UE ID(s) is added in the request send from the VFL server to the VFL client to indicate the VFL client which UE will be used as the sample to let VFL client collect the data and perform the VFL inference. The Editor's note: Whether the VFL model correlation ID is needed or using the model ID is FFS. is removed. The local model in each client can be different. Model ID is designed to identiy the sepcfic model. The model ID is same means the model should have the same topology. However, in the VFL case, the topology for each client local model can be different. It is not suitable to use the same model ID for every client. The VFL model correlation ID is perferred to indicate those local model in each client should used for the same VFL task e.g. for the same analytcis ID. The Editor's note: Whether the VFL client needs to retrieval of stored local models is FFS. is removed. Since performing the VFL training and VFL inference client is the same. The VFL client can store the model locally after finishing the VFL training and using the same local model to perform the VFL inference later based on the VFL model correlation ID. The VFL model correlation ID is added when the VFL client send the VFL inference result to the VFL server. VFL model correlation ID is used to let VFL server know which inference result from which client need to be aggregated to generate the VFL inference result. NEF behaviour is added when the untrusted AF is the VFL server to collect local intermediate inference result from each VFL client and send all collected local intermediate inference results to untrust AF in one notification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AF initiate VFL case is added into the general inference procedure Several ENs are resolved for the VFL inference procedure. The UE ID(s) is added in the request send from the VFL server to the VFL client. The VFL model correlation ID is added when then VFL client send the VFL inference result to the VFL server. NEF behaviour is added when the untrusted AF is the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL inference sucessully if some open issue can not be resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2G	Vertical Federated Learning

6.2G.1	General

This clause specifies how NWDAF(s) and AF(s) can leverage Vertical Federated Learning technique to perform VFL inference.

6.2G.2	Procedures

6.2G.2.1	General inference procedure for vertical federated learning

Figure 6.2G.2.1-1: General inference procedure for vertical federated learning

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF sends a subscription request to VFL server.

If the NWDAF is the VFL server, the NWDAF containing AnLF sends a subscription request to VFL server NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs.

If the AF is the VFL server, the NWDAF containing AnLF sends a subscription request to VFL server AF using Naf_AnalyticsExposure_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs. If the AF is the untrusted AF, the subscription request is sent via NEF.

Editor's note:	Which service used between the NWDAF containing AnLF and the VFL server AF is FFS.

2.	Based on the information received in the step 0 or 1, VFL server decides to iniate the VFL inference procedure with the VFL clients. VFL determines the VFL clients as described in the clause 6.2H.2.1.

NOTE: If the AF is the VFL server, the VFL clients can only be NWDAF(s).

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

If the VFL server is the NWDAF and the VFL client is the NWDAF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the UE ID(s), the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

If the VFL server is the NWDAF and the VFL client is the AF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the UE ID(s), the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

If the VFL server is the AF and the VFL client is the NWDAF, VFL server AF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the UE IDs, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server AF sends the request to the NEF, and NEF forwards the request to the VFL client NWDAF(s).

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

5.	Each VFL Client sends the intermediate local inference results to the VFL server including the VFL model correlation ID.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to aggregates and generates the VFL inference results.

If the untrusted AF is the VFL server, the NEF may wait to collect local intermediate inference result from each VFL client and send all collected local intermediate inference results to untrust AF in one notification.

6.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model correlation ID.

Editor's note:	Further details of the VFL server computing the VFL inference is FFS.

7.	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

If the AF is the VFL server, the VFL server AF sends Naf_AnalyticsExposure_ Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

# * * * * End of changes * * * *