SA WG2 Meeting ##166-Ad-Hoc-e	S2-2500038

20 - 24 January, 2025, Electronic meeting

3GPP TSG-CT WG4 Meeting #126	C4-245430

Orlando, U.S; 18th – 22nd November 2024

# Title:	Reply LS on slice mapping between the HPLMN and EHPLMN

# Response to:	LS S2-2411063 on slice mapping between the HPLMN and EHPLMN

# Release:	Rel-18

# Work Item:	TEI18, 5GS_Ph1, 5GProtoc17

Source:	CT4

To:	SA2

Cc:	CT1

Contact Person:

Name:	Shuang Liang

Tel. Number:	+86 15850539616

E-mail Address:	liang.shuang3@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT4 would like to thank SA2 for their reply on CT4’s questions. As indicated in reply LS:

Q3: If the answer of Q1 is yes, how is the admission control of the HPLMN S-NSSAI and the mapped S-NSSAI of EHPLMN performed in the 5GC? Are there two NSACFs, i.e. one for HPLMN and the other for EHPLMN?

SA2 answer: When different S-NSSAI values are used for HPLMN and EHPLMN, the (I-)SMF performs NSAC procedures separately for EHPLMN S-NSSAI and HPLMN S-NSSAI. Whether to deploy one NSACF (supporting both S-NSSAIs) or two NSACFs is per operator deployment.

CT4 is working on clarification for NSAC procedures based on SA2 answer in CT4#126 and has reached the following assumptions in common:

1. For non-roaming case of NSAC for number of UEs, as indicated in SA2 answer, the operator may deploy only one NSACF which serves both EHPLMN S-NSSAI and HPLMN S-NSSAI. In this case, the AMF may send one request to this NSACF, carrying the serving PLMN (i.e. EHPLMN) S-NSSAI together with the serving PLMN ID and the corresponding mapped HPLMN S-NSSAI together with the HPLMN ID.

2. For non-roaming case of NSAC for number of PDU sessions, I-SMF shall provide the S-NSSAI of the serving PLMN (i.e. EHPLMN) to the NSACF and A-SMF shall provide the S-NSSAI of the home PLMN to the NSACF.

However CT4 has found that I-SMF was not allowed to interact with NSACF as specified in clause of 5.15.11.2.1 in TS 23.501.

NOTE 2:	I-SMF does not interact with NSCAF.

Thus CT4 would like to ask the following questions to SA2:

Q1: Could the restriction of interaction between I-SMF and NSACF be removed in the case of different S-NSSAI values are used for EHPLMN and HPLMN?

Q2: If the answer of Q1 is yes, are the CT4’s assumptions still conflicting with any other stage2 principles?

Q3: If the answer of Q1 is no, what is the correct way to handle the EHPLMN NSAC in above scenario from stage 2 perspective?

2. Actions:

To SA2

ACTION:

CT4 kindly asks SA2 to answer the above questions and provide clarification accordingly.

3. Date of Next CT4 Meetings:

CT4#127		February 17-21, 2025 				Athens, GR