SA WG2 Meeting ##166-Ad-Hoc-e	S2-2500066

20 - 24 January, 2025, Electronic meeting

3GPP TSG-WG SA4 Meeting #130	S4-242169

Orlando, USA, 18th – 22nd Nov, 2024

# Title:	LS on Time Synchronization for MBS

# Response to:	—

# Release:	Rel-19

# Work Item:	FS_AMD

Source:	SA4

To:	SA2, RAN2

Cc:

Contact Person:

Name:	Thomas Stockhammer

Tel. Number:

E-mail Address:	tsto AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

# 1.	Overall Description

SA4 is in the process of concluding a feasibility study on Advanced Media Delivery. One Key Issue is a systematic analysis of MBMS User Services features and their potential relevance for MBS. In this context it was identified that MBMS in TS 26.346 supports time synchronization between the BM-SC and MBMS UEs. This is needed for user service protocols, such as the schedule in the user service description or the File Delivery Table (FDT) for object delivery, both containing NTP-encoded time values.

To support these user service functionalities consistently, time synchronization between the MBS client and the network entity that generates this time stamp is needed. SA4 considers that time synchronization for MBS could be supported by referencing the required functionality in TS 26.346 [16] in clause 4.6. However, this requires that an SNTP server is available on the network. Candidate solutions to support the network functionalities (i.e. the SNTP server) for this feature include

- the addition of a time server functionality in the MBSTF, or

- the addition of a time server functionality in the MBSF, or

- a new network entity supporting time server functionality that is shared between all entities requiring time synchronization.

Before concluding on the appropriate solution, SA4 would kindly ask if SA2 or RAN2 are aware of a network entity that can be used by UEs and network functions for time synchronization, and whether the reference points are defined. If this is not available, would SA2 provide such a functionality. If not available or foreseen to be specified, we consider extending the MBSF or MBSTF to add this functionality as part of the MBS User Service architecture in TS 26.502.

# 2.	Actions

To RAN2 group.

ACTION: 	SA4 asks RAN2 to take the above into account and provide feedback if any.

To SA2 group.

ACTION: 	SA4 asks SA2 to take the above into account and provide feedback if any.

3. Date of Next TSG SA WG4 Meetings:

TSG-SA4 Meeting #131		17th – 21st Feb, 2025			Geneva, CH

TSG-SA4 Meeting #131-bis-e	11th – 17th Apr, 2025			Online