SA WG2 Meeting #166 Ad Hoc-e	S2-2500088

# Online, 20 - 24 January, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0635 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ML Model retrieval from ADRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-12-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As written in TS 23.288 clause 6.2A.1, the LMF may retrieve ML Model information from NWDAF containing MTLF. The ML Model information includes the ML Model file address or ADRF (Set) ID. Therefore the NWDAF may provide ADRF information to LMF then LMF retrieve ML Model from ADRF accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that LMF may retrieve ML Model from ADRF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The defined procedure may not working. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.22.5	LMF retrieval of a trained AI/ML Model for positioning

The LMF may retrieve a trained AI/ML Model from NWDAF. The LMF obtains a trained AI/ML model prior to a UE positioning request that requires a trained ML Model.

Figure 6.22.3-1: The LMF retrieves a trained AI/ML based positioning model from NWDAF

1.	The LMF determines using local policies, that a positioning method requires a trained ML Model, the trained ML Model is not trained locally but is retrieved from NWDAF.

2.	The LMF discovers the NWDAF that can provide a trained model, using Nnrf_NFdiscovery including a request for AI/ML Model for UE positioning, ML Model Filter Information that includes the positioning case information and an Area(s) of Interest, the ML Model Interoperability indicator applicable to the trained ML Model for UE Positioning, optionally the ML Model Accuracy checking capability.

NOTE:	NWDAF discovery and selection is described in clause 5.2 of TS 23.288 [37], the applicable NWDAF parameters used for discovery of an NWDAF supporting an AI/ML Model for UE Positioning are described in clause 5.18.

3.	The LMF receives a list of candidates NWDAF instances, validity period for the discovery, per NWDAF instance the NWDAF instance ID, FQDN or IP address, and the ML Model Interoperability Indication.

4.	The LMF selects a NWDAF out of the list of candidates NWDAF instances, that can provide a ML Model that supports the same ML Model Interoperability Indication then request a trained AI/ML Model using Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe including the request for a trained AI/ML Model for UE Positioning according to the information listed in clause 5.18.

Editor´s Note:	Whether the LMF provides Inference Input Data information to NWDAF in the Nnwdaf_MLModelProvision service is FFS.

5.	The NWDAF provides one or more AI/ML Models to LMF including a ML Model Identifier and the ML Model Information as described in clause 5.18. If LMF received ADRF (set) ID inside ML Model information, the LMF may retrieve ML Model file from ADRF as described in clause 6.2B.7 of TS 23.288 [37].

6.	Same as in step 5.

The LMF updates the received trained ML Model(s) when the NWDAF provides new ones.

* * * End of Changes* * *