3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500112

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0636 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Resolving open issues for LMF data exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editor´s Note Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS.: It is preferable that LMF selects target ground truth label (considering for instance the number of available PRUs in an area), but the NWDAF can provide a target ground truth label precision as input to obain training data safistying precision requirements. It can be assumed that only UE maessurements or NR RAN maessurements can be used to train models, depending on deployment, and so the MTLF should be able to indicate what meassurements it desires Editor´s Note: Whether the LMF provides Inference Input Data information to NWDAF in the Nnwdaf_MLModelProvision service is FFS It can be assumed that only UE maessurements or NR RAN maessurements can be used to train models, depending on deployment, and so when retrieving a model the LMF should be able to indicate to the MTLF a model for what meassurements it desires. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Requested type of meassurements (UE based meassurements and/or NG RAN based meassurements) and target ground truth label precision are added as input parameters to the Nlmf_DataExposure_Subscribe service operation. - The LMF indicates the type of meassurements (UE based meassurements and/or NG RAN based meassurements) for which it requests a model in the Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe service operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved open issue remains. NWDAF is not able to request input data for models to derive location from UE maessurements or from RAN maesurements only. NWDAF is not able to control the precision of ground truth lables is uses to train the models. LMF is nor able to request models matching the maessurements it intends to perform. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.4, 6.22.5, 8.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.22.4	Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement related data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Editor's note:	It is FFS how to adapt this procedure to ML model performance monitoring.

Figure 6.22.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters. If the NWDAF wants to collect the input data of PRUs, the NWDAF may also include a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

Editor's note:	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes an AoI the requested type of meassurements (UE based meassurements and/or NG RAN based meassurements),  and a notification target address to request the input data from LMF. The NWDAF may also include requested number for data samples and time window for data collection, and a target ground truth label precision.

4.	The LMF performs steps 1 to 4 in clause 6.22.2, or steps 1 to 7 in clause 6.22.3. The LMF takes the received “requested type of messurements”, “requested number for data samples”, “time window for data collection”, and “target ground truth label precision” into considertation during those procedures.

5.	The LMF sends the collected data samples to the NWDAF by invoking Nlmf_DataExposure_Notify service operation, then the NWDAF trains the ML model or performs ML model performance monitoring based on the data samples.

2nd change

### 6.22.5	LMF retrieval of a trained AI/ML Model for positioning

The LMF may retrieve a trained AI/ML Model from NWDAF. The LMF obtains a trained AI/ML model prior to a UE positioning request that requires a trained ML Model.

Figure 6.22.3-1: The LMF retrieves a trained AI/ML based positioning model from NWDAF

1.	The LMF determines using local policies, that a positioning method requires a trained ML Model, the trained ML Model is not trained locally but is retrieved from NWDAF.

2.	The LMF discovers the NWDAF that can provide a trained model, using Nnrf_NFdiscovery including a request for AI/ML Model for UE positioning, ML Model Filter Information that includes the positioning case information and an Area(s) of Interest, the ML Model Interoperability indicator applicable to the trained ML Model for UE Positioning, optionally the ML Model Accuracy checking capability.

NOTE:	NWDAF discovery and selection is described in clause 5.2 of TS 23.288 [37], the applicable NWDAF parameters used for discovery of an NWDAF supporting an AI/ML Model for UE Positioning are described in clause 5.18.

3.	The LMF receives a list of candidates NWDAF instances, validity period for the discovery, per NWDAF instance the NWDAF instance ID, FQDN or IP address, and the ML Model Interoperability Indication.

4.	The LMF selects a NWDAF out of the list of candidates NWDAF instances, that can provide a ML Model that supports the same ML Model Interoperability Indication then request a trained AI/ML Model using Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe including the request for a trained AI/ML Model for UE Positioning according to the information listed in clause 5.18. The LMF indicates the type  of meassurements (UE based meassurements and/or NG RAN based meassurements) for which it requests a model,

5.	The NWDAF provides one or more AI/ML Models to LMF including a ML Model Identifier and the ML Model Information as described in clause 5.18.

6.	Same as in step 5.

The LMF updates the received trained ML Model(s) when the NWDAF provides new ones.

3rd change

#### 8.3.4.2	Nlmf_DataExposure_Subscribe service operation

Service operation name: Nlmf_DataExposure_Subscribe

Description: Allow the consumer NF to subscribe the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Target Address, Notification Correlation ID, Area of Interest(AoI), requested type of meassurements (UE based meassurements and/or NG RAN based meassurements).

Input, Optional: Time window when to collect data, number of samples, Subscription Correlation ID (in the case of modification of the subscription), target ground truth label precision.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

When the requested number of samples has been provided or the time window for data collection has elapsed, the subscription is automatically terminated.

End of changes