3GPP TSG-SA WG2#166-166-AdHoc-e	S2-2500136

Electronic meeting, 20-24 January 2025	(was S2-2411675)

Source:	Qualcomm Incorporated

Title:	On the use of monitoring list and wait time for S&F

Document for:	Discussion/Approval

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC/Rel.19

Abstract of the contribution: Discusses the possible use of monitoring list and wait timer for S&F .

# 1.	Discussion

In SA2#165 and SA2#166 meetings it was not possible to reach consensus on the normative text for the topic of how to handle the “monitoring lists” that were agreed in TR 23.700-29 . The main controversial issue was how to model the use of “S&F Monitoring List” that will be signalled by the network and the resulting UE behaviour.

The conclusions of TR 23.700-29 in clause 8.2 related to “List of Satellite IDs” are as follows (text related is bold and highlighted):

With the following normative impacts: 1) When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information). NOTE 1: The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal. 2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB. 3) How the UE process this information is up to UE implementation and during the wait timer the UE can search for another terrestrial or satellite PLMN to get normal service. 4) MME may indicate to HSS the "Request Time", allowing the HSS to check that no other (e.g. terrestrial) MME has sent an Update Location Request after the "Request Time", and fetches the authentication vector and other details from HSS following current Authentication and security procedures. The MME may triggers Update location with the HSS and Update location ACK is received by MME. i.e. all the subscription details are retrieved by MME-ground. The Update Location Request includes an indication that this location update is provisional i.e. the HSS must not consider the UE as registered until it receives the final Update Location Request. 5) When the wait timer has expired given to the UE in step 2, if the UE has not successfully attached to another PLMN and the UE finds the cell which broadcast the Satellite ID valid to re-attempt the attach procedure, the UE re-sends the Attach or TAU Request message. 6) During the Attach or TAU procedure with the UE, the MME may also provide a list of Satellite IDs over which the UE may exchange the signalling and data, and a wait-timer that indicates to the UE the time it should wait before attempting signalling and data exchanges in those satellites. 7) The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation. NOTE 2: The estimated delivery time is the estimated time to send data from the UE to Gateway. 8) The core network can indicate to external SCS/AS whether UE is registered in S&F Mode and the estimated delivery time. NOTE 3: Whether any existing monitoring events or procedures can be used or enhanced to achieve above will be determined during normative phase. NOTE 4: SA WG2 will align further based on agreements in SA WG3 e.g. on security procedures and Attach/TAU procedures. […] With the following normative impacts: - Store and forward is only supported by EPS. - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data. - The UE needs to be aware that a satellite supports S&F mode. NOTE 9: How the UE is aware that a satellite supports S&F mode of operation depends on RAN. - A UE may be rejected if the satellite cannot support the UE at this time. The attach reject may provide a timer for the time the UE should wait before reattempting and S&F monitoring list which the UE can attempt attach again.

Observation 1: In the conclusions of TR 23.700-29 the inclusion of S&F monitoring lists in Attach Accept, Attach Reject or TAU Reject message is optional. Notice that it is optional both for the network to signal the information element that includes the S&F monitoring lists but also optional (“up to UE implementation”) for how the UE handles this information element.

Observation 2: The UE behaviour for S&F in general, and how the UE processes the information provided by the network is up to UE implementation. During the wait timer the UE can search for another terrestrial or satellite PLMN to get normal service.

In SA2#165 and SA2#166 a use case was explained that for the “split MME” scenario the UE context may not be synchronised in all the satellites of the deployment. This in turn means that when S&F mode is activated, if the UE attempts to Attach or perform TAU on satellites that are NOT included in the monitoring list it is certain that the related NAS procedure will fail.

This different behaviour in itself though will not justify the definition of two different S&F monitoring lists and wait timers that will add extra complexity in the UE procedures. Instead, in order to address this use case of partial UE context synchronisation, it is proposed to add a new indicator on whether the Wait Timer and S&F monitoring list are mandatory or advisory to be followed by the UE.

This in itself allows the network to control the UE behaviour: if the indication is set to “mandatory” then the network will expect that the UE will only attempt to perform Attach or TAU in satellites with IDs included in the S&F monitoring list; if, alternatively, the indication is set to “advisory” the network will expect that certain UE implementations may attempt Attach or TAU in any satellite, i.e., also the ones NOT included in the S&F monitoring list.

Note: in both cases it is possible for the UE to select another PLMN terrestrial or satellite based on procedures defined in TS 23.122.

Proposal 1: In order to address this use case of partial UE context synchronisation, it is proposed to add a new indicator on whether the Wait Timer and S&F monitoring list are mandatory or advisory to be followed by the UE.

# 2.	Text proposal

It is proposed to agree CR 3804 for TS 23.401.