3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500170

# Electronic Meeting, 20-24 January 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1532 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IMS data channel capability exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedures shown in figure AG.2.1-1 are not matched with the consequense description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Figure AG.2.1-1 is updated by adding the missing steps from IMS AS-2 to NEF and DC AS associated with steps 4 and 5 of the procedural text. The Editor’s Note in AG.1 is removed. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompletion of IMS DC capability exposure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AG.1, AG.2.1, AG.2.2, AG.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

# AG.1	Description

The following scenarios utilizing IMS data channel capability exposure services are depicted in clause AG.2:

-	Adding, removing or updating the data channel to an existing IMS session, for which Nimsas_SessionManagement_Update is used

-	Creating a standalone IMS data channel session, for which Nimsas_SessionManagement_Create is used.

-	Terminating a standalone IMS data channel session, for which Nimsas_ImsSessionManagement_Delete is used.

The procedures can be triggered via the NEF on the originating or terminating side.

# >>>>NEXT CHANGE<<<<

## AG.2.1	Updating an existing IMS session for a Data Channel

Figure AG.2.1-1 shows the procedure of adding, removing or updating bootstrap data channel(s) or application data channel(s) to an existing IMS session.

Figure AG.2.1-1: Procedure of adding data channel(s) to an existing IMS session

0.	DC AS may determine to add or remove data channel to an existing IMS session of UE B based on local policy or event notification of the related IMS Session which DC AS subscribes to.

NOTE 1:	It is assumed that the OMA API can be used for non-DC session notification in step 0.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF for adding or removing bootstrap data channel(s) or application data channel(s) in the exisiting IMS session.

The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified for the progress of the session update.2.	The NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS-2 instance serving the target user.

3.	The NEF sends a Nimsas_ImsSessionManagement_Update request to IMS AS-2 for adding or removing bootstrap data channel(s) or application data channel(s) in the specific session.

4.	IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE B and determines whether the data channel request can be accepted. If UE B does not have subscription of IMS data channel or does not have the IMS DC capability, then the request shall be rejected with appropriate cause and skip the following step. If IMS AS-2 accepts the request, it returns a successful Nimsas_SessionManagement_Update response to the NEF.

If the same IMS data channel (bootstrap or application data channel) is already established, then the request shall be rejected with an appropriate cause and the following steps are skipped.

5.	The NEF returns a successful Nnef_SessionManagement_Update response to the DC AS.

6.	IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to 3rdPartySessionUpdate. The notification includes the Media operation set as received in step 3.

DCSF-2 instructs IMS AS-2 to set up or remove the data channel(s) by sending a Nimsas_MediaControl request. Depending on the instruction from DCSF-2, IMS AS-2 may interact with the MF-2 to allocate data channel resources and media connection to the DC AS. The procedures depicted in clause AC.7 are reused for interaction with MF.

NOTE 2:	The type of data channel established is dependent on the instruction given by DCSF.

7.	IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel required by the DC AS with the other existing media descriptions.

IMS AS-2 sends the re-INVITE request(s) to UE A according to the instructions received from DCSF-2 in step 4.

NOTE 3:	The IMS DC session update procedures described in clauses AC.7.1 and AC.7.2 are followed.

8.	The DC media negotiation is completed and corresponding data channel(s) are established.

9.	IMS AS-2 notifies the DCSF for the IMS session events with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7.

If the DC AS included a subscription to the DC session events to the Nnef_ImsSessionManagement_Update request in step 1, IMS AS-2 notifies the NEF for the progress of the session update with the Nimsas_SessionManagement_Notify service operation.

10.	The NEF notifies the DC AS for the progress of the session update with the Nnef_SessionManagement_Notify service operation.

# >>>>NEXT CHANGE<<<<

## AG.2.2	Establishing standalone IMS data channel session

Figure AG.2.2-1 shows the establishment procedure of a standalone data channel session.

Figure AG.2.2-1: Establishment procedure of a standalone data channel session

0.	The DC AS decides to establish a standalone data channel session for UE B. A standalone data channel can be a standalone bootstrap or standalone application data channel.

1.	The DC AS sends a Nnef_ImsSessionManagement_Create request to the NEF for creating a standalone DC session from UE B to UE A.

The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Create request to be notified for the progress of the session update.

2.	The NEF queries the HSS for the address of IMS AS-2 serving UE B. If UE B is unregistered, that is, the HSS fails to return the address of IMS AS-2, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response and the following steps shall be skipped.

3.	The NEF sends a Nimsas_SessionManagement_Create request to IMS AS-2 requiring to create a standalone DC session.

4.	IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE B to determine whether the data channel request can be accepted. If UE B does not have subscription of IMS data channel, then the request shall be rejected with appropriate cause. If IMS AS-2 allows the request to proceed, it returns a successful Nimsas_SessionManagement_Create response to the NEF.

5.	The NEF returns a successful Nnef_SessionManagement_Create response to the DC AS.

6.	The DC media negotiation is completed and corresponding data channel are added, removed or updated.	IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to DCSF-2 with event set to 3rdPartySessionCreate. The notification includes the Media operation set as received in step 3.

DCSF-2 instructs IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. IMS AS-2 interacts with MF-2 to allocate data channel resource and media connection to the DC AS. The procedures in clause AC.7 are reused to interact with the MF.

NOTE:	The type of data channel established is dependent on the instruction given by DCSF-2.

7.	IMS AS-2 generates an INVITE request(s) in which the SDP offer contains the media information of the data channel(s) requested by the DC AS.

IMS AS-2 sends the INVITE request(s) according to the IMS DC session establishment procedures described in clauses AC.7.1 and AC.7.2.

8.	The DC media negotiation is completed and corresponding data channel are established. IMS AS-2 notifies DCSF-2 for the IMS session events with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7.

9.	The IMS AS notifies the DCSF for the IMS session events with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7.

If the DC AS included a subscription to the DC session events to the Nnef_ImsSessionManagement_Create request in step 1, IMS AS-2 notifies the NEF for the progress of the session update with the Nimsas_SessionManagement_Notify service operation.

10.	The NEF notifies the DC AS for the progress of the session update with the Nnef_SessionManagement_Notify service operation.

# >>>>NEXT CHANGE<<<<

## AG.2.3	Terminating standalone IMS data channel session

Figure AG.2.3-1 shows the terminating procedure of a standalone IMS data channel session. A standalone data channel can be a standalone bootstrap or standalone application data channel.

Figure AG.2.3-1: Terminating of a standalone data channel session

1.	The DC AS sends a Nnef_ImsSessionManagement_Delete request to the NEF for terminating a standalone DC session between UE B and UE A.

2.	The NEF queries the HSS for the address of the IMS AS-2 serving UE B if needed.

3.	The NEF sends a Nimsas_ImsSessionManagement_Delete request to the IMS AS-2 requiring to terminate the standalone IMS data channel session identified by the Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to DCSF-2 with the indication of terminate the standalone data channel. Then DCSF-2 instructs the IMS AS-2 to terminate the data channel media by sending a Nimsas_MediaControl request.

5.	The IMS AS-2 sends the BYE request according to existing IMS procedures to terminate the data channel session.

6.	The media negotiation is completed, and corresponding data channel are removed.

7.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Delete response to the NEF.

8.	The NEF returns a successful Nnef_ImsSessionManagement_Delete response to the DC AS.

# >>>>END OF CHANGES<<<<