SA WG2 Meeting #166-AdHoc-e	S2-2500269

E-meeting, 20 – 24 January 2025	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5912 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the ENs of energy consumption information collection and exposure for certain application |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are three ENs in current TS for energy information exposure. Editor's note: Whether there are other required parameters for energy consumption information exposure are FFS. Editor's note: Whether and how the renewable energy is introduced is FFS and depending on SA WG5. Editor's note: Whether and how the per application energy consumption information can be exposed are FFS. For current discussion, no other new information is needed for exposure. And for the renewable energy part can be left to R20 to solve. For the per application energy information, if the consumer requests the energy information for certain application: - For UL traffic, the IP 5-Tuple can be set as: source IP address is set to wildcard that represents any UE, and the target IP address is set to application IP address and port number. - For DL traffic, the IP 5-Tuple can be set as: source IP address is set to application IP address and port number, and the target IP address is set to wildcard that represents any UE. - And EIF receives the collected information from OAM or UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification of how to collect information of per application. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that how to collect the energy related information for certain application. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change* * *

5.51.2.4	Energy Consumption information exposure

AF/NEF or 5GC NF subscribes the EIF for Energy consumption information of required granularities (UE, PDU session and/or QoS flow).

-	For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

-	For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

-	For QoS flow level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and Flow description(s).

-	The consumer NF may also subscribe the above information exposure with providing reporting period, reporting frequency.

-	For application-level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and Flow description(s). The flow descriptions are configured as below:

-	For UL traffic, the Flow description(s) can be set as: source IP address is set to wildcard that represents any UE, and the target IP address is set to application server IP address and port number.

-	For DL traffic, the Flow description(s) can be set as: source IP address is set to application server IP address and port number, and the target IP address is set to wildcard that represents any UE.

NOTE: How to collect and expose renewable energy related information is not specified in this release.

* * * End of Change * * *