SA WG2 Meeting #166-AdHoc-e	S2-2500278

E-meeting, 20 – 24 January 2025	(revision of S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0278 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UE mobility for PDU session supporting Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This paper provides some corrections of UE mobility for PDU session supporting Local Offloading Management. - During the UE mobility, if the EASDF IP address is not changed, it is not needed to always update the EASDF IP address to UE. - The UE in new location, if new I-SMF is selected, the SMF still needs to receive the Local Offloading Management Policy under this new location for the new I-SMF. - When the new I-SMF is selected by AMF in new location, the new I-SMF performs Nsmf_PDUSession_Update towards SMF. This is different compared to old I-SMF, the old I-SMF performs Nsmf_PDUSession_Create. This will be reflected in 502 CR. - For old I-SMF removal, the DNS context should be released and the in order to support the SSC mode 2 or 3, it follows the procedure defined in section 4.3.5.1 and 4.3.5.2 of TS 23.502 to replace SMF 1 to old I-SMF and SMF 2 to new I-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing procedure during UE mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to support local offloading during UE mobility is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st of Change * * *

6.10.2.4	UE mobility for PDU session supporting Local Offloading Management

For a PDU session supporting Local Offloading Management, during mobility events, e.g. Hand-Over or AMF change, the mobility handling is executed same as the respective mobility handling defined in clause 4.23 of TS 23.502 [3] with the following difference:

-	The retrieval of the Local Offloading Management Allowed indication as part of the SMF selection subscription data from UDM, the new AMF determines the new I-SMF as follows:

-	If the service area of SMF includes the new UE location and the UE location is out of local offloading management service area, the AMF removes the old I-SMF (if available).

-	If the service area of SMF include the new UE location and the UE location is within local offloading management service area, the AMF selects a new I-SMF unless the existing I-SMF can serve the new UE location. If there is an old I-SMF which cannot serve the new UE location, the AMF also removes the old I-SMF.

-	To support the local offloading management, the selected I-SMF need to support Local Offloading Management, serves the new UE location; and support the DNN and S-NSSAI associated with the PDU session.

NOTE:	When the UE moves from an AMF which is not upgraded to act based on the Local Offloading Management support indication to an AMF which is upgraded to act based on the Local Offloading Management support indication, the above procedure also applies.

-	For I-SMF insertion and relocation case, under new UE location, the SMF and PCF interacts as follows:

-	If the new I-SMF is selected and allows new I-SMF to perform Local Offloading Management under new UE location, the SMF initiates SM Policy Association Modification procedure towards PCF with local offloading support indication to receive the updated Local Offloading Management Policy or Offload Identifier.

-	For I-SMF insertion and relocation case, the target I-SMF and SMF interacts as follows:

-	The target I-SMF sends new selected EASDF IP address and Local Offloading Management allowed indication to the SMF.

-	(Only applicable to the case that the selected EASDF by new I-SMF is not the same as the EASDF IP address that selected by old I-SMF) The SMF sends the new selected EASDF IP address to the UE via PCO.

-	The SMF sends the newly retrieved Local Offloading Management Policy or Offload Identifier to the I-SMF.

-	For I-SMF removal case, the SMF performs as follows:

-	(Only applicable to the case that the EASDF selected by new I-SMF is not the same as the EASDF that selected by old I-SMF) A new EASDF is selected by the SMF and the SMF sends the new selected EASDF IP address to the UE via PCO.

-	(Only applicable to the case that the EASDF selected by new I-SMF is not the same as the EASDF that selected by old I-SMF) The SMF performs Neasdf_DNSContext_Create towards EASDF.

-	For I-SMF removal case, the old I-SMF performs as follows:

-	The old I-SMF performs Neasdf_DNSContext_Delete to EASDF to remove the DNS context.

-	If SSC mode 2 or SSC mode 3 is supported during I-SMF relocation, it follows the procedure defined in section 4.3.5.1 and 4.3.5.2 of TS 23.502[3] to replace SMF 1 to old I-SMF and to replace SMF 2 to new I-SMF.

-	If edge relocation is needed during I-SMF relocation, it follows the procedure defined in section 6.3.5 to reduce the packet loss.

* * * End of Change * * *