3GPP TSG WG2 Meeting #166AH	S2-2500284

E-meeting,  2025-01-20 – 2025-01-24

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5915 | rev |  | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: |  | Update on Handling of Payload Headers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: |  | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: |  | F | F |  |  |  |  | Release: |  |  | Rel-19 |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: | According to the FS_UPEAS_Ph2 conclusion, Handling of Payload Header feature can cause UPF load increase due to frequent notification for (Specifically, for 'any UE' case and 'every match'). AF and SMF wants how many header handling actions were enforced for a given event. Using this information, And SMF should control UPF load issue due to frequent actions. AF can also use these statistics for its other purpose. So, Reporting the performed actions event is needed without high signalling load due to frequent notification. |  |  |  |  |  |  |  |  |  |  |
| Summary of change: | The followings are updated - Reporting suggestion information is added in the Header Handling Reporting information. - A description text of UPF action for receiving ‘Reporting suggestion information’ is added. |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | Handling of Payload Headers Enhancements cannot reduce UPF performance impact due to frequent event notification. |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 5.6.17, 5.8.2.22 |  |  |  |  |  |  |  |  |  |  |
|  | Y |  |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

# * * * * first change  * * * *

5.6.17	Handling of Payload Headers

5.6.17.1	General

Handling of Payload Headers is an optional feature that allows exchange of information in-band in the user plane packets between the application (in the UE or in the application server) and UPF in the 5GS by supporting detection, insertion, replacement, or removal of payload headers of PDUs. Handling of Payload Headers can be requested by the AF.

NOTE 1:	The Handling of Payload Headers is not limited to the protocol header but can be any field or tag supported by a protocol.

NOTE 2:	Handling of Payload Headers is expected to be used instead of Header Enrichment (see clause 5.8.5.6) as it allows more actions than just insertion, supports different protocols and allows the usage of actions in both directions.

The content of this clause applies to non-roaming and to Home Routed roaming scenario, i.e. to cases where the involved entities (AF, PCF, SMF, UPF) belong to the Home PLMN and the AF has an agreement with the Home PLMN.

The PCF controls Handling of Payload Headers in UPF by provisioning Header Handling Control information in the PCC rule. The PCF derives the Header Handling Control information as described in 6.1.3.30 of TS 23.503 [45] based on the AF request and provides it to the SMF in the related PCC rule as described in clause 6.3.1 of TS 23.503 [45]. SMF instructs UPF via N4 interactions by means of FAR (see clause 5.8.5.6).

The UPF performs Handling of Payload Headers and may notify events related to the Handling of Payload Headers to SMF, to NEF/AF or to both as described in clause 5.6.17.2.

5.6.17.2	Application Function influence on Handling of Payload Headers

An AF may request the Handling of Payload Headers in order to insert, replace, or remove payload headers of PDUs as well as to subscribe to notifications on events related to detection of payload headers or related to the actions performed on payload headers.

For the Handling of Payload Headers, it is assumed that a service level agreement (SLA) exists between the operator and the third party AF. The mechanism(s) required for detection and actions performed on payload headers (e.g. protocol layer, type of encryption, etc.) is agreed as part of the SLA. The reference(s) to the UPF configuration(s) corresponding to the header detection as well as reference(s) to SMF configuration corresponding to SMF context information are also part of the SLA. The AF is required to provide such reference(s) in the AF requests for Handling of Payload Headers.

In the non-roaming scenario, the features Handling of Payload Headers, Application Function influence on Service Function Chaining (as defined in clause 5.6.16) and Application Function influence on traffic routing (as defined in clause 5.6.7) can be applied simultaneously.

In the Home Routed roaming scenario, the features Handling of Payload Headers and Application Function influence on Service Function Chaining (as defined in clause 5.6.16) can be applied simultaneously.

The AF requests for Handling of Payload Headers are sent to the PCF via N5 (in the case of requests targeting specific on-going PDU Sessions of individual UE(s), for an AF allowed to interact directly with the 5GC NFs) or via the NEF. The AF requests that target existing or future PDU Sessions of multiple UE(s) or of any UE are sent via the NEF and may target multiple PCF(s), as described in clause 6.3.7.2. The PCF(s) transform(s) the Header Handling Control information provided in the AF requests into Header Handling Control information of the PCC rules that apply to the respective PDU Sessions. When the AF has subscribed to events related to Handling of Payload Headers, such notifications are sent by the UPF(s) either directly to the AF or via an NEF.

When the request is sent via NEF, Nnef_TrafficInfluence service is used. The procedures for AF to request Handling of Payload Headers are described in clause 4.3.6 in TS 23.502 [3].

The AF requests may contain the information as described in the Table 5.6.17.2-1:

Table 5.6.17.2-1: Information elements contained in AF request

| Information Name | Applicable for PCF or NEF (NOTE 1) | Applicable for NEF only | Category |
| --- | --- | --- | --- |
| Traffic Description | Defines the target traffic on which to apply handling of headers, represented by the combination of DNN and optionally S-NSSAI and application identifier or traffic filtering information. | The target traffic can be represented by AF-Service-Identifier. | Mandatory |
| Target UE Identifier(s) | Indicates the UE(s) that the request is targeting, i.e. an individual UE, a group of UE represented by Internal Group Identifier(s) (NOTE 2), or any UE accessing the combination of DNN and S-NSSAI. | GPSI can be applied to identify the individual UE, or External Group Identifier can be applied to identify a group of UEs. | Mandatory |
| Spatial Validity Condition | Indicates that the request applies only to the traffic of UE(s) located in the specified location, represented by areas of validity. | The specified location can be represented by geographical area. | Optional |
| Temporal Validity Condition | Time interval(s) when the request applies or duration(s). | N/A | Optional |
| AF transaction identifier | The AF transaction identifier refers to the AF request. | N/A | Mandatory |
| Header Handling Control information |  |  |  |
| Header Detection Reference | A reference to a UPF configuration which defines how to detect the protocol or the message in a protocol for which to perform the header handling actions. | N/A | Mandatory |
| Header Detection Support Information (NOTE 3) | Any dynamic information provided by the AF which is required for the detection of the headers and cannot be preconfigured for the Header Detection Reference. | N/A | Optional |
| Header Handling Reporting Endpoints | List of notifications endpoints, i.e. per notification endpoint a Notification Target Address and a Notification Correlation ID. | N/A | Optional |
| Header Handling Control Reference (NOTE 5) | A reference to a Header Handling Action related information pre-configured in the UPF. | N/A | Optional |
| Header Handling Direction (NOTE 4) | Indicates if the header handling applies to UL or DL direction. | N/A | Optional |
| Header Handling Action (NOTE 4) | Indicates the action to be performed on a specific header field. | N/A | Optional |
| Header Information (NOTE 4) | A reference to a UPF configuration which defines how to identify or build a specific header field for which to perform the header handling action. | N/A | Optional |
| Header Value (NOTE 4) | A string providing the value (or a reference to information to be provided by SMF) of the specific header field. | N/A | Optional |
| Header Handling Condition (NOTE 4) | Indicates the condition for performing the header handling action. | N/A | Optional |
| Header Handling Reporting (NOTE 4) | Indicates whether reporting is requested for the performed Header Handling Action and the notification endpoint(s). | N/A | Optional |
| NOTE 1: When the AF request targets existing or future PDU Sessions of multiple UE(s) or of any UE and is sent via the NEF, as described in clause 6.3.7.2, the information is stored in the UDR by the NEF and notified to the PCF by the UDR. NOTE 2: Internal Group ID can only be used by an AF controlled by the operator and only towards PCF. NOTE 3: The NEF, PCF and SMF do not need to understand the Header Detection Support Information. NOTE 4: These parameters are provided together to request a Header Handling Action. Multiple sets of these parameters (and thus multiple header handling actions) can be provided. NOTE 5: If a Header Handling Control Reference is provided, one or more of these parameters (marked by NOTE 4) can be provided to overwrite Header Handling Control information that is pre-configured in the UPF as per the Header Handling Control Reference. Multiple Header Handling Control References can be provided. |  |  |  |

For each information element mentioned above as part of Header Handling Control Information in the AF request, a detailed description follows:

1)	Header Detection Reference:

A reference to a UPF configuration which defines how to detect the protocol or the message in a protocol for which to perform the header handling actions (e.g. information about protocol layer, type of encryption, message type etc.).

2)	Header Detection Support Information:

Header Detection Support Information is not standardised, but it can be interpreted by UPF based on SLA. It is sent transparently by NEF, PCF and SMF to UPF. It includes dynamic information provided by the AF which is required for the detection of the headers and cannot be preconfigured for the Header Detection Reference in UPF.

3)	Header Handling Reporting Endpoints:

A list of notifications endpoints for the notifications related to the Handling of Payload Headers. Per notification endpoint in the list, a Notification Target Address and a Notification Correlation ID has to be provided.

4)	Header Handling Direction:

Header Handling Direction indicates to which direction the Header Handling Action applies to. It can have the values uplink (UL) and/or downlink (DL).

5)	Header Handling Action:

The action to be performed on a specific header field. One of the following can be requested:

-	Detect. It is used to request the detection of a header field that is identified based on the UPF configuration referenced by the Header Information parameter and the information contained in the Header Value parameter, if it is provided.

-	Remove. It is used to request the removal of a header field that is identified based on the UPF configuration referenced by the Header Information parameter and the information contained in the Header Value parameter, if it is provided.

-	Replace. It is used to request the replacement of information in a header field that is identified based on the UPF configuration referenced by the Header Information parameter. The information to be replaced in the header field is also defined by that UPF configuration. The Header Value contains information that is used for the replacement.

-	Insert. It is used to add a header field according to the UPF configuration referenced by the Header Information parameter. The Header Value contains information that is used for the insertion, if it is provided.

When multiple actions are requested on the target traffic, the enforcement of the Header Handling Action is performed in the following order: Detect, Remove, Replace and Insert.

NOTE 1:	The execution order is relevant to build a request that has the intended impact on the traffic. For example, a header that is removed cannot be replaced.

6)	Header Information:

A reference to a UPF configuration which defines how to identify or build a specific header field for which to perform the header handling action.

7)	Header Value:

A string providing the value (or a reference to information to be provided by SMF) of the specific header field relevant for the action (i.e. to detect in case of actions Detect or Remove and for updating in case of actions Insert or Replace). Header Value may optionally be provided apart from the Replace action for which it is mandatory.

8)	Header Handling Condition:

This defines how to apply the Header Handling Action. It may have one of the following values: every match or only for the first match.

NOTE 2:	In addition, Spatial Validity Condition and Temporary Validity Condition parameters in the request can be used to indicate that the request applies only in certain UE location(s) or during certain time interval(s), respectively.

9)	Header Handling Reporting:

Indicates whether reporting is requested for the performed Header Handling Action or not. If reporting is requested, the notification endpoint(s) for this reporting have to be indicated.

NOTE 4: 	In addition, Report suggestion information parameter as defined in clause 5.8.2.17 can be included to reduce the UPF performance impacts if Header Handling Condition is ‘every match’.

10)	Header Handling Control Reference:

A reference to a Header Handling Action related information that is pre-configured in the UPF. This pre-configuration in UPF comprise parameters 4-9. The Header Handling Control Reference can be provided in the AF request instead of providing the parameters of the Header Handling Control Information individually.

NOTE 3:	The reference to a Header Handling Control information that is pre-configured in the UPF and the corresponding header handling behaviour has to be agreed as part of the SLA. In a deployment, for simplicity, an operator can choose to set this reference to the same value as the Header Handling Detection Reference.

# * * * * next change (all new) * * * *

5.8.2.22	Handling of Payload Headers

Support of Handling of Payload Headers is an optional UPF feature.

The SMF receives Header Handling Control information in a PCC rule (as described in clause 6.3.1 of TS 23.503 [45]), it forwards the Header Handling Control information to the UPF via N4 interactions. The SMF adds the Header Handling Control information relevant for the UL direction into the FAR that corresponds to the UL PDR of that PCC rule and/or the Header Handling Control information relevant for the DL direction into the FAR that corresponds to the DL PDR of that PCC rule.

If the Header Value parameter contains a reference to information to be added by SMF, the SMF shall overwrite the Header Value parameter with the indicated information of the corresponding PDU Session.

If SMF determines based on local configuration that it shall receive notifications related to the Handling of Payload Headers, it can modify the Header Handling Reporting of the header handling actions of interest by setting it to "yes" and by adding itself as an endpoint. In addition, the SMF shall add its Notification Target Address and a Notification Correlation ID to the Header Handling Reporting Endpoints.

When UPF receives a Header Handling Control Rule from SMF over N4 (see clause 5.8.5.6), it contains a Header Detection Reference (see clause 5.6.17.2) which points to a predefined configuration in UPF which includes the details of the protocol and possibly the message for which a header handling action shall be performed (e.g. HTTP, HTTP message). The interpretation of the Header Detection Reference is part of the SLA and thus, implementation specific.

The Header Handling Control Rule contains other explicit Header Handling Control information to be used for handling the traffic that matches the Header Detection Reference, see clause 5.6.17.2.

When the Header Handling Control Rule includes Header Handling Reporting information, UPF may send notifications related to the Handling of Payload Headers to SMF and/or NEF/AF.

If Header Handling condition is ‘every match’ and Reporting suggestion information(as defined in clause 5.8.2.17) is included in the Head Handling Report information, UPF can concatenate several notification messages to the same Notification Target Address in one notification message to reduce the UPF performance impacts due to frequent event notification.

# * * * * End of changes * * * *