3GPP TSG-SA WG2 Meeting #166E	S2-2500294

# EMeeting, 20 - 24 January, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1547 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication with early media with two AGWs allocated |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Handling of early media for UE-satellite-UE communication has been discussed. S2-2411950 discuss the issue for early media handling, and proposes to sustain using ground-AGW for early media based on operator policy. This document proposed an alternative way for optimized the procedure after IMS session is answered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedure for supporting UE-Satellite-UE communication with early media in IMS that P-CSCF manages IMS AGW on ground and on satellite for a UE to optimize the IMS session update procedure after answering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early media may be terminated or interrupted when UE-satellite-UE communication is activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.5.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change (Alternative for optimization) ---

AE.5.y	At call setup with early media with two IMS AGWs allocated

During the call setup with both originating and terminating UEs using regenerative NR satellite access, the originating P-CSCF selects the IMS-AGW on satellite to transfer regular media and uses the IMS-AGW on ground to transfer early media then releases the IMS-AGW on ground after the IMS session is answered. See Figure AE.5.y-1 that describes the procedure.

In this procedure, the IMS AS for early media uses existing method for providing early media and updating the IMS session when answered. The example procedure assumes that the IMS AS for early media is configured according operator policy to update media only with the originating side when the IMS session is answered, which has less siganlling round-trip over satellit comparing to the configuration of updating media with both originating side and terminating side.

Figure AE.5.y-1: IMS session establishment procedure with early media with two IMS AGW allocated

1-10.	Steps 1-10 of clause AE.5.1 apply.

11.	Step 11 of clause AE.5.1 applies with the difference that the IMS AS serving early media allocates early media resources with MRF, as well as includes the P-Early-Media header field and a modified SDP answer for early media in the 18X response.

12.	Based on the identification of the satellite serving UE B and the P-Early-Media header field in step 11, P-CSCF A determines to perform UE-satellite-UE communication with early media. Step 12 of clause AE.5.1 applies with differences that P-CSCF A does not release the IMS-AGW A' on ground, and interacts with IMS-AGW A' on ground for early media to allocate and configure transport resources for IMS access side as well as configure transport resources for IMS core network side.

13. Step 13 of clause AE.5.1 applies.

The early media is able to be sent from the MRF to the UE A via the IMS AGW A' on ground, the UPF A' on ground, and the UPF A on satellite.

14.	Step 14 of clause AE.5.1 applies with the difference that P-CSCF A includes in the SIP 18X response an SDP answer containing the transport address of IMS-AGW' on ground obtained in step 12.

15-16.	Steps 15-16 of clause AE.5.1 apply.

17-18.	Step 17-18 of clause AE.5.1 apply.

19.	UE B returns SIP 200.

20.	P-CSCF B forwards the SIP 200 with SDP answer towards IMS core network, which contains the IMS-AGW transport address obtained in step 17.

21.	P-CSCF A interacts with IMS-AGW A on satellite for regular media to allocate and configure transport resources for IMS access side and configure transport resources for IMS core network side.

22.	The procedure continues and UE B is ringing.

23.	The UE B answers the IMS session and sends SIP 200 response for the INVITE.

24.	The IMS AS serving early media instructs the MRF to stop playing early media. UE A may play local ring tone when early media is stopped.

25.	The IMS AS serving early media sends SIP UPDATE request with the SDP offer for regular media towards originating side.

26.	P-CSCF A releases the IMS AGW A' on ground.

27.	P-CSCF A forwards the SIP UPDATE request with modified SDP offer to the UE A, which contains the IMS-AGW transport address obtained in step 21. The UE A returns SIP 200 response for the UPDATE with an SDP answer.

28.	P-CSCF A sends SIP 200 response with generated/modified SDP answer towards IMS core network side, which contains the IMS-AGW transport address obtained in step 15.

NOTE 1:	In order to reduce the latency, P-CSCF A can perform step 28 right after step 25.

29.	The IMS AS serving early media sends SIP 200 response for the INVITE to the originating side. P-CSCF A forwards the SIP 200 response to UE A.

After receiving the SIP 200 response, the UE A is able to send regular media and is ready to receive regular media. The regular media uses UE-satellite-UE communication path.

NOTE 2:	In order to reduce the latency, P-CSCF A can send SIP 200 response for the INVITE to UE A (29b) and send SIP ACK on behave of UE A towards terminating side (step 30b) simultaneously.

30.	UE A sends SIP ACK towards terminating side. P-CSCF A forwards the SIP ACK towards terminating side.

After receiving the SIP ACK, the UE B is able to send regular media.

## ---End of the Change---