3GPP TSG-WG SA2 Meeting #166 Ad-Hoc	S2-2500319

# 20 - 24 January, 2025, Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0279 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on local offloading management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. There exists several editorial mistakes in clause 6.10 of TS 23.548. 2. There is a remaining Editor’s Note in clause 6.10.3.1 as follows: Editor's note: whether the AF traffic influence information is transferred to I-SMF is FFS. As specified in clause 6.10.3.2, The Local Offloading Management Policy is generated by PCF and retrieved by SMF for Local Offloading Management.The SMF forwards the received Local Offloading Management Policy or Offload Identifier to the I-SMF transparently. Therefore, this EN has already been solved, and it is proposed to remove it directly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Editorial changes. 2. Removing an EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remaining editorial mistakes and unnecessary EN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.2.2, 6.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.10	Support of the Local Offloading Management

6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading Management allowed indication to SMF, the SMF does not discover, select or configure EASDF, nor perform DNS message handling for the locally offloaded traffic towards the local part of DN.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

# * * * * Second change * * * *

6.10.2	Procedure

6.10.2.1	General

This clause describes EAS (re)discovery procedure within PDU session applied for Local Offloading Management.

6.10.2.2	I-SMF insertion during the PDU Session establishment

Figure 6.10.2.2-1: Procedure for PDU Session establishment supporting Local Offloading Management

1.	During the Registration procedure, the AMF receives the Local Offloading Management allowed indication per DNN/S-NSSAI as part of SMF selection subscription data from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

If Local Offloading Management is allowed for the requested DNN/S-NSSAI in the step 1 of clause 4.3.2.2.1 of TS 23.502 [3], during the PDU session establishment procedure the AMF selects a SMF that supports Local Offloading Management. For the selected SMF, the AMF executes as follows:

-	If the UE is not within the local offloading management service area , then the I-SMF for local offloading management is not needed and step 2a is executed.

-	If the UE is within the local offloading management service area, then the I-SMF for local offloading management is needed and step 2b is executed.

2a.	Same as the step 1-6 defined in Figure 6.2.3.2.2, the SMF selects an EASDF and instructs the EASDF to create DNS context for PDU session. Step 3-4 are skipped.

2b.	The AMF selects an I-SMF that serves the UE location and supports Local Offloading Management. The PDU Session Establishment procedure as described in clause 6.7.2.2 is used with the following differences and additions:

-	This procedure applies for non-roaming PDU session and all NFs are located in HPLMN.

-	The V-SMF and V-UPF are replaced by I-SMF and I-UPF, respectively.

-	The H-SMF and H-UPF are replaced by SMF and UPF(PSA), respectively.

-	The AMF sends a Local Offloading Management allowed indication to the I-SMF in the step 3a of clause 4.3.2.2.2 of TS 23.502 [3] with V-SMF replaced by I-SMF.

If the I-SMF receives the Local Offloading Management allowed indication from AMF, the I-SMF:

-	Selects UL CL/BP UPF and L-PSA UPF based on UE location information.

-	Selects an EASDF.

-	Obtains the EASDF IP address based on local configuration.

-	May obtain EASDF DNS security information based on local configuration or via interaction with EASDF.

-	Sends the request for the establishment of the PDU Session, Local Offloading Management support indication, IP address and (if exists) DNS security information of EASDF and I-SMF supported DNAI(s) to the SMF in the Nsmf_PDUSession_Create Request in the step 6 of the procedure in figure 4.3.2.2.2-1 of TS 23.502 [3] with V-SMF, H-SMF replaced by I-SMF, SMF respectively.

When the SMF receives the Local Offloading Management support indication and allows I-SMF to perform Local Offloading Management based on SM subscription data, the SMF:

-	Skips the EASDF Discovery and Selection procedure and DNS message handling.

-	Retrieves the Local Offloading Management Policy (i.e. FQDN/IP range allowed to be routed to the local part of DN as described in clause 6.4 of TS 23.503 [4]) from PCF by providing local offloading Management support indication. Each Local Offloading Management Policy is provided with an Offload Identifier which is assigned by PCF.

-	Sends the retrieved Local Offloading Management Policy or Offload Identifier to the I-SMF.

-	Sends IP address and optional DNS security information of the EASDF received from the I-SMF to the UE via PCO.

-	Sends Local Offloading Management Allowed indication to the I-SMF.

NOTE:	Local Offloading Management Allowed indication can be part of SMF selection subscription data and Session Management Subscription data. UDM implementation are responsible for keeping the consistency of this information.

3.	The I-SMF configures the EASDF with the DNS handling rules using the Local Offloading Information received from SMF.

If the I-SMF obtains the EASDF IP address based on local configuration, the I-SMF invokes Neasdf_DNSContext_Create Request to configure the EASDF.

The I-SMF configures the UL CL UPF and local PSA UPF selected in the step 2 to forward DNS messages to EASDF.

4.	EAS Discovery procedure with EASDF is performed as described in clause 6.10.2.3.

# * * * * Third change * * * *

6.10.3	Policy provisioning for Local Offloading Management

6.10.3.1	General

For Local Offloading Management, traffic routing related policy (e.g., local offloading management policy) need be provided to the I-SMF.

Based on the received policy information and EAS Deployment information, the I-SMF may (re)discover and (re)select ULCL and local PSA UPF for routing the traffic locally.

NOTE:	The AF does not need to be aware of whether Local Offloading Management is applied or not.

# * * * * End of changes * * * *