3GPP TSG-WG SA2 Meeting #166 Ad Hoc-e	   S2-2500333

20th – 24th January, 2025, Online

Source:	China Mobile

Title:	Removing one EN related to BS reader selection in KI#1

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes methods for BS reader selection.

# Discussion

These conclusions related to AIOT RAN architecture are documented In clause 6.4.1 in RAN TR38.769 v2.0.1:

- Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

- For topology 1, the XXAP is terminated at an A-IoT RAN node.

- In Topology 1, an A-IoT RAN node may serve one or more readers.

ATOT RAN node and BS reader terms are suggested to keep consistent with RAN. The BS reader is equal to the reader description in RAN and is used to distinguish the UE reader.  And NGAP is terminated at an AIOT RAN node, not BS readers. At the same time, an AIOT RAN Node may service one or more BS readers.

Proposal 1:  Adding one note: For topology 1, the NGAP is terminated at an AIOT RAN node, and an AIOT RAN Node may service one or more BS readers.

In topology 1, the AIoTF can select one or multiple targeted AIoT RAN Nodes based on matching between location information in the service request from AF via NEF and AIOT RAN Node serving location information.

In direct connection mode, an AIOT RAN Node should perform lite-NGAP procedure towards AIOTF to register itself into AIOTF successfully.

In indirect connection mode, an AIOT RAN node should perform lite-NGAP procedure towards AMF, once it is registered into AMF successfully, AMF should forward AIOT RAN Node relevant information, such as AIOT RAN node status, AIOT RAN node serving location information, to AIOTF via Nz interface, Simultaneously, AIOTF should send subscriber/Notification request via Nz interface towards AMF for AIOT RAN node status update.

Note 1: AIOT RAN Node may obtain the serving AIOT NF by OAM configuration in direct connection mode. AMF may obtain serving AIOT NF by querying NRF or OAM configuration in indirect connection mode.

AIOTF in direct connection mode or AMF in indirect connection mode may obtain AIOT RAN Node’s serving location by lite-NGAP procedure or OAM configuration. And AIOT RAN Node may inform 5GC of its AIOT serving location per AIOT RAN granularity or per reader granularity.

the AF need provide the expected location information list to NEF in the ambient IOT service request. The location information can be geographic location or external location identifier pre-negotiated between operator and the AF. NEF can convert geographic location or the external location identifier to internal location identifier (TA list or cell list). NEF can query NRF to obtain serving AIOTF based on internal location identifier. Then NEF will forward the ambient service request to AIOTF with internal location identifier.

Note 2: The external identifier definition and negotiation between operator and the AF is out of the scope of 3GPP.

AIOTF can select AIOT RANs with location information or with readers information depending on the AIOT RAN serving location granularity from AIOT RAN:

If AIOT RAN inform AIOTF of serving location information on AIOT RAN granularity, AIOT will select targeted AIOT RAN Nodes by matching between location information in the service request from AF via NEF and AIOT RAN Node serving location information.  Once the targeting AIOT RAN Node is selected, AIOTF will forward the ambient IOT service requests to it with internal location identifier list. AIOT RAN , which receive the ambient IoT service request, can make the decision to choose readers based on internal location identifier list info.

If AIOT RAN inform ATOTF of serving location information on reader granularity, AIOT will select both targeted AIOT RAN Nodes and readers by matching between location information in the service request from AF via NEF and AIOT RAN Node serving location information. Once the targeting AIOT RAN Node is selected, AIOTF will forward the ambient IOT service requests to it readers list. AIOT RAN , which receive the ambient IoT service request, can choose the readers decided by AIOTF to execute AIOT service operations.

In the case, there is no matching between location information in the service request from AF and AIOT RAN Node serving location information, the AIOTF shall reject the service request with appropriate cause.

In the case, there are multiple AIOT RAN Nodes matching between location information in the service request from AF and AIOT RAN Node serving location information, the AIOTF shall send the ambient IOT service request to these matched AIOF RAN nodes simultaneously.

Proposal 2:   For topology 1, the AIOTF selects the AIOT RAN nodes, and the BS readers list or locations list information.

NOTE X:     If AIOTF obtains BS readers' information served by the AIoT RAN node and their serving location information from AIOT RAN,  AIOTF can inform AIOT RAN of BS readers directly. If AIOTF obtains only serving location information from AIOT RAN, AIOTF needs to inform AIOT RAN of location information to assist BS reader selection in AIOT RAN.

NOTE X:     BS Reader selection needs coordination between SA WG2 and RAN WG(s)

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

8.1	Interim Conclusion on Key Issue #1

8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the AIOT RAN nodes, and the BS readers list or locations list information.  For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

NOTE X:     For topology 1, the NGAP is terminated at an AIOT RAN node, and an AIOT RAN Node may service one or more BS readers.

NOTE X:     If AIOTF obtains BS readers' information served by the AIoT RAN node and their serving location information from AIOT RAN, AIOTF can inform AIOT RAN of BS readers directly. If AIOTF obtains only serving location information from AIOT RAN, AIOTF needs to inform AIOT RAN of location information to assist BS reader selection in AIOT RAN.

NOTE X:     BS Reader selection needs coordination between SA WG2 and RAN WG(s)

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

# * * * * End of changes * * * *