SA WG2 Meeting #166-Ad Hoc-e	S2-2500364

eMeeting, 20 – 24 January 2025

Source:	Ericsson

Title:	Analysis on MWAB Access control and Additional ULI

Document for:	Agreement

Agenda Item:	19.49

Work Item / Release:	VMR_Ph2 / Rel-19

Abstract of the contribution: This contribution discusses the MWAB access control and additional ULI based on LS response from RAN3.

# Background

RAN3 has provided LS reply in S2-2500052/R3-247909 which addressed the MWAB Access Control and Additional ULI aspects. The LS reply highlights two things:

1) For the prevention of MWAB-UE accessing MWAB-gNB, RAN3 agreed that during mobility, the target MWAB-gNB is responsible to fail the HO preparation for MWAB-UE based on S-NSSAI. The discussion for restriction of MWAB-UE initial accessing MWAB-gNB is still ongoing.

Observation 1: During HO of the MWAB-UE, the target MWAB-gNB is responsible for the HO preparation based on supported S-NSSAIs.

Proposal 1: SA2 to improve the description of MWAB-gNB configuration and referring to RAN specs for details.

2) For the Additional ULI for a UE accessing via MWAB-gNB, RAN3 indicates that MWAB-gNB can provide Additional ULI consisting of TAI and NR CGI, regardless of whether the MWAB-UE and MWAB-gNB are in the same PLMN or different PLMNs. RAN3 further indicates that both option1 and option 3 provided by SA2 can be supported. RAN3 is still checking the case of MWAB-UE accessing via NTN.

Observation 2: RAN3 decided that the Additional ULI consists of TAI and NR CGI.

Observation 3: RAN3 considers that both option 1 (i.e. TAI/NR CGI in Additional ULI is from a PLMN/SNPN that is same as the one in TAI/NR CGI in the ULI) and option 3 (i.e. TAI/NR CGI in Additional ULI is from a PLMN/SNPN that is different from the one in TAI/NR CGI in the ULI) can be supported.

# Discussion on TAI/NG CGI in Additional ULI

During the study phase, SA2 has studied 3 different options to formulate the Additional ULI. Based on LS response and observation 2 and 3 above, it’s clear that SA2 needs to decide if option 1 or option 3, or both shall be supported for the formulation of Additional ULI when MWAB-UE is registered in a PLMN/SNPN that is different from the PLMN/SNPN broadcasted by MWAB-gNB.

Based on the conclusion about the support of Additional ULI when MWAB-UE is registered in a PLMN/SNPN that is same as the PLMN/SNPN broadcasted by MWAB-gNB, it clear that option 1 (i.e. TAI/NR CGI in Additional ULI is from a PLMN/SNPN that is same as the one in TAI/NR CGI in the ULI) is already supported.

With Option 3 (i.e. TAI/NR CGI in Additional ULI is from a PLMN/SNPN that is different from the PLMN/SNPN in TAI/NR CGI in the ULI), it is not clear how the Additional ULI can be applied by the CN serving the MWAB-gNB. Furthermore, certain function will not work properly (e.g. PWS).

On the other hand, with option 1, it is a great benefit if same solution can be applied even in case MWAB-gNB and MWAB-UE are in different PLMNs. However, it requires MWAB-gNB has the capability to map the TAI/NR CGI of a PLMN MWAB-UE registered (i.e. ULI of the MWAB-UE) into a TAI/NR CGI of its own. This can be done in different ways by the MWAB-gNB based on implementation with potential assistance from the OAM. RAN3 already confirmed the feasibility of such in the LS Response.

Based on above, it’s recommended to conclude the support of Additional ULI based on option 1 in case of MWAB-gNB and MWAB-UE are in different PLMNs/SNPNs.

Proposal 2: Conclude the support of Additional ULI based on option 1(i.e. TAI/NR CGI in Additional ULI is from a PLMN/SNPN that is same as the one in TAI/NR CGI in the ULI).

Proposal 3: Reply LS to RAN3 with the SA2 outcomes.

# Proposal

Based on the above observations, It is proposed to do the following:

Proposal 1: SA2 improve the description of MWAB-gNB configuration and referring to RAN specs for details. See CR S2-250365.

Proposal 2: Conclude the support of Additional ULI based on option 1(i.e. TAI/NR CGI in Additional ULI is from a PLMN/SNPN that is same as the one in TAI/NR CGI in the ULI). See CR S2-2500365.

Proposal 3: Reply LS to RAN3. See CR S2-2500363.