3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500426

Online, January 20 - 24, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	KI#3 Conclusion Update of AIoT Device NAS Layer

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This pCR discuss and update KI#1 interim conclusion to resolve the EN about AIoT Device NAS Layer.

1. Introduction

In SA2#166, regarding the AIoT Device NAS Layer, interim conclusions had been reached with some open questions left as an EN:

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

The AIoT Device NAS layer is a protocol layer between the AIoT Device and CN (i.e., AIOTF) as depicted in the protocol stacks below:

In Rel-19, the supported use cases are focused on inventory and command:

- For inventory use case, the AIOTF sends Inventory Request to AIoT-RAN. AIoT-RAN performs A-IoT Paging toward AIoT devices. The AIoT devices respond A-IoT Paging by including their permanent device IDs in Inventory Response message. As paging for UEs, the A-IoT paging request does not contain NAS layer protocol between the AIoT devices and the CN. The Inventory Response messages encoded by the AIoT devices are to be decoded by the AIOTF. And thus, it requires AIoT Device NAS layer to support Inventory Response message.

- For command use case, the Command Request and the Command Response messages are to be encoded/decoded between the AIoT devices and the AIOTF. And depends on SA3 conclusions, those messages may be further encrypted/decrypted between the AIoT devices and the AIOTF. So, it requires AIoT Device NAS layer to support Command Request and Command Response messages.

For type 1 devices do not support DO-A traffic. They can only receive requests from the network and send responses towards the network. In Rel-19, use cases other than inventory and command are not supported. From use cases point of view, there are no needs to support other messages in AIOT Device NAS layer.

[Proposal-1] Resolve the EN.

Regarding the command use case, it has been agreed on that besides the read and write, enable and disable are also required.

[Proposal-2] Add enable and disable in the AIoT Device NAS layer.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.2.1:

* * * Start of Change * * * *

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read, Write, Enable and Disable) Request and Response.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

* * * End of Change * * * *