3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500428

Online, January 20 - 24, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	KI#1#3 Conclusion Update of Aggregation

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This pCR discuss and update KI#1 and KI#3 interim conclusions to resolve the EN about Aggregation.

1. Introduction

In SA2#166, regarding the aggregation of the Ambient IoT service operation results, there are some interim conclusion reached with some open questions left as ENs.

In General parts of interim conclusion on KI#1:

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

……

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

In principles on the procedure to support AIoT services of interim conclusion on KI#3:

5).	The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier.

……

9).	Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included/aggregated in the service response(s).

According to the interim conclusions, the following observations can be found:

[Observation-1] The reader may perform aggregation.

[Observation-2] The AIOTF may perform aggregation.

[Observation-3] The aggregation is to aggregate results from multiple sources, including the removal of duplicated records.

Based on the observations above, it seems not an issue about how aggregation can be performed. However, in our view, there is still an open question:

Based on which information, the reader/AIOTF can perform the aggregation?

Regarding the benefits of the aggregation, it is clear on that it can improve the communication efficiency. The reader/AIOTF can aggregate the results from multiple sources and send them out in one message. By including the logic of removing duplicated records, it can slightly improve the efficiency in the transmission and AF handling further.

The drawback of the aggregation is on the latency, as the reader/AIOTF needs to wait for a period to buffer the results. Till the expiry of the period or the reader/AIOTF determine there will be no further results from the sources, the aggregation handling will be triggered. After the processing, the aggregation result will be sent out.

To perform the aggregation for each service operation, an aggregation period needs to be determined by the reader/AIOTF. It is not suitable to fix one aggregation period for all service requests from different AFs. The AFs may have different expectations on the aggregation period for the service requests they send out (or even without aggregation). And the reader/AIOTF may be configured locally minimal aggregation period values to ensure the communication efficiency (the value 0 could indicate that it allows no aggregation). In our view, the aggregation period in AIOTF can be determined based on the demands from the AF and the configuration in the AIOTF. And the aggregation period in the reader can be determined based on decision from the AIOTF and the configuration in the reader.

[Proposal-1] The aggregation is performed based on the aggregation period based on the information provided by the AF and configuration values in the reader/AIOTF.

Another open question is:

Does gNB perform aggregation in topology 2 for RRC based solution?

For RRC based solution, gNB is the node between the reader and the AIOTF. It can perform the aggregation if needed. However, gNB is under the remit of RAN WGs. It is up to RAN WGs to determine whether gNB should perform the aggregation or not.

In Rel-19, topology 2 has been down prioritized. SA2 does not have to push for the conclusion without the input from RAN WGs.

[Proposal-2] Keep the EN open in Rel-19. SA2 and RAN WGs can work on them in later phases.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.2.1:

* * * Start of Change * * * *

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF. The aggregation period can be determined based on the aggregation information provided by the AF and local configuration in the AIOTF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

* * * Next Change * * * *

### 8.3.4	Principles on the procedures to support AIoT services

To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported:

-	Inventory Procedure.

-	Command Procedure, to e.g. transfer AF AIoT Data to/from AIoT Device(s) as AIoT specific NAS messages.

There are requests that are used from the AIOTF towards the Reader and responses from the Reader to the AIOTF. The routing of the request and response messages and their encoding depends on the topology and transport to the Reader (see KI#1).

All the procedures follow have the following steps:

1.	The AF makes a service request to the NEF, including parameters to identify the target AIoT Device(s), target Readers and service operation specific parameters.

2.	The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF.

3.	For the requested operation the AIOTF:

1).	Performs initial reader selection by either:

-	The AF providing information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader to use for the request.

NOTE 1:	Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

- 	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

- 	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected then the request is rejected.

2).	Determines A-IoT Device Identification information  based on the information from the AF, to be included in the paging message on the AIoT radio interface to find the AIoT Devices. AIoT Devices compare the A-IoT Device Identification information  with their own AIoT Device Identifier (part of or full AIoT Device Identifier) to determine whether respond to the paging message.

Editor's note:	Whether and how the A-IoT Device Identification information will be security protected will be concluded by SA WG3.

3).	Determines Reader Assistance information required for the operation used to the Reader, taking into account assistance information from the AF.

4).	Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection.

Editor’s note:	If the “command-only” case applies, pending SA WG3, if a command and paging can be performed in a single operation, then an AIoT specific NAS message may be included in the request.

See clause 8.1 for how to provide the request to a Reader.

5).	The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier. The aggregation period can be determined based on the aggregation information determined by the AIOTF and local configuration in the reader.

6).	The AIOTF may, depending on the information within the AIoT Device identifier, obtain subscription-like information from either:

-	the serving network performing the operation (either as identified by a AIoT Device Identifier or operator policy to check it been provided with information for a specific AIoT Device), or

-	another network as identified by a AIoT Device Identifier, or

-	A third party as identified by a AIoT Device Identifier.

7).	Checks if the AIoT Device is subscribed.

NOTE 2:	Whether and how AIOT Device Identifiers are verified depends on SA WG3.

8).	If the operation is a command operation, the AIOT generates a request, including an AIoT specific NAS message for the command, along with any additional information required by the Reader to execute the command, a TASK ID etc. The request is then routed to the Reader. The Reader executes the command, passing the AIoTF specific NAS message to the AIoT Device and collecting any AIoT specific NAS responses. The AIoT specific NAS responses are then routed back to the AIOTF.

NOTE 3:	Whether and how security protection is applied to the AIoT specific NAS message send to the AIoT Device and the response AIoT specific NAS message from the AIoT Device depends on SA WG3.

9).	Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included/aggregated in the service response(s).

4.	Provide the results of the operation from the NEF to the AF.

* * * End of Change * * * *