3GPP TSG-WG SA2 Meeting #166-AH-e	S2-2500442

Electronic Meeting, 20th Jan – 24th Jan, 2025

Source:	InterDigital Inc.

Title:	Discussion on providing multi-modal awareness to NG-RAN

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2, NR_XR_Ph3-Core / Rel-19

Abstract: This paper discusses how to provide multi-modal awareness to NG-RAN.

# 1. Introduction

In SP-241921 / S2-2500074, SA provided the following information to RAN, RAN2, RAN3, SA2:

- SA has discussed the RAN2(R2-2409272), RAN3(R3-245682) and SA4(S4-242223) LSs on Multimodality awareness. Based on the understanding that in R2-2409272, RAN2 agreed that multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping and that RAN3 agreed, as reported in R3-245682, that it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node, SA has reached consensus that the multimodality awareness can be achieved in the RAN by the CN providing multimodality information (i.e., the MMSID) to the NG-RAN by N2 SM information.

- SA expects SA2 to handle the above LSs and provide related specification updates based on the principle described above.

TS 23.501 includes the following statements about the Multi-modal Service ID.

-	The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements:

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application.

-	In addition to the features that are provided for the case that the data flows are associated with a single UE, the following features are provided for the case where the data flows are associated with more than one UE:

-	The AF should use the same Multi-modal Service ID in the interactions with the PCF(s) for all the involved UEs that relate to a multi-modal service. The PCF may take this information into account (e.g. to apply a specific QoS policy) when processing each AF request independently. The data flows contribute to the service experience, but are still valid stand-alone, as they are transmitted over separate PDU Sessions to/from the involved UEs.

TS 29.514 includes the following statements about the Multi-modal Service ID.

-	To provide service requirements for a multi-modal service, the NF service consumer shall follow the procedures described in clause 4.2.2.2 for initial provisioning of service information, with the following additional considerations:

-	When the multi-modal service combines several media, the NF service consumer shall provide the service information of each media within the "medComponents" attribute. The media subcomponent(s), when provided for a media component, only contain the description of the service data flow(s).

TS 29.514 specifies that multiModalId is a string and that AfAppId is a string.

This paper discusses what multimodality information needs to be provided by the CN to achieve multimodality awareness in NG-RAN.

# 2. Discussion

Observation 1: In the current (i.e. Rel-18) 5G System, the Multi-modal Service ID is provided by the AF and used only by the PCF. The Multi-modal Service ID passes through the NEF but is not used by the NEF.

Observation 2: In the current (i.e. Rel-18) 5G System, it cannot be assumed that different AFs will not assign the same Multi-modal Service ID to different multi-modal services. For example, the following operations can take place at a single PCF.

- AF#1 can assign a multiModalId of “xyz” to 5 flows of multi-modal service #1 and

- AF#2 can assign a multiModalId of “xyz” to 10 flows of multi-modal service #2.

In the Rel-18 5G System, this is not a problem because a PCF implementation can be expected to use the AF ID to differentiate between the two multi-modal services.

Observation 3: Sending only the Multi-modal Service ID (multiModalId) to NG-RAN is not sufficient for providing multi-modal awareness to NG-RAN. In the example above, the same multiModalId would be provided to NG-RAN for all 15 flows however the 15 flows are not part of the same multi-modal service.

Observation 4: Providing both the multiModalId and AfAppId to NG-RAN would provide multi-modal awareness to NG-RAN. In the example above, the same multiModalId would be provided to NG-RAN for all 15 flows however a different AfAppId would be provided for each service. Thus, NG-RAN would know which 5 flows are associated with multi-modal service #1 and which 10 flows are associated with multi-modal service #2.

Solution Proposal 1: Achieve multimodality awareness in NG-RAN by sending both the multiModalId and AfAppId to NG-RAN.

Observation 5: In home routed scenarios, the PCF that serves the flows will be part of the home network, but NG-RAN may be part of a visited network. Providing the AfAppId to the visited network provides multi-modal awareness, but privacy concerns may arise from exposing the AfAppId to the visited network. In other words, sending the multiModalId and AfAppId to the visited network tells the visited network which flows are part of the same multi-modal service but also results in the visited network knowing what AfAppId is associated the multi-modal service. It would be preferable to avoid exposing the AfAppId.

Observation 6: Instead of the PCF sending both the multiModalId and AfAppId towards NG-RAN, the PCF can send a single identifier that is based on both the multiModalId and AfAppId with concealment of at least the AfAppId to preserve the privacy of the AfAppId.

Solution Proposal 2: Achieve multimodality awareness in NG-RAN by sending an identifier to NG-RAN that is based on both the multiModalId and AfAppId but conceals at least the AfAppId.

# 3. Conclusion

If the core network sends an identifier to NG-RAN that is based on both the multiModalId and AfAppId but conceals at least the AfAppId, then multi-modal awareness in NG-RAN will be achieved and subscriber privacy will be protected.