3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500465

Online, 20th Jan 2025 - 24th Jan 2025	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#2 on the device subscription-like info (Topic#10)

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#2 on the device subscription-like information are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#2 on the subscription-like information aspect related to the ENs below.

Editor’s note:	Where to store the AIoT device subscription is FFS.

It is proposed that the AIoT device static (subscription-like) information that can be used by 5GC to check the device identity is stored in UDR as part of application data.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.2.1.

FIRST CHANGE

### 8.2.2	Subscription Management

The AIoT device may or may not have operator’s subscription data in the network. If the AIoT device doesn’t have the operator’s subscription data in the network, the corresponding related data (e.g. Device ID or credentials) is stored in the AAA server external to the network.

Subscription data for an Ambient IoT Device is required in case the Ambient IoT Device is managed by the network:

-	It is used by the AIOTF to check whether the device is subscribed.

NOTE 1:	Checking whether the device is subscribed needs coordination with SA WG3 study outcomes on validation of the ambient IoT Device identifier.

-	It is dedicated subscription data for Ambient IoT Devices, i.e. different with UE subscription data.

-	It contains at least a permanent Ambient IoT Device ID, etc.

Editor’s note:	SA WG2 will align security related materials in subscription data with SA WG3 decision later.

The Ambient IoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate where the AIoT device profile data of an Ambient IoT Device is. The AIoT device profile data includes the AIoT Device ID, allowed services (such as inventory or command), and, optionally, application-specific information. When the AIoT device profile data is managed by the 5GC, the UDR stores this data, and a specialized UDM, exclusively supporting AIoT devices, retrieves and utilizes the AIoT device profile data.

Subscription data for 3rd party AF is required:

-	It is used by the AIOTF together with SLA and operator policy at NEF to perform authorization of the AF request targeting for Ambient IoT service.

-	it is stored at UDM within 5G network.

Editor’s note:	Items within the subscription data for 3rd party AF need to be figured out later.

UE Subscription data is enhanced to include an indication that whether the UE is allowed to operate as a UE reader.

END OF CHANGES