3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500494

# Online, 20th Jan 2025 - 24th Jan 2025					                           	  revision of S2-25xxxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0283 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | I-SMF operation for N6 Delay Measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An SMF can not only update a DNS handling rule but also create it if N6 delay measurement is available. The current text however only describes update operation, which cuases misunderstanding. Further terminology alignment is neede for ECS option related operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification on SMF’s creation of DNS handling rule with consideration of N6 delay measurement Terminology alignment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

6.9	Use of N6 Delay Measurement

To enhance EAS and local UPF (re)selection by considering N6 delay measurements per pair of 5GC N6 termination point (i.e. UPF/L-PSA) and the measurement endpoint at application side, as described in clause 4.3.3 of TS 23.502 [3].

The N6 delay measurement results received from the UPF may be used by the SMF in the following way:

-	The SMF may create or update the DNS message handling rule on EASDF, e.g. to build/replace the EDNS Client Subnet (ECS) option or replace Local DNS Server to be used for a given FQDN.

-	For an already established connection to an EAS, the SMF may perform L-PSA UPF re-selection and/or an edge relocation considering N6 delay measurement and user plane latency requirement (clause 6.3.6) and then it may also trigger an EAS re-discovery procedure (clause 6.2.3.3).

>>>> End of Changes <<<<