3GPP TSG-WG SA2 Meeting #166 Ad-Hoc-e	S2-2500520

20th – 24th January, 2024, Online

Source:	China Mobile

Title:	AMF Services for Ambient IoT service

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT

Abstract: This paper defines AMF services for Ambient IoT.

1. Introduction/Discussion

This contribution is prepared to propose AMF services definition in clause 7.4.

2. Text Proposal

It is proposed to approve the following changes.

FIRST CHANGE

7   Network Functions Services

7.4   AMF Services

To support AIoT services, the AMF in indirect path of Topology1 adds a new Namf_AIoT service.

Table 7.4-1: NF Services for AIoT provided by AMF

| Service Name | Description |
| --- | --- |
| Namf_AIoT | Enables an NF consumer to acquire and transfer messages and information related to AIoT services. |

Table 7.4-2: List of AMF Services for AIoT

| Service Name | Service Operations | Operation Semantic | Known Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageTransfer | Request/Response | AIoTF |
|  | MessageSubscribe | Subscribe/Notify | AIoTF |
|  | MessageUnSubscribe | Subscribe/Notify |  |
|  | MessageNotify | Subscribe/Notify |  |

7.4.1 Namf_AIoT_MessageTransfer service operation

Service operation name: Namf_AIoT_MessageTransfer.

Description: CN NF requests to transfer downlink message to the AIoT Device or the AIoT RAN node through the AMF.

Input, Required: CN NF ID, Message Container (s).

Input, Optional: AIoT RAN node list or reader list.

Output, Required: Result indication.

Output, Optional: None.

A result indication means that whether the AMF is able to successfully send the message towards the AIoT RAN node or towards the Device via AIoT RAN node. For messages towards AIoT Device, the message container may be NAS DL Transport N2 message including Transaction ID, target AIoT Device information and AIoT service type (e.g. Inventory, Command). For messages towards AIoT RAN node, the message container may contain Transaction ID, target device information and assistance information for resource allocation. The target AIoT Device information may be AIoT Device ID list, part of the AIoT Device ID, TA list, AIoT RAN node list or reader list to identify one or multiple AIoT Devices. The assistance information may be AIoT service type, approximate number of AIoT devices, and approximate D2R message size based on AF request.

7.4.2 Namf_AIoT_MessageSubscribe service operation

Service operation name: Namf_AIoT_MessageSubscribe

Description: An NF can subscribe with the AMF to get notified of a N1 or N2 message for the AIoT Device or AIoT RAN node.

Input, Required: CN NF ID.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

The consumer NF invokes the Namf_AIoT_MessageSubscribe service operation (NF ID) on the AMF to get notified N1 message from AIoT Device, N2 message from AIoT RAN node. The N1 message may be NAS message from AIoT Device to AIoTF. The N2 message may contain Transaction ID and the aggregation result. The N2 message may also contain the AIoT RAN node information (i.e. the serving location information). If the consumer NF is allowed to subscribe message requested, the AMF creates a binding for the consumer NF to deliver subsequent Namf_AIoT_MessageNotify towards that NF.

7.4.3 Namf_AIoT_MessageUnSubscribe service operation

Service operation name: Namf_AIoT_MessageUnSubscribe.

Description: An NF can unsubscribe with the AMF to stop notifying message.

Input, Required: CN NF ID

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

The consumer NF invokes the Namf_AIoT_MessageUnSubscribe service operation (NF ID) on the AMF. The AMF deletes the binding for the consumer NF for the requested AIoT messages.

7.4.4 Namf_AIoT_MessageNotify service operation

Service operation name: Namf_AIoT_MessageNotify.

Description: The AMF uses this service operation to transfer messages towards the NFs that have subscribed (implicitly or explicitly) for the specific information.

Input, Required: AMF ID, Message Container(s).

Input, Optional:None.

Output, Required: None.

Output, Optional: None.

END OF CHANGES