3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500552

# 20-24 January, 2025, Online		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2145 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying non-3GPP devices behind 5G-RG and NAUN3 devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the case of 5G-RG the Non-3GPP devices can alaredy be identified as AUN3 or NAUN3 device. Specifcially the case of NAUN3 device, which are Non-3GPP devices connected behind a 5G-RG which are not authenticated, but for which traffic differentiation may be performed shall not be confused with the N3GPP device which are not authenticated connected behind the 5G-RG for which UIA feature is applied. Therefore in order to avoid wrong implementation it is proposed to clarify the difference and that how the distinction is performed is outside the 3GPP scope. Whether other SDO , such as BBF or Cablelabs would specify some mechanism is left to their decision an dit is transparent to 3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added in clause 4.100 , 4.10e and in Annex C a specification clarifying that the same Non-3GPP device can be considered as NAUN3 or as a device for which UIA traffic differentition is performed is outside 3GPP scope. Furthermore that the role can changed as describe in clause 4.10.e |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.10b, 4.10e, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2] , TS 23.502 [3] and TS 23.503 [4] apply. A term defined in TS 23.501 [2], TS 23.502 [3] or TS 23.503 [4] takes precedence over the definition of the same term, if any, in any other specifications.

RG Level Wireline Access Characteristics: Wireline access technology specific QoS information corresponding to a specific wireline access subscription, which is provided by the AMF to the W-AGF at RG registration.

Wireline access Control Plane protocol (W-CP): Protocol used to transport AS and NAS signalling between the 5G-RG and the W-AGF over the Y4 reference point. W-CP is specified by BBF and CableLabs. There is no assumption that W-CP refers to only a single protocol or only a specific protocol layer.

Wireline access User Plane protocol (W-UP): Protocol used to carry PDU Session user plane traffic between the 5G-RG and the W-AGF over the Y4 reference point. W-UP is specified by BBF and CableLabs. There is no assumption that W-UP refers to only a single protocol or only a specific protocol layer.

Legacy Wireline access Control Plane protocol (L-W-CP): L-W-CP is a legacy control plane protocol between the FN-RG and W-AGF. L-W-CP is specified by BBF and CableLabs. There is no assumption that L-W-CP refers to only a single protocol or only a specific protocol layer.

Legacy Wireline access User Plane protocol (L-W-UP): L-W-UP is a legacy user plane protocol between the FN-RG and W-AGF. W-UP is specified by BBF and CableLabs. There is no assumption that L-W-UP refers to only a single protocol or only a specific protocol layer.

Authenticable Non-3GPP (AUN3) device: A device that does not support NAS signalling, is connected to 5GC via a RG and can be authenticated by 5GC over the RG.

5GS specifications do not support a device using the same subscription to access 5GS as a UE and as an AUN3 device.

Non-Authenticable Non-3GPP (NAUN3) device: A device that does not support NAS signalling, is connected to 5GC via a RG and for which authentication with 5GC is not supported.

NOTE 1:	AUN3 and NAUN3 device can connect to RG through WLAN (collocated or not collocated with the RG) and/or wired Ethernet connections.

NOTE 2:	A device can operate as a UE over NG-RAN and as a AUN3 or NAUN3 via a RG or as a non-3GPP device for which QoS differentiation by 5GC is performed, if the device implements UE functionality.

# * * * * 2nd change * * * *

## 4.10b	Differentiated services for NAUN3 devices behind 5G-RG

NAUN3 devices cannot be authenticated by 5GC but may e.g. be locally authenticated by the 5G-RG using e.g. pre-shared secret. Differentiated services (QoS, network slicing) may be provided for NAUN3 devices as defined in this clause.

"Connectivity Group IDs" may be defined on the 5G-RG where each Connectivity Group ID corresponds to a separate physical or virtual port on the 5G-RG. These ports could, for example, refer to separate physical ethernet ports and/or to separate WLAN SSIDs and/or to a separate VLAN. The devices that connect to a certain logical port are considered part of the same Connectivity Group ID. How this configuration on the 5G-RG is done is out of scope of this specification.

Each Connectivity Group ID is then mapped to a separate PDU Session that is established by the 5G-RG based on the procedures defined in clause 7. The overall architecture is illustrated in Figure-4.10b-1.

Figure 4.10b-1: Example scenario for NAUN3 devices behind 5G-RG based on connectivity groups

The 5G-RG is configured with the (virtual) port information (e.g. VLANs and SSIDs) based on TR-69 [18], TR-360 and TR-181 [46]. URSP rules can be provided to the 5G-RG to indicate how to map Connectivity Group ID to the parameters of the PDU Session used to carry the traffic of corresponding devices e.g. DNN, S-NSSAI, etc.

NOTE:	In addition, the mapping between a "virtual port" and DNN/S-NSSAI can be configured via TR-69 [18]/TR-181 [46].

Whether and how the NAUN3 devices are configured to use a specific SSID or connect to a certain Ethernet port on the 5G-RG is out of scope of this specification.

Differentiation of charging and QoS may be provided via PCC rules (for different service flows) related with dedicated PDU Sessions for NAUN3 devices. Isolation of devices using a specific Connectivity Group ID into a specific network slice, i.e. with separate S-NSSAI may also be provided.

How the 5G-RG determines that a Non-3GPP device connecting behind a 5G-RG is considered as a NAUN3 or a Non-3GPP device for which QoS differentiation is performed as defined in clause 4.10e is outside the scope of 3GPP. The 5G-RG may change whether a Non-3GPP device is treated, i.e. from NAUN3 to Non-3GPP device for which traffic differention is performed based on a Non-3GPP Device Identifier and viceversa, based on mechanism not defined by 3GPP, but it is assumed that a Non-3GPP device connected behind a 5G-RG  never plays both roles simultaneously.

# * * * * 3nd change * * * *

## 4.10e	Differentiated QoS for non-3GPP devices behind 5G-RG

This clause defines the support for identifying the traffic of individual non-3GPP devices behind a 5G-RG and providing them differentiated QoS.

TS 23.501 [2] clause 5.x applies to the 5G-RG with the following deltas:

-	The UE is replaced by 5G-RG.

The overall architecture is illustrated in Figure 4.10x-1. Non-3GPP devices associated with the same PDU Session can be further differentiated using their Non-3GPP Device Identifiers. This is further described in Annex C.

How the 5G-RG determines that a Non-3GPP device connecting behind a 5G-RG is considered as a NAUN3 as defined in clause 4.10a or as a Non-3GPP device for which QoS differentiation is performed as defined in this clause is outside the 3GPP scope.

The 5G-RG may change how a Non-3GPP device is considered, i.e. from NAUN3 to Non-3GPP device for which traffic differention is performed based on a Non-3GPP Device Identifier  and viceversa based on mechanism outside the 3GPP scope, but it is assumed that a Non-3GPP connected behind a 5G-RG  never plays both roles at the same time.

Figure 4.10x-1: Example scenario for mapping traffic of individual non-3GPP devices behind 5G-RG to a PDU Session

# * * * * 4th change * * * *

Annex C (informative):
QoS differentiation of traffic of individual non-3GPP devices behind 5G-RG

This Annex describes how the traffic of individual non-3GPP devices behind a 5G-RG can be identified and provided with differentiated QoS.

Figure C-1: Example scenario for mapping traffic of individual non-3GPP devices to a PDU Session

As in this example, two non-3GPP devices mapped to PDU Session A initially used the default QoS Flow (QFI 1); when differentiated QoS is requested for one device, the 5G-RG binds its traffic to a Non-3GPP Device Identifier, and its traffic is mapped to a separate QoS Flow (QFI 2). Four non-3GPP devices mapped to PDU Session B based on their Connectivity Group ID X initially used the default QoS Flow (QFI 3); when differentiated QoS is requested for two of those four devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to separate QoS Flows (QFI 4 and QFI 5). Similarly, three non-3GPP devices mapped to PDU Session C based on their Connectivity Group ID Y initially used the default QoS Flow (QFI 6); when differentiated, but the same, QoS is requested for two of those three devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to a separate QoS Flow (QFI 7).

How the Non-3GPP device is identified by 5G-RG is considered not in the 3GPP scope and it may be either implementation specific (for example based on device MAC address) or specified by other SDOs such as BBF.

The same Non-3GPP device, as in the example, can be either considered as NUN3 device per clause 4.10b or as a Non-3GPP device for which traffic differention is performed per clause 4.10e, but it can not assume both roles at the same time. The change of role does not require any actions from the 3GPP network which only see either a NAUN3 or a Non-3GPP device Identifier and in both case the 3GPP network is not aware of any specific identifiers used by 5G-RG to identify the Non-3GPP device role.

Figure C-2 illustrates a procedure which enables the 5GS to identify the traffic of individual non-3GPP devices initially using the same PDU Session behind a 5G-RG and provide differentiated QoS.

0a.	Non-3GPP device 1 is connected to the 5G-RG.

0b.	To provide connectivity to the non-3GPP device 1, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using UE Local Configuration, to map the traffic of the non-3GPP device 1 to a PDU Session.

0c.	Non-3GPP device 2 is connected to the 5G-RG.

0d.	To provide connectivity to the non-3GPP device 2, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using UE Local Configuration, to map the traffic of the non-3GPP device 2 to the same PDU Session as non-3GPP device 1.

1.	The 5G-RG subscription owner or an authorized user, using mechanisms out of scope of 3GPP, requests differentiated QoS for the non-3GPP device 2 through the AF.

NOTE 1:	The request for differentiated service can be made through an operator portal hosted either in the 5G-RG or in the AF.

2.	AF provisions the Non-3GPP Device Identifier Information for the non-3GPP device 2 into the UDR, as defined in clause 4.15.6.15 of TS 23.502 [3].

NOTE 2:	Provisioning of the Non-3GPP Device Identifier Information into the UDR is done only for the non-3GPP devices that require differentiated QoS. This provisioning could be done before a device is connected to the 5G-RG.

3.	Based on the network configuration, either a 5G-RG requested PDU Session Modification, or a 5G-RG requested PDU Session Establishment procedure is triggered.

3a.	(5G-RG requested PDU Session Modification) As defined in clause 7.3.2.

3b.	(5G-RG requested PDU Session Establishment) As defined in clause 7.3.1.

Editor's note:	Whether the PCF initiated SM Policy Association Modification procedure as defined in clause 4.16.5.2 of TS 23.502 [3] can also be used is FFS.

4.	In response to step 1, using mechanisms out of scope of 3GPP, the operator portal returns a response to the 5G-RG subscription owner or the authorized user about the completion of the differentiated QoS request for non-3GPP device 2.

Annex X (informative):
QoS differentiation of traffic of individual non-3GPP devices behind 5G-RG

This Annex describes how the traffic of individual non-3GPP devices behind a 5G-RG can be identified and provided with differentiated QoS.

Figure X-1: Example scenario for mapping traffic of individual non-3GPP devices to a PDU Session

As in this example, two non-3GPP devices mapped to PDU Session A initially used the default QoS Flow (QFI 1); when differentiated QoS is requested for one device, the 5G-RG binds its traffic to a Non-3GPP Device Identifier, and its traffic is mapped to a separate QoS Flow (QFI 2). Four non-3GPP devices mapped to PDU Session B based on their Connectivity Group ID X initially used the default QoS Flow (QFI 3); when differentitated QoS is requested for two of those four devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to separate QoS Flows (QFI 4 and QFI 5). Similarly, three non-3GPP devices mapped to PDU Session C based on their Connectivity Group ID Y initially used the default QoS Flow (QFI 6); when differentiated, but the same, QoS is requested for two of those three devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to a separate QoS Flow (QFI 7).

How the Non-3GPP device is identified by 5G-RG is considered not in the 3GPP scope and it may be either implementation specific (for example based on device MAC address) or specified by other SDOs such as BBF.

The same Non-3GPP device, as in the example, can be either considered as NUN3 device per clause 4.10b or as a Non-3GPP device per clause 4.10x, but it can not assume both roles at the same time. The change of role does not require any actions from the 3GPP network which only see either a NAUN3 or a Non-3GPP device Identifier and in both case the 3GPP network is not aware of any specific identifiers used by 5G-RG to identify the Non-3GPP device role.

Figure X-2 illustrates a procedure which enables the 5GS to identify the traffic of individual non-3GPP devices initially using the same PDU Session behind a 5G-RG and provide differentiated QoS.

Figure X-2: Example procedure for enabling QoS differentiation for individual non-3GPP devices behind 5G-RG

0a. Non-3GPP device 1 is connected to the 5G-RG.

0b.	To provide connectivity to the non-3GPP device 1, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using UE Local Configuration, to map the traffic of the non-3GPP device 1 to a PDU Session.

0c. Non-3GPP device 2 is connected to the 5G-RG.

0d. To provide connectivity to the non-3GPP device 2, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using UE Local Configuration, to map the traffic of the non-3GPP device 2 to the same PDU Session as non-3GPP device 1.

1.	The 5G-RG subscription owner or an authorized user, using mechanisms out of scope of 3GPP, requests differentiated QoS for the non-3GPP device 2 through the AF.

NOTE 1: 	The request for differentiated service can be made through an operator portal hosted either in the 5G-RG or in the AF.

2.	AF provisions the Non-3GPP Device Identifier Information for the non-3GPP device 2 into the UDR, as defined in clause 4.15.6.X of TS 23.502 [3].

NOTE 2:	Provisioning of the Non-3GPP Device Identifier Information into the UDR is done only for the non-3GPP devices that require differentiated QoS. This provisioning could be done before a device is connected to the 5G-RG.

3.	Based on the network configuration, either a 5G-RG requested PDU Session Modification, or a 5G-RG requested PDU Session Establishment procedure is triggered.

3a. (5G-RG requested PDU Session Modification) As defined in Clause 7.3.2.

3b. (5G-RG requested PDU Session Establishment) As defined in Clause 7.3.1.

Editor’s note:	Whether the PCF initiated SM Policy Association Modification procedure as defined in clause 4.16.5.2 of TS 23.502 [3] can also be used is FFS.

4.   In response to step 1, using mechanisms out of scope of 3GPP, the operator portal returns a response to the 5G-RG subscription owner or the authorized user about the completion of the differentiated QoS request for non-3GPP device 2.

# * * * * End of changes * * * *