3GPP TSG-WG SA2 Meeting #166 Ad Hoc	S2-2500585

20th – 24th January 2025, E-meeting	(revision of XXXXXX)

Source:	Lenovo, Futurewei, NEC

Title:	KI#1 Addressing EN on AIoT service responses aggregation

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to resolve the EN for the general part of the KI#1 conclusions, especially on the AIoT service responses aggregation.

1	Discussion

This paper mainly focuses on solving the existing ENs in the General part for KI#1 conclusions, i.e., the following ones on aggregation.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

1] For the first Editor’s note, the coordination with the RAN WG is needed in the future, and this issue can be further studied and revisited when the topology 2 study restarts. So, as for now, this EN can become a NOTE as below:

NOTE: For RRC based solution of topology 2, whether the aggregation can be performed by the RAN can be revisited and studied later, and coordination with RAN WGs is needed.

2] For the second Editor’s note about how to perform the aggregation, the basic principle is to aggregate the service operation results received from multiple AIoT devices for the same service that share the same service identifier (e.g., task ID), which can be done either by the AIoT reader (for both UE reader in topology 2 and AIoT RAN reader in topology 1), and/or the AIOTF.

What’s more, the AF may also provide the aggregation configuration details to the 5GS on how to conduct the aggregation, e.g., based on what kind of criteria. For example, the aggregation configuration can include time/period based aggregation (aggregate the responses for a duration), group of devices based aggregation (aggregate the responses for a group of devices, or for a specific number of devices, e.g., 5,000), location based aggregation, etc. The aggregation configuration details will instruct the AIoT related entities that includes at least the AIOTF and the AIoT reader (for both UE reader in topology 2 and AIoT RAN reader in topology 1), on how to perform the aggregation of the service operation responses.

With the discussion above, these two Editor’s notes can be removed accordingly.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

8.1	Interim Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIOTF can aggregate the service operation results that share the same aggregation criteria (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF. The aggregation may be performed based on the configuration information provided by the AF, which may include several criteria, e.g., correlation identifier (s) which can be used to correlate and aggregate service operation results, such as service ID, group ID; location information for location-based aggregation; aggregation operation restriction: such as waiting time duration for the aggregation period or time based aggregation, maximum device number or device number based aggregation, etc. The AIoT reader (BS reader and UE reader) may also perform the service operation results aggregation, based on the configuration information from AF and AIOTF.

NOTE X: For RRC based solution of topology 2, whether the aggregation can be performed by the NG-RAN (AIoT enabled) can be revisited and studied later, and coordination with RAN WGs is needed.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:  Where to store the AIOT device related information is FFS.

# * * * * End of changes * * * *