3GPP TSG-SA2 Meeting #166AH-e	S2-2500615

# 20th– 24th January, 2025, E-meeting

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5983 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of UE ATSSS capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current specification of Clause 5.32.6.1 describes steering functionalities in combination of the followings: 1) ATSSS-LL functionality with any steering mode. 2) MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. 3) MPQUICUDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. 4) MPQUIC IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. 5) MPQUIC E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. 6) MPQUIC IP functionality with any steering mode without any ATSSS-LL functionality. “MPQUIC-E functionality with any steering mode without any ATSSS-LL functionality” is a necessary option to establish Ethernet type MA PDU Session for a UE that doesn’t support ATSSS-LL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following to UE’s ATSSS capability. 7) MPQUIC-E functionality with any steering mode without any ATSSS-LL functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GS cannot support an Ethernet type MA PDU Session for a UE that doesn’t support ATSSS-LL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

5.32.6.1	General

The functionality in an ATSSS-capable UE that can steer, switch and split the MA PDU Session traffic across 3GPP access and non-3GPP access, is called a "steering functionality". An ATSSS-capable UE may support one or more of the following types of steering functionalities:

-	Steering functionalities based on multi-path protocols:

-	Steering functionality that applies the MPTCP protocol (IETF RFC 8684 [81]) and is called "MPTCP functionality" (see clause 5.32.6.2.1). This steering functionality can be applied to steer, switch and split the TCP traffic flows identified in the ATSSS/N4 rules. The MPTCP functionality in the UE may communicate with an associated MPTCP Proxy functionality in the UPF, by using the MPTCP protocol over the 3GPP and/or the non-3GPP user plane.

-	Steering functionalities based on MPQUIC that apply the QUIC protocol (IETF RFC 9000 [166], IETF RFC 9001 [167], IETF RFC 9002 [168]) and its multipath extensions (draft-ietf-quic-multipath [174]). The MPQUIC functionality in the UE may communicate with an associated MPQUIC Proxy functionality in the UPF, by using the QUIC protocol and its multipath extensions over the 3GPP and/or the non-3GPP user plane. The following MPQUIC-based steering functionalities are supported:

-	"MPQUIC-UDP" steering functionality that can be applied to steer, switch and split UDP traffic flows identified in the ATSSS/N4 rules.

-	"MPQUIC-IP" steering functionality that can be applied to steer, switch and split IP traffic identified in the ATSSS/N4 rules.

-	"MPQUIC-E" steering functionality that can be applied to steer, switch and split Ethernet traffic identified in the ATSSS/N4 rules.

-	Steering functionalities based on data switching:

-	A steering functionality called "ATSSS Low-Layer functionality", or ATSSS-LL functionality (see clause 5.32.6.3.1): This steering functionality can be applied to steer, switch and split all types of traffic, including TCP traffic, UDP traffic, Ethernet traffic, etc.

NOTE:	Filters used in ATSSS rules related with a MA PDU Session of type Ethernet can refer to IP level parameters such as IP addresses and TCP/UDP ports.

The UE indicates to the network its supported steering functionalities and steering modes by including in the UE ATSSS Capability one or more of the following capabilities:

1)	ATSSS-LL functionality with any steering mode.

In this case, the UE indicates that it is capable to steer, switch and split all traffic of the MA PDU Session by using the ATSSS-LL functionality with any steering mode allowed for ATSSS-LL, as specified in clause 5.32.8.

2)	MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. The MPTCP functionality shall not be used when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split the MPTCP traffic of the MA PDU Session by using the MPTCP functionality with any steering mode specified in clause 5.32.8; and

b)	it is capable to steer and switch all other traffic (i.e. the non-MPTCP traffic) of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode specified in clause 5.32.8.

3)	MPQUICUDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. The MPQUIC-UDP functionality shall not be used when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split the MPQUIC traffic of the MA PDU Session by using the MPQUIC functionality with any steering mode specified in clause 5.32.8; and

b)	it is capable to steer and switch all other traffic (i.e. the non-MPQUIC traffic) of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode specified in clause 5.32.8.

4)	MPQUIC IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. The MPQUIC-IP functionality shall not be used when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split all traffic of the MA PDU Session, by using the MPQUIC-IP functionality with any steering mode as specified in clause 5.32.8. and

b)	it is capable to steer and switch all traffic of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode specified in clause 5.32.8.

5)	MPQUIC E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. MPQUIC E functionality shall be used only when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split all traffic of the MA PDU Session of type Ethernet, by using the MPQUIC-E functionality with any steering mode as specified in clause 5.32.8. and

b)	it is capable to steer and switch all traffic of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode specified in clause 5.32.8.

6)	MPQUIC IP functionality with any steering mode without any ATSSS-LL functionality. The MPQUIC-IP functionality shall not be used when the type of the MA PDU Session is Ethernet. This capability shall be provided when the UE supports only MPQUIC-IP functionality.

In this case, the UE indicates that it is capable to steer, switch and split all traffic of the MA PDU Session by using the MPQUIC-IP functionality with any steering mode as specified in clause 5.32.8.

7)	MPQUIC-E functionality with any steering mode without any ATSSS-LL functionality. The MPQUIC-E functionality shall be used when the type of the MA PDU Session is Ethernet. This capability shall be provided when the UE supports only MPQUIC-E functionality.

In this case, the UE indicates that it is capable to steer, switch and split all traffic of the MA PDU Session by using the MPQUIC-E functionality with any steering mode as specified in clause 5.32.8.

The above steering functionalities are schematically illustrated in the Figure 5.32.6.1-1, which shows an example model for an ATSSS-capable UE supporting the MPTCP functionality, the MPQUIC-UDP functionality and the ATSSS-LL functionality. The MPTCP flows and the MPQUIC flows in this figure represent the traffic of the applications for which MPTCP can be applied and for which MPQUIC can be applied respectively. The five different IP addresses illustrated in the UE are further described in clause 5.32.6.2.1 and in clause 5.32.6.2.2. When the MPTCP functionality and the MPQUIC functionality are both applied, the addresses (IP@1, IP@2) used for MPTCP may be the same as the addresses (IP@4, IP@5) used for MPQUIC.

Figure 5.32.6.1-1: Steering functionalities in an example UE model

Within the same IP type MA PDU Session, it is possible to simultaneously steer different packet flows by different steering functionalities. For the same packet flow, only one steering functionality shall be used. For the same Ethernet type MA PDU Session, either ATSSS-LL steering functionality or MPQUIC-E steering functionality is enabled.

All steering functionalities in the UE shall take ATSSS decisions (i.e. decide how to steer, switch and split the traffic) by using the same set of ATSSS rules. Similarly, all ATSSS decisions in the UPF shall be taken by applying the same set of N4 rules, which support ATSSS. The ATSSS rules and the N4 rules supporting ATSSS are provisioned in the UE and in the UPF respectively, when the MA PDU Session is established.

If the UE supports multiple steering functionalities, it shall use the provisioned ATSSS rules (see TS 23.503 [45]) to decide which steering functionality to apply for a specific packet flow.

* * * * End of Changes * * * *