SA WG2 Meeting #166-Ad-Hoc-e	S2-2500644

20 - 24 January, 2025, Electronic meeting	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.xyz: NEF Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper adds NEF service definition.

1. Discussion

The paper defines the NEF services which include Inventory, Command (Read, Write, or Disable) service operations; these services are consumed by AF.

2. Proposal

It is proposed to capture the following changes into TS 23.xyz.

* * * * Start of Changes * * * *

7.4	NEF services

7.4.1	General

In addition to those defined in TS 23.501 [X] clause 7.2.8 and TS 23.502 [Y] clause 5.2.6, the following table illustrates additional NEF services to support Ambient IoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.4.2	Nnef_AIoT service

7.4.2.1	General

This service enables the consumer to request to perform inventory, command (including read, write, or disable) operation for an AIoT service or a group of AIoT devices; or receive notification of the status and results of the requested service operation.

7.4.2.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_ AIoT_Inventory

Description: The consumer requests to perform an inventory device operation for an AIoT device, a group of AIoT devices, or AIoT devices in a target area.

Input, Required: AF ID, Service operation type ('Inventory').

Input, Optional: AIoT device ID(s), AIoT device ID filter, AIoT Reader information, Target area information, Approximate number of AIoT devices, Approximate D2R message size.

Output, Required: Service status, Correlation information.

Output, Optional: None.

7.4.2.2	Nnef_AIoT_Command service operation

Service operation name: Nnef_ AIoT_Command

Description: The consumer requests to perform a command device operation for an AIoT device, a group of AIoT devices, or AIoT devices in a target area.

Input, Required: AF ID, Service operation type ('Read', 'Write', 'Disable').

Input, Optional: AIoT device ID(s), AIoT device ID filter, Target area information, AIoT device service data section (for 'Read' operation type), Service data (for 'Write' operation type), Approximate number of AIoT devices, Approximate D2R message size.

Output, Required: Service status, Correlation information.

Output, Optional: None.

7.4.2.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_ AIoT_ Notify

Description: The consumer receives notification of the status and results of the requested service operation.

Input, Required: Service operation type, Operation result indication, Correlation information.

Input, Optional: AIoT device ID(s), AIoT device service data (for 'Read' operation type).

Output, Required: Result.

Output, Optional: None.

Editor's note:  Whether other service operations are needed and their definition is FFS.

Editor's note:  The above parameter design for the service operations needs further consideration.

* * * * End of Changes * * * *