3GPP TSG-WG2 Meeting #166-Ad-Hoc-e	S2-2500657

# 20 - 24 January, 2025, Electronic meeting				                             (revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1356 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding VFL related service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The service operations for VFL currently are missing in newest specification, and this CR is proposed to add the VFL related service operations. The proposed parameters are primarily derived from the VFL procedures and the HFL service operation description in TS 23.288. Sample ID(s), indicates which data is requested, e.g. group of UEs identified by a list of Internal-Group-Ids or External-Group-Ids. VFL Accuracy check flag, identifies that the request is for using the data corresponding to the target sample ID(s) as the validation dataset to calculate the Model accuracy. (the VFL Client do not update their local model based on the intermediate model training information of this iteration) Inference target period, indicates the time interval for inference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clauses to introduce the VFL related services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL cannot be implemented since services are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.X(new), 7.Y(new), X(new), Y(new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF, NRF, SCP, ADRF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_VFLTraining | Subscribe | Subscribe / Notify | NWDAF, AF, NEF |
|  | Unsubscribe |  | NWDAF, AF, NEF |
|  | Notify |  | NWDAF, AF, NEF |
|  | Request | Request / Response | NWDAF, AF, NEF |
| Nnwdaf_VFLInference | Subscribe | Subscribe / Notify | NWDAF, AF, NEF |
|  | Unsubscribe |  | NWDAF, AF, NEF |
|  | Notify |  | NWDAF, AF, NEF |
|  | Request | Request / Response | NWDAF, AF, NEF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting HFL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |
| Signalling Storm | Analytics ID: Signalling storm | Statistics or predictions on controlling signalling storm for network abnormal behaviour mitigation or prevention. |
| QoS and Policy Assistance | Analytics ID: QoS and Policy Assistance | Analytics on predicted QoE for QoS parameter set(s) and candidate QoS parameter sets associated to the corresponding predicted QoE. |

# * * * * Next change (All new text)* * * *

7.X	Nnwdaf_VFLTraining Service

7.X.1	General

Service Description: This service is provided by an NWDAF acting as VFL client and enables an NWDAF VFL server, an AF VFL server or an NEF acting on its behalf as consumer to request the NWDAF to participate in VFL preparation and model training as VFL client.

7.X.2	Nnwdaf_VFLTraining_Subscribe service operation

Service operation name: Nnwdaf_VFLTraining_Subscribe

Description: Subscribes to NWDAF as VFL client for VFL training.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID

-	the parameters negotiated during the preparation phase (e.g. VFL Interoperability Information, final list of samples or feature ID per VFL client)

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	Analytic filter information

-	maximum response time

-	intermediate model training information

-	sample ID(s) (e.g. for training or for validation)

-	VFL accuracy check flag

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	VFL Correlation ID (e.g. confirm of the subscription for this VFL process).

7.X.3	Nnwdaf_VFLTraining_Unsubscribe service operation

Service operation name: Nnwdaf_VFLTraining_Unsubscribe

Description: Terminate NWDAF VFL training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code

7.X.4	Nnwdaf_VFLTraining_Notify service operation

Service operation name: Nnwdaf_VFLTraining_Notify

Description: NWDAF notifies the consumer of client intermediate training result of the local ML model.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	intermediate training result

-	VFL correlation ID

-	Analytics ID

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.X.5	Nnwdaf_VFLTraining_Request service operation

Service operation name: Nnwdaf_VFLTraining_Request

Editor´s note: Name of this service operation and its necessity is FFS. It is also FFS whether it can be replaced by a subscription request for the preparation.

Description: In preparation of VFL training, requests NWDAF VFL client to check if it can support requirements for VFL.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	suggested list of samples

-	Suggested VFL Interoperability Information

Inputs, Optional: none

Outputs Required:

- An indication of whether it accepts the VFL training requirements,

When the request is accepted:

- 	list of sample IDs,

- 	VFL Interoperability Information

When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

When the request is accepted:

- 	supported Feature IDs

# * * * * Next change (All new text)* * * *

7.Y	Nnwdaf_VFLInference Service

7.Y.1	General

Service Description: This service is provided by an NWDAF/AnLF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the NWDAF/AnLF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

7.Y.2	Nnwdaf_VFLInference_Subscribe service operation

Service operation name: Nnwdaf_ VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID

-	Target of VFL inference;

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	VFL inference filter

-	Analytics ID

-	inference target period

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None

7.Y.3	Nnwdaf_ VFLInference_Unsubscribe service operation

Service operation name: Nnwdaf_ VFLInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.Y.4	Nnwdaf_ VFLInference_Notify service operation

Service operation name: Nnwdaf_VFLInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information.

Inputs, Optional:

- intermediate results

- VFL correlation ID

- Analytics ID

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.Y.5	Nnwdaf_VFLInference_Request service operation

Service operation name: Nnwdaf_VFLInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference;

-	VFL Correlation ID

Inputs, Optional:

-	VFL inference filter

-	Analytics ID

Outputs, Required: If the request is accepted, then intermediate results. When the request is not accepted, an error response.

Outputs, Optional: none

# * * * * Next change (All new text)* * * *

X	AF Services to support network data analytics

X.1	General

Table X.1-1 illustrates the AF Services to support network data analytics.

Table X.1-1: NF services provided by AF to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naf_VFLTraining | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  | Unsubscribe |  | NWDAF, NEF |
|  | Notify |  | NWDAF, NEF |
|  | Request | Request / Response | NWDAF, NEF |
| Naf_VFLInference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |

X.2	Naf_VFLTraining Service

X.2.1	General

Service Description: This service is provided by an AF acting as VFL client and enables an NWDAF VFL server or an NEF acting on its behalf as consumer to request the AF to participate in VFL model training as VFL client and train a local model for a feature as defined in clause 6.2H.2.3

For VFL, this service may also be used by the consumer (i.e. VFL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

X.2.2	Naf_VFLTraining_Subscribe service operation

Service operation name: Naf_VFLTraining_Subscribe

Description: Subscribes to AF as VFL client for VFL training.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID

-	the parameters negotiated during the preparation phase (e.g. VFL Interoperability Information, final list of samples and feature ID per VFL client)

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	Analytic filter information

-	maximum response time

-	intermediate model training information

-	sample ID(s) (e.g. for training or for validation)

-	VFL accuracy check flag

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	VFL Correlation ID (e.g. confirm of the subscription for this VFL process).

X.2.3	Naf_VFLTraining_Unsubscribe service operation

Service operation name: Naf_VFLTraining_Unsubscribe

Description: Terminate AF VFL training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code

X.2.4	Naf_VFLTraining_Notify service operation

Service operation name: Naf_VFLTraining_Notify

Description: AF notifies the consumer of client intermediate training result of the local ML mode (for VFL).

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	intermediate training result

-	VFL correlation ID

-	Analytics ID

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

X.2.5	Naf_VFLTraining_Request service operation

Service operation name: Naf_VFLTraining_Request

Editor´s note: Name of this service operation and its necessity is FFS. It is also FFS whether it can be replaced by a subscription request for the preparation.

Description: In preparation of VFL training, requests NWDAF VFL client to check if it can support requirements for VFL.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	suggested list of samples

-	VFL Interoperability Information

Inputs, Optional: none

Outputs Required:

- An indication of whether it accepts the VFL training requirements,

When the request is accepted: list of sample IDs, VFL Interoperability Information.

When the request is not accepted, an error response with cause code (e.g. AF does not meet the VFL training requirements

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

When the request is accepted:

- 	supported Feature IDs

X.3	Naf_VFLInference Service

X.3.1	General

Service Description: This service is provided by AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

X.3.2	Naf_VFLInference_Subscribe service operation

Service operation name: Naf_VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID

-	Target of VFL inference;

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	VFL inference filter

-	Analytics ID

-	Inference target period

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None

X.3.3	Naf_VFLInference_Unsubscribe service operation

Service operation name: Naf_ VFLInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

X.3.4	Naf_VFLInference_Notify service operation

Service operation name: Naf_VFLInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information.

Inputs, Optional:

- intermediate results

- VFL correlation ID

- Analytics ID

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

X.3.5	Naf_VFLInference_Request service operation

Service operation name: Naf_VFLInference_Request

Description: The consumer requests the AF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference;

-	VFL Correlation ID

Inputs, Optional:

-	VFL inference filter

-	Analytics ID

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional: none

# * * * * Next change * * * *

Y	NEF Services to support network data analytics

Y.1	General

Table Y.1-1 illustrates the NEF Services to support network data analytics.

Table Y.1-1: NF services provided by NEF to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_VFLTraining | Subscribe | Subscribe / Notify | NWDAF, AF |
|  | Unsubscribe |  | NWDAF, AF |
|  | Notify |  | NWDAF, AF |
|  | Preparation | Request / Response | NWDAF, AF |
| Nnef_VFLInference | Subscribe | Subscribe / Notify | NWDAF, AF |
|  | Unsubscribe |  | NWDAF, AF |
|  | Notify |  | NWDAF, AF |
|  | Request | Request / Response | NWDAF, AF |
| Nnef_VFLNFDiscovery | NwdafDiscovery | Request / Response | AF |
|  | NwdafRelease | Request / Response | AF |

Y.2	Nnef_VFLTraining Service

Y.2.1	General

Service Description: This service is provided by an NEF on behalf of an AF acting as VFL client and enables an NWDAF VFL server as consumer to request the AF to participate in VFL model training as VFL client and train a local model for a feature as defined in clause 6.2H.2.3

For VFL, this service may also be used by the consumer (i.e. FL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

Y.2.2	Nnef_VFLTraining_Subscribe service operation

Service operation name: Nnef_VFLTraining_Subscribe

Description: Subscribes to AF as VFL client for VFL training.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID

-	the parameters negotiated during the preparation phase (e.g. VFL Interoperability Information, final list of samples and feature ID per VFL client)

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	Analytic filter information

-	maximum response time

-	intermediate model training information

-	sample ID(s) (e.g. for training or for validation)

-	VFL accuracy check flag

-	external NWDAF ID(s)

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	VFL Correlation ID (e.g. confirm of the subscription for this VFL process).

Y.2.3	Nnef_VFLTraining_Unsubscribe service operation

Service operation name: Nnef_VFLTraining_Unsubscribe

Description: Terminate AF VFL training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code .

Y.2.4	Nnef_VFLTraining_Notify service operation

Service operation name: Nnef_VFLTraining_Notify

Description: NEF notifies the consumer of intermediate training result of the local ML mode.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	intermediate training result

-	VFL correlation ID

-	Analytics ID

-	external NWDAF ID(s)

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Y.2.5	Nnef_VFLTraining_Request service operation

Service operation name: Nnef_VFLTraining_Request

Editor´s note: Name of this service operation and its necessity is FFS. It is also FFS whether it can be replaced by a subscription request for the preparation.

Description: In preparation of VFL training, requests NEF to check at untrusted AF acting as VFL client if it can support requirements for VFL.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	suggested list of samples

-	Suggested VFL Interoperability Information

Inputs, Optional:

-	external NWDAF ID(s)

Outputs Required:

- An indication of whether it accepts the VFL training requirements,

When the request is accepted: list of sample IDs, VFL Interoperability Information.

When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the VFL training requirements

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

When the request is accepted:

- 	supported Feature IDs

Y.3	Nnef_VFLInference Service

Y.3.1	General

Service Description: This service is provided by an NEF on behalf of an AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Y.3.2	Nnef_VFLInference_Subscribe service operation

Service operation name: Nnef_ VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID

-	Target of VFL inference;

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	VFL inference filter

-	Analytics ID

-	inference target period

-	external NWDAF ID(s)

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None

Y.3.3	Nnef_VFLInference_Unsubscribe service operation

Service operation name: Nnef_ VFLInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: external NWDAF ID(s)

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Y.3.4	Nnef_VFLInference_Notify service operation

Inputs, Required:

- Notification Correlation Information.

Inputs, Optional:

- intermediate results

- VFL correlation ID

- Analytics ID

- external NWDAF ID(s)

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Y.3.5	Nnef_VFLInference_Request service operation

Service operation name: Nnef_VFLInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference;

-	VFL Correlation ID

Inputs, Optional:

-	VFL inference filter

-	Analytics ID

-	external NWDAF ID(s)

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional: none

Y.4	Nnef_VFLNFDiscovery Service

Y.4.1	General

Service Description: This service is provided by an NEF towards an untrusted AF acting as VFL server to enable the AF to interact with NWDAFs acting as VFL client. It enables the AF to detect NWDAFs as VFL clients as described in in clause 6.2H.2.1.

Y.4.2	Nnef_VFLNFDiscovery_NwdafDiscovery service operation

Service operation name: Nnef_VFLNFDiscovery_NwdafDiscovery

Description: The consumer requests the NEF to discover an NWDAF that is to act as VFL client.

Inputs, Required:

- Analytics ID

- required NF type (i.e. NWDAF type)

-	VFL capability type (i.e. VFL client),

Inputs, Optional:

-	required feature IDs

-	Time Period of Interest

-	Area or Interest;

Outputs, Required: If the request is accepted, external NWDAF ID(s). When the request is not accepted, an error response.

Outputs, Optional:

-	none.

Y.4.3	Nnef_VFLNFDiscovery_NwdafRelease service operation

Service operation name: Nnef_VFLNFDiscovery_NwdafDiscovery

Description: The consumer informs the NEF that it no longer wants to use an assigned temporary NWDAF ID.

Inputs, Required:

-	external NWDAF ID

Inputs, Optional:

-	None;

Outputs, Required:.

-	None;

Outputs, Optional:

-	none.

# * * * * End of changes * * * *