3GPP TSG SA WG2#166AHE	S2-2500664

# E-meeting, Jan. 20 -24, 2025		(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0290 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Clarification on EAS Discovery using IP replacement to support Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on TR conclusion for KI#1, the Local Offloading Policy is sent to I-SMF. The I-SMF can configure the UL CL and L-PSA UPF based on the Local Offloading Policy. Existing description for HR-SBO can be reused to support the EAS discovery procedure with EASDF/Local DNS Server using IP replacement mechanism for supporting Local Offloading Management with some clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description on how to support EAS discovery procedure with EASDF/Local DNS Server using IP replacement mechanism for Local Offloading Management. |  |  |  |  |  |  |  |  |
| ` |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS discovery procedure with EASDF/Local DNS Server using IP replacement mechanism for supporting Local Offloading Management is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 6.10.2.X	EAS discovery procedure with EASDF/Local DNS Server using IP replacement mechanism for supporting Local Offloading Management

When the IP replacement mechanism is used for EAS discovery in Local Offloading Management scenario, theprocedure described in clause 6.7.2.5 is reused with following differences:

- the H-SMF is replaced by SMF;

- the V-SMF is replaced by I-SMF;

- the VPLMN Specific Offloading Information is replaced by Local Offloading Information;

- the V-EASDF is replaced by EASDF;

- the UPF in VPLMN is replaced by L-PSA UPF;

- the DNS server of VPLMN is replaced by Loca DNS server;

- the UL CL/L-PSA of VPLMN is replaced by UL CL;

- step 4a is originated from UL CL;

- step 5 refers to clause 6.10.2.3.

# * * * * End of changes * * * *