3GPP TSG-WG SA2 Meeting #166-AHe	S2-2500699

# Electronic meeting, January 20-24, 2024		(revision of S2-2412905,10315,08851,10315, 11882, 12451, 12674)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5518 | rev | 7 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-satellite-UE communications when serving satellite changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, CSCN, Tencent, Honor, Thales, CATT, NTT DOCOMO, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To maintain service continuity and consider the user’s QoE when the UE serving satellite changes, the following principles serve as the baseline for implementing this feature: Assumption #1: It is assumed that the gNB, UL CL/L-PSA, and AGW are all onboard the same satellite. Assumption #2: During a change in the serving satellite, an Xn/N2 handover will be executed. To minimize user plane updates, it is proposed that the UL CL/L-PSA does not change simultaneously with the gNB. Assumption #3: The SMF shall determine when to fall back the PDU session to the ground, e.g. N2 handover, there is no UL CL on the target satellite. And if the 5GC user plane is fell back to the ground, the SMF will send early notification with empty satellite ID. Assumption #4: If the 5GC user plane fall back to ground, it is proposed that the UE-SAT-UE communication should be terminated. Therefore, to achive service continuity based on above mentioned principles: 1). SMF should send the target satellite ID to the PCF. 2). SMF may change the user plane back to the ground (e.g. N2 handover, no UL CL on the target satellite) or keep it on the source satellite (e.g. Xn handover). 3). IMS will be notified about the 5GC user plane status: - In case of falling back the PDU session to ground-PSA, IMS will be notified with an early notification, with no satellite ID; the UE-satellite-UE will be terminated - In case when UL CL/L-PSA in the source satellite is used, IMS will be notified with an early notification, with the target satellite ID; the UE-satellite-UE can be continued or terminated. The SMF will follow the DNAI sent by P-CSCF to deal with the user plane. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe the feature of when UE’s gNB onboarded serving satellite changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Voice/video call drop rate significantly increased when the serving satellite changes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | On top of Rev.6, only changes over changes is removed. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.x.y	Support for UE-satellite-UE communication when serving satellite changes

NOTE:	The following mechanism assumes gNB, UL CL/L-PSA, IMS-AGW are onboard the same satellite.

In the UE-SAT-UE communication, if one of the UE’s serving satellite changes (e.g. due to UE’s movement, or the satellite’s movement), the Xn/N2-based handover procedure will be performed to change the serving gNB from the source satellite to the target satellite. The AMF will include the target satellite ID in the Nsmf_PDUSession_UpdateSMContext Request to the SMF, and the SMF will include this satellite ID to the PCF via Npcf_SMPolicyControl.

If the SMF decides to route the PDU session to the PSA on the ground, the procedure for removing the UL CL/L-PSA as described in clause 4.3.5.5 in TS 23.502 [3] could be reused, and an early notification without satellite ID will be sent to the P-CSCF via PCF to notify the 5GC user plane status change.

If the SMF decides to retain the existing UL CL/L-PSA on the source satellite, it provides an early notification with the target satellite ID via Nsmf_EventExposure_Notify to PCF and the PCF forwards this to the P-CSCF via Npcf_PolicyAuthorisation_Notify. The IMS informs 5GC whether the UE-SAT-UE can be continued or not, as outlined in clause AX.5.2.1 of TS 23.228 [15], i.e. the PCF will receive a DNAI associated with the IMS-AGW of satellite or on ground, and an IMS-derived transport address and forward them to the SMF:

-	If the received DNAI is associated with the IMS-AGW on the target satellite, the SMF allocates the UL CL/L-PSA to the target satellite by performing a simultaneous change of UL CL and additional PSA for the PDU Session, as described in clause 4.3.5.7 of TS 23.502 [3], where the UL CL rule shall be updated with the IMS-derived transport address. A late notification is sent by SMF to the P-CSCF via PCF afterwards, as outlined in clause 4.3.6.3 of TS 23.502 [3].

-	If the received DNAI is associated with the IMS-AGW on ground, the SMF removes the UL CL/L-PSA on the source satellite by following the procedure described in clause 4.3.5.5 of TS 23.502 [3].

# * * * * End of change * * * *