3GPP TSG-WG SA2 Meeting #166AHe	S2-2500700

# Electronic meeting, January 20-24, 2025		(revision of S2-2411888, 00125)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1476 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional descriptions of UE-satellite-UE communications when satellite changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of features to support UE-satellite-UE communications for satellite communications based on the conclusions for KI#3 in TR 23.700-29. The descriptions are modified based on the parallel session agreed discussions in SA2#166 “S2-2500699_wasS2-2412905, 23501 functional descriptions” and offline discussions on 23228 mobility procedures (continuation and ground fallback) - mobility support of UE-SAT-UE communication: To ensure service continuity when a UE's serving gNB satellite changes during UE-SAT-UE communication , the P-CSCF determines whether the UE-SAT-UE can be continued by taking 5GC provisioned early notification and satellite ID (if applicable) into account. - AGW reallocation: AGW re-allocation in UE-SAT-UE communication involves selecting a new AGW, re-negotes transport addresses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 2 new clauses are added. - mobility support of UE-SAT-UE communication when satellite changes - AGW reallocation when satellite changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-SAT-UE communication cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AY.3.y(new), AY.4.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# AX.3	Optimized media routing activation

## AX.3.y	At change of satellite

To support service continuity when one of the UE’s serving gNB satellite in the UE-SAT-UE communication changes (e.g. UE’s movement, or satellite’s movement), the P-CSCF in the satellite change network side needs to be notified of an early notification and optionaly a target satellite ID.

The continuation/termination of the UE-SAT-UE communication decision is triggered at the satellite change network side (i.e. named as originating network side).

-	If there is a new target satellite ID received (i.e. the user plane is still kept on a satellite), the P-CSCF will further determine whether the UE-SAT-UE communication can be continued based on the identifiers of UE-1’s new serving satellite and UE-2’s serving satellite. The P-CSCF obtains the UE’s new satellite identifier from PCF as defined in TS 23.502 [94], and UE-2’s satellite identifier from local storage.

-	If there is no new target satellite ID received, (i.e. the user plane is fallback to the ground), the P-CSCF will determine to terminatethe UE-SAT-UE communication.

If the P-CSCF determines that UE-SAT-UE communication can be continued at change of satellite, the AGW shall be relocated from the old satellite to the new satellite, and a message containing the IP address is to be sent to the PCF by P-CSCF as defined in TS 23.502 [94] to establish a path on the new satellite for optimized media routing.

If the P-CSCF determines that UE-SAT-UE communication cannot be continued at change of satellite e.g. the new serving satellite ID of the UE has no ISL with the other side UE’s serving satellite, the AGWs shall be relocated from the old satellite to the ground, and a message containing IP address to the PCF as defined in TS 23.502 [94] to delete the path on the old satellite. This applies to both originating side and terminating side.

# * * * * Second change * * * *

# AX.4	Interaction with IMS AGW

## AX.4.y	At change of satellite

In case when one of the UE’s serving gNB-onboarded satellite in the UE-SAT-UE communication changes,

- A new AGW on the satellite needs to be selected if UE-SAT-UE is to be continued,

- New AGWs on the ground need to be selected on both the originating side and the terminating side if UE-SAT-UE is to be terminated.

As it is during the call session, transport address re-negotiation is needed. The SIP RE-INVITE/200 OK (RE-INVITE) may be used to exchange the updated transport addresses reserved with the newly selected AGW(s). The IMS AS is triggered by P-CSCF to send re-Invite to both the terminating side AGW and to originating side UE. Subject to operator policy and implementation, the interactions between P-CSCF and IMS AS may be internal to P-CSCF having a B2BUA capability.

# * * * * End of change * * * *