3GPP TSG-WG SA2 Meeting #166AH-e	S2-2500757

E-meeting, January 20-24th, 2025	(revision of S2-25xxxxx)

Source:	China Telecom

Title:	Discussion on the enhanced discontinuous network coverage for satellite access under S&F mode

Document for:	Agreement

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARCH / Rel-19

Abstract: This paper analyses the impact of S&F Satellite Operation on the current procedures of enhanced discontinuous network coverage for satellite access, and proposes that in S&F mode, the UE and the MME may not perform procedures of enhanced discontinuous coverage for satellite access as described in clause 4.13.8.

1. Discussion

SA#106 meeting has approved 23401 CR3801, CR3800 and CR3815 (SP-241485) which introduce the general clause, split MME architecture and full EPC onboard satellite architecture respectively for Store and Forward Satellite Operation. The enhancements introduced by S&F Satellite Operation may influence current specifications for the enhanced support of discontinuous network coverage for satellite access.

In clause 4.13.8 of TS 23.401, the enhancements to discontinuous network coverage for satellite access are as follows:

For satellite access that provides discontinuous network coverage, and in case both the UE and the network support Enhanced Discontinuous Coverage, then the Mobility Management and UE Power Saving Optimization functionality may be used. If both the UE and the network indicate support for "Enhanced Discontinuous Coverage Support" and if the UE determines it will lose coverage and will become unavailable, and decides to remain in no service during that time, then: - The UE may be able to determine, including considering current and expected future locations of the UE, a Start of Unavailability Period and/or Unavailability Period Duration for when it expects to be out of coverage. NOTE 1: A UE, based on implementation, can combine successive periods of no satellite coverage into a single continuous period that is notified to the network if the UE does not require network access during this period. NOTE 2: UE informing the network of coverage gaps would increase signalling and UE power consumption if coverage gaps are more frequent than UE's communication period. - The UE triggers a TAU procedure and includes an indication of upcoming loss of coverage in the TAU Request message to the MME. If the UE is able to determine a Start of Unavailability Period and/or Unavailability Period Duration it also includes them in the TAU Request message. - The UE should trigger the TAU procedure early enough such that the procedure, under normal conditions, is able to complete before the start of the unavailability period. - The UE and the MME re-negotiate unavailability at every TAU procedure, if it is required. If Start of Unavailability Period and/or Unavailability Period Duration is not included in a TAU Request message any pending loss of coverage configuration stored in the UE context at MME is discarded. - If the UE determines an upcoming loss of coverage to the network no longer applies or determines a new Start of Unavailability Period or Unavailability Period Duration related to the upcoming loss of coverage, the UE sends a new TAU Request to the MME to update the Start of Unavailabililty Period and/or Unavailability Period Duration. Upon receiving a TAU Request message from the UE including an indication of upcoming loss of coverage or Attach request message: - If the UE did not include a Start of Unavailability Period in the TAU Request message, the MME considers implicitly the Start of Unavailability Period to be the time at which it has received the TAU Request message from the UE. If the UE included a Start of Unavailability Period, the Start of Unavailability Period indicates the time at which the UE determines it expects to lose coverage, i.e. time until which the UE determines it is available. - The MME may determine, if not provided by the UE, or update the Unavailability Period Duration and/or the Start of Unavailability Period. If the MME knows an Unavailability Period Duration and/or the Start of Unavailability Period (e.g. based on information available to the MME as described in clause 4.13.8.4) for the UE, and the UE not include an Unavailability Period Duration and/or the Start of Unavailability Period or included an Unavailability Period Duration and/or the Start of Unavailability Period different to the Unavailability Period Duration and/or the Start of Unavailability Period known to the MME, the MME should include the Unavailability Period Duration and/or the Start of Unavailability Period known to the MME in the TAU Accept or Attach Accept message and use those values in subsequent steps of this procedure. How the UE treats the MME provided Unavailability Period Duration and/or the Start of Unavailability Period is up to UE implementation e.g. to help to determine when to return to coverage after a discontinuous coverage period, whether to listen to paging in eDRX, not to initiate any NAS signalling (including Service Request for MO data) within the discontinuous coverage period in case of any UL signalling/data request or the UE may deactivate its Access Stratum functions for satellite access in order to optimise power consumption until coverage returns, etc. The MME indicates to the UE in the Attach Accept or TAU Accept whether the UE is not required to perform a TAU procedure when the unavailability period has ended. - The MME stores the information that the UE is unavailable at the Start of Unavailability Period in the UE context, and considers the UE is unreachable from then until the UE enters ECM_CONNECTED state. NOTE 3: The UE does not include indication of upcoming loss of coverage, Unavailability Period Duration and the Start of Unavailability Period in Attach Request message. If the UE requests power saving features the MME uses procedures defined in other clauses to provide the UE with timers (e.g. periodic TAU timer, extended idle mode DRX (see clause 5.13a), and PSM mode configuration (see clause 4.3.22)), and may also consider the Unavailability Period Duration (if available) and Start of Unavailability Period (if available). Unless the MME indicated that the UE is not required to perform a TAU procedure when the unavailability period has ended, then once the event which makes the UE unavailable is completed in the UE, the UE triggers a TAU procedure. If the event which makes the UE unavailable is delayed to a future time or cancelled or unavailability period deviates from negotiated value in the UE, the UE triggers TAU procedure. The MME should adjust the mobile reachable timer or Implicit Detach timer or both such that the MME does not implicitly detach the UE while the UE is out of coverage, see clause 4.3.5.2. Features described for High latency communication in clause 4.3.17.7 may be used to handle mobile terminated (MT) communication with UEs being unreachable due to satellite access with discontinuous coverage and the Unavailability Period Duration (if available) and Start of Unavailability Period (if available) may be used when determining the Downlink Buffering Duration time. The UE may send Tracking Area Update request message to inform the network of its UE unavailability period even if Mobility Management back-off timer is running.

In clause 4.13.9 of TS 23.401, an S&F Wait Timer is introduced and related contents are as follows:

When an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If a UE indicates Store and Forward capability, the MME may provide to the UE a S&F Wait Timer when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause.

Considering the above specifications for enhanced discontinuous network coverage with satellite access and for S&F Satellite Operation, we can provide an example to better illustrate the interaction between them. Figure 1-1 shows six satellites providing UE with discontinuous network coverage while operating in S&F mode, and only a subset of satellite (Satellite 1 and Satellite 4) can provide services to the UE. The UE receives an S&F Wait Timer (e.g. with duration of d3 as described in Figure 1-1) sent by Satellite 1 at Time A, which indicates that the UE can perform a subsequent NAS procedure or reattempt a NAS procedure with Satellite 4 when the S&F Wait Timer expires. Then the UE may receive a new S&F Wait Timer (e.g. with duration of d6-d3 as described in Figure 1-1) from Satellite 4, which indicates that the UE can perform a subsequent NAS procedure or reattempt a NAS procedure with Satellite 1 once the newer S&F Wait Timer expires.

Following the existing procedures of enhanced discontinuous network coverage for satellite access, if both the UE and the network indicate support for "Enhanced Discontinuous Coverage Support" and if the UE determines it will lose coverage and will become unavailable, and decides to remain in no service during that time, the UE triggers a TAU procedure and includes an indication of upcoming loss of coverage in the TAU Request message to the MME. Unless the MME indicated that the UE is not required to perform a TAU procedure when the unavailability period has ended, then once the event which makes the UE unavailable is completed in the UE, the UE triggers a TAU procedure.

Figure 1-1: Example of S&F Wait Timer(s) in the case where a subset of satellites serves the UE.

However, when only a subset of satellites serves the UE in S&F mode, the TAU procedure for enhanced discontinuous network coverage initiated during the S&F Wait Timer period may fail. This can occur, e.g. if the overhead satellite (e.g., Satellite 2 or Satellite 3 in Figure 1-1) does not have the UE context or if the UE has not yet attached to the overhead satellite.

Observation 1: In the case where a subset of satellites serve the UE under S&F mode, the TAU procedure for enhanced discontinuous network coverage initiated during the S&F Wait Timer period may fail.

When the UE is about to lose coverage from the current serving satellite (e.g., Satellite 4 in Figure 1-1), it may initiate a TAU procedure to the network, including the Start of Unavailability Period and/or Unavailability Period Duration. However, the Unavailability Period Duration refers to the interval between the current satellite's departure and the coverage provided by the next passing satellite (e.g., Satellite 5 in Figure 1-1), rather than the next serving satellite (e.g., Satellite 1 in Figure 1-1). This discrepancy can result in the Unavailability Period Duration being shorter than the S&F Wait Timer, making the Unavailability Period Duration useless for the UE and the MME when e.g. determining PSM mode configuration or setting the periodic TAU timer.

Observation 2: In the case where a subset of satellites serve the UE under S&F mode, it's quite possible that the time indicated by the Unavailability Period Duration will be shorter than that of the S&F Wait timer, making the Unavailability Period Duration useless for the UE and the MME.

In the deployment where all satellites serve the UE under S&F mode, the enhanced discontinuous network coverage procedures for satellite access can operate to negotiate the discontinuous network coverage parameters, particularly when the S&F Wait Timer is not provided considering all satellites can serve the UE. However, each TAU procedure for enhanced discontinuous coverage will lead to synchronization of UE NAS context across all other satellites, even when no user data is being transmitted, resulting in excessive signalling overhead. Operators can decide whether to enable procedures for enhanced discontinuous coverage based on, e.g. policy or size of the satellite constellation. If the procedures are not enabled, the UE can internally determine e.g. when to lose coverage and when to return to coverage after a discontinuous period using ephemeris data, while the MME can similarly configure e.g. PSM mode configuration internally based on ephemeris information.

Observation 3: In the case where all satellites serve the UE under S&F mode, the procedures of enhanced discontinuous network coverage for satellite access can work, but it may cause excessive signalling load. Operators can determine whether to enable enhanced discontinuous coverage based on e.g. policy or size of the satellite constellation. If these procedures are not performed, the UE and the MME can determine e.g. when to lose coverage and when to return to coverage after a discontinuous coverage period internally based on e.g. ephemeris information.

For full EPC onboard satellite deployment architecture in S&F mode, as specified in Annex O.3 of TS 23.401, shortly before satellite coverage will be lost, the UE can be detached. Even if the UE follows the current procedures for enhanced discontinuous coverage prior to detachment, the information e.g. the Unavailability Period Duration, the Start of Unavailability Period and the PSM mode configuration will be discarded both in UE and MME onboard after detachment. Therefore, the UE and MME do not need to perform procedures for enhanced discontinuous coverage when the satellite provides discontinuous coverage and S&F satellite operation with a full EPC onboard deployment architecture.

Observation 4: The UE and MME do not need to perform procedures for enhanced discontinuous coverage when the satellite provides discontinuous coverage and S&F satellite operation with a full EPC onboard deployment architecture.

Proposal 1: In S&F mode, the UE may not indicate "Enhanced Discontinuous Coverage Support" to the network based on e.g. local configuration. If the UE indicates "Enhanced Discontinuous Coverage Support" to the MME and the MME supports this feature, the MME decides whether to enable enhanced discontinuous coverage based on e.g. policy, deployment or size of the satellite constellation for S&F operation. For example, in cases where full EPC onboard satellite deployment architecture is adopted, or when a subset of satellites serves the UE, the MME may choose not to indicate "Enhanced Discontinuous Coverage Support" to the UE.

2. Conclusion and proposal(s)

Observation 1: In the case where a subset of satellites serves the UE under S&F mode, the TAU procedure for enhanced discontinuous network coverage initiated during the S&F Wait Timer period may fail.

Observation 2: In the case where a subset of satellites serves the UE under S&F mode, it's quite possible that the time indicated by the Unavailability Period Duration will be shorter than that of the S&F Wait timer, making the Unavailability Period Duration useless for the UE and the MME.

Observation 3: In the case where all satellites serve the UE under S&F mode, the procedures of enhanced discontinuous network coverage for satellite access can work, but it may cause excessive signalling load. Operators can determine whether to enable enhanced discontinuous coverage based on e.g. policy or size of the satellite constellation. If these procedures are not performed, the UE and the MME can determine e.g. when to lose coverage and when to return to coverage after a discontinuous coverage period internally based on e.g. ephemeris information.

Observation 4: The UE does not need to perform procedures for enhanced discontinuous coverage when the satellite provides discontinuous coverage and S&F satellite operation with a full EPC onboard deployment architecture.

Proposal 1: In S&F mode, the UE may not indicate "Enhanced Discontinuous Coverage Support" to the network based on e.g. local configuration. If the UE indicates "Enhanced Discontinuous Coverage Support" to the MME and the MME supports this feature, the MME decides whether to enable enhanced discontinuous coverage based on e.g. policy or size of the satellite constellation. For example, in cases where full EPC onboard satellite deployment architecture is adopted, or when a subset of satellites serves the UE, the MME may choose not to indicate "Enhanced Discontinuous Coverage Support" to the UE.

Corresponding 23.401 CR(S2-2500758) is submitted, incorporating the aforementioned proposal.