3GPP TSG-WG SA2 Meeting #166 -Ad Hoc-e	S2-2500765

# 20-24 Jan 2025, Emeeting

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0540 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ProSe Multihop U2N Relay reselection for Model B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 6.4.3.9.3, the 5G ProSe Intermediate UE-to-Network Relay informs the 5G ProSe Remote UE of the link failure or link quality degradation of the PC5 link so that the 5G ProSe Remote UE may initiate multihop UE-to-Network Relay reselection procedure. The correct information should be Relay re-selection indication, which is a straightforward information for 5G ProSe Remote UE to initiate multihop UE-to-Network Relay reselection procedure. Relay reselection indication proposed in this procedure is consistent with other procudures for Relay reselection, e.g. 6.7.4.2, 6.7.4.3, 6.7.4.4, in which Relay re-selection indication is sent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction on information send by the 5G ProSe Intermediate UE-to-Network Relay to 5G ProSe Remote UE. It is Relay re-selection indication with the cause (e.g. the link failure or link quality degradation of the PC5 link). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information send by the 5G ProSe Intermediate UE-to-Network Relay to 5G ProSe Remote UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new)* * * *

6.4.3.9.3	Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE and 5G ProSe Intermediate UE-to-Network Relay keep performing the measurement of the signal strength of PC5 unicast link with its parent Relay (Intermediate Relay or U2N relay) for relay reselection.

When a 5G ProSe Intermediate UE-to-Network Relay detects the link failure or link quality degradation, the 5G ProSe Intermediate UE-to-Network Relay sends to the 5G ProSe Remote UE a Relay re-selection indication with the cause (e.g. the link failure or link quality degradation of the PC5 link) so that the 5G ProSe Remote UE may initiate multihop UE-to-Network Relay reselection procedure.

The 5G ProSe Remote UE may perform the 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 to discover available 5G ProSe UE-to-Network Relays or 5G ProSe multi-hop UE-to-Network Relay Discovery procedures defined in clause 6.3.2.5.3 to discover available multihop paths to the 5G ProSe UE-to-Network Relays for 5G ProSe (multihop) UE-to-Network Relay reselection.

After the 5G ProSe multihop UE-to-Network relay reselection procedure, if the same 5G ProSe UE-to-Network Relay is used for connection with a 5G ProSe Remote UE, the existing IP address of the 5G ProSe Remote UE is reused. The IP address of the Remote UE may be included by the 5G ProSe UE-to-Network Relay and Intermediate UE-to-Network Relays in the DCA/LMA message.

After a preconfigured duration, the Relay that detected the need for reselection releases the link with the previous and next relays if applicable, unless it is reselected by the Remote UE. This will trigger them to perform the same procedure until the complete path has been released. Then, the 5G ProSe UE-to-Network Relay removes its association with the 5G ProSe Remote UE including the allocated IP address.

# * * * * End of changes * * * *