3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2500771

# E-meeting, 20 – 24 January 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5999 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on subscription information and policy control aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Editor's note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS. Regards to the above EN, the energy saving indicator is not enough for the PCF to make policy control. The PCF also need to take the energy consumption information of the UE into consideration to better decide the policies for network energy saving. So this paper proposes to add the notification from EIF to PCF to sovle this EN. - Editor's note: Whether the Energy Saving indicator is also stored in the UDR is FFS. Regards to the above EN, this paper assumes that the Energy Saving indicator could also be stored in the UDR. The related changes are in clause 5.51.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.51.5, regards to the third Editor's note, the paper proposes that the Energy Saving indicator could also be stored in the UDR. - In clause 5.51.6, regards to the Editor's note, the paper proposes that the EIF notification to PCF is needed for better policy control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Editor's notes are not solved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5, 5.51.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.51.5	Subscription information aspects

The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The Energy Saving indicator is stored in UDM/UDR. The AMF receives the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF.

Editor's note:	The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS.

Editor's note:	Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS.

5.51.6	Policy control aspects

During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.51.5. The PCF may also subscribe the notification from EIF for energy consumtion information. Based on the Energy Saving indicator and EIF notification, the PCF may make policy decisions reusing the existing parameters based on the operator policy.

# * * * * End of changes * * * *