3GPP TSG-WG SA2 Meeting #166 Ad Hoc-e	S2-2500781

20 – 24 January 2025, Online	(revision of S2-250xxxx)

# Title:	[DRAFT] Reply to LS on Satellite IDs for store-and-forward operation

# Response to:	LS (S2-2500047/R2-2411197) on <LS on Satellite IDs for store-and-forward operation> from RAN2

# Release:	Rel-19

# Work Item:	5GSAT_Ph3-ARC

Source:	[Huawei to be] SA2

To:	RAN2

Cc:

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven DOT wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 Rel-19 CR 3867 (S2-2500782)

1. Overall Description:

SA2 thanks RAN2 on the LS on Satellite IDs for store-and-forward operation. SA2 provides feedback and answers to the questions raised by RAN2 as below:

RAN2 discussed how an IoT NTN UE capable of store-and-forward operation uses the MME-configured satellite ID list in the access stratum. RAN2 made the assumption that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite.

RAN2 would like to request SA2 to provide feedback on whether the following RAN2 understanding can be confirmed:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

[SA2 answer] SA2 confirms the above RAN2 understanding is aligned with SA2. SA2 agreed that how UE behaves when receiving the satellite list is UE implementation and has updated specification accordingly (please see the attached CR for details).

Also, RAN2 would like to respectfully ask the question below to SA2:

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s):

- the satellite has the UE context but does not necessarily support store-and-forward operation; or

- the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)?

[SA2 answer] The satellites included in the satellite list are expected to be able to support the UE (e.g.  have a UE context) and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located).

2. Actions:

To RAN2 group.

ACTION: 	SA2 asks RAN2 group to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #167		17th Feb – 21st Feb, 2025			Athens, GR

TSG-SA2 Meeting #168		7th April – 11th April, 2025			Goteborg, SE