3GPP TSG SA WG2 #166-Ad-Hoc-e	S2-2500792

20 - 24 January, 2025, Electronic meeting

# Title:				Discussion on DC Application Information mismatches

# Source:				Samsung

# Document for:				Discussion

Agenda Item:				9.14.2

Work Item / Release:	NG_RTC / Rel-18

Abstract:	This paper proposes discussing the need to address the mismatch between the DC Application and binding information in the UE and in the IMS DC network.

# 1.	Background

In Release 18, IMS DC network architecture and related procedures have been specified in TS 23.228. To use a data channel service, the UE establishes a bootstrap data channel with the IMS DC netework and download the data channel applications through the boostrap data channel, and then establishes an application data channel to use an individual data channel application.

The data channel applications supported by the IMS DC network and the UEs are uploaded and matained in the DCSF and DCAR with the related information such as binding information and DC control policy.

In this paper, some issues occurring due to the mismatch between the DC Application and binding information received by the UE and available in the IMS DC network are described. It is proposed to discuss about the issues and address them in the upcoming SA2 meetings.

# 2.	Discussion

Based on current specification, the UE downloads the data channel application list or the data channel applications through the established bootstrap data channel. The DCSF provides the application list and proper data channel applications upon UE’s request.

[Clause AC.7.1 in TS 23.228 V18.8.0]

21-24:	The bootstrap data channels have been established between originating MF and UE#1/UE#2. The UEs send application request messages to MF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 and UE#2 based on their data channel capabilities and their choices through MF. (…)

In DCSF side, the DCSF retireves the data channel applications from DCAR when required, but it is not specified in detail how and who determine when it is required.

[Clause AC.2.2.2 in TS 23.228 V18.8.0]

The Data Channel Application Repository stores the verified data channel applications which are retrieved by the DCSF when required.

Meanwhile, binding information is associated with each data channel application and it is provided to the UE by the DCSF through bootstrap data channel together with the data channel applications. The UE provides the received binding information within the attribute of the SDP offer when the UE requests application data channel setup for a data channel application.

[Clause AC.5 in TS 23.228 V18.8.0]

The binding information is maintained by the HPLMN or retrieved from DC application provider when a DC Application is uploaded to DCSF. DCSF may be configured with a DC application profile associated with the binding information, which indicates the DC control policy (e.g. whether the Application DC establishment follows the P2P, P2A/A2P or P2A2P procedure). The UE receives the binding information of a DC application via the Bootstrap DC, e.g. when the DC application is downloaded to the UE.

When the UE is establishing an IMS Application DC as defined in clause AC.7.2, the binding information is provided by the UE in the attribute of the SDP offer as specified in clause 6.2.13 of TS 26.114 [76]. The IMS AS provides the binding information to the DCSF. The DCSF may use the binding information for controlling the Application DC setup.

Based on above three parts of the specification, it is observed that the data channel application and the binding information available at the DCSF may not always the same as the data channel application and the binding information downloaded by the UE. For example, the data channel application related information (e.g. version information, possible DC control policies, supporting UEs) can be updated in the IMS DC network side, but it cannot be noticed by the UE after the UE downloads the data channel applications through bootstrap data channel.

Observation: The data channel application and the binding information available at the DCSF may not always the same as the data channel application and binding information received by the UE.

This mismatch between the data channel application and related information in the UE and the IMS DC network becomes an issue, for example, when the UE tries to initiate an application data channel for a data channel application whose related information in the UE is outdated.  For example, the UE may request to the network an ADC session for a data channel application, but that data channel application has been upgraded with new version in the DCAR and the DCSF by the DC application provider. In this case it is not clear how the IMS DC network should handle this ADC session request – whether to accept the request or reject it. Additionally, if the DC application is being served by an ongoing ADC session, it is also unclear how the IMS DC network should proceed – whether to maintain the session until the UE hangs up or to release the session for hard migration. These kinds of issues occur not only in the IMS DC network, but also in the remote UE in case of P2P ADC session. It cannot be guaranteed that both the UEs have the same version of the data channel application.

Issue: This mismatch between the data channel application and related information in the UE and the IMS DC network becomes an issue, for example, when the UE tries to initiate an application data channel for a data channel application whose related information in the UE is outdated. It is not clear to the IMS DC network or the remote UE how to proceed with this session request – whether to accept or reject it, and how to handle the process.

To avoid the issues above, it is proposed to resolve in the upcoming SA2 meetings.

Proposal: It is proposed to address the issues described above with appropriate solutions in the upcoming SA2 meetings.

# 3.	Proposal

It is proposed to address the issues occuring due to the mismatch between the data channel application and related information in the UE and the IMS DC network.