3GPP SA WG2 Meeting #166AH	S2-2500882

20-24 Jan 2025, E-meeting	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0295 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN for policy provisioing for Local offloading management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following EN remains in TS23.548 Editor's note: whether the AF traffic influence information is transferred to I-SMF is FFS. The AF influence on traffic routing information is provided per DNAI by the AF to influence the traffic routing over N6 towards local DN. The SMF uses this information to generate the N4 information and provide it to the L-UPF via I-SMF. For Local Offloading Management, the SMF can still provide the N4 information (e.g. UPF event report, etc.) via I-SMF towards the L-PSA(s). Therefore there is no need to provide the AF influence on traffic routing information to I-SMF. This EN can be removed. Additional clarification is needed to make Offload Identifier as optional feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN. Clarify that the Offload Identifier is optional, even if the I-SMF is locally configured with the Offload Identifer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not completed and hard to be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.3.1, 6.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.10	Support of the Local Offloading Management

6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Management Policy from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF. as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

Next CHANGE

6.10.3	Policy provisioning for Local Offloading Management

6.10.3.1	General

For Local Offloading Management, traffic routing related policy (e.g., Local Offloading Management Policy) need be provided to the I-SMF.

Based on the received policy information and EAS Deployment information, the I-SMF may (re)discover and (re)select ULCL and local PSA UPF for routing the traffic locally.

NOTE:	The AF does not need to be aware of whether Local Offloading Management is applied or not.

6.10.3.2	Local offloading management policy handling

The Local Offloading Management Policy is generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.10.2. The Local Offloading Management Policy may also be provided with a Offload Identifier. The Offload Identifier is assigned by PCF, and is unique within the PLMN. The SMF and I-SMF for Local Offloading Management may also be preconfigured with Local Offloading Management Policy and each Local Offloading Management Policy may be tagged with related Offload Identifier(s). The PCF may only provide the Offload Identifier to the SMF.

The detailed information of Local Offloading Management Policy is described in clause 6.4 of TS 23.503 [4].The SMF forwards the received Local Offloading Management Policy or Offload Identifier to the I-SMF transparently.

End of CHANGES