SA WG2 Meeting #S2-166-Ad-Hoc-e	S2-2500889

20 - 24 January, 2025, Electronic meeting	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6030 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the policy control for network energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The AMF not only needs to send PCF the Energy Saving indicator during the AM policy association establishment, but also in AM policy association modification. There is an EN in the clause 5.51.6, Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS. The EIF can collect and calculate the UE or more finer granularity energy consumption information. Because the idea of the UE energy credit/budget (in the subscription data) based control is not agreed in this release, PCF subscription of UE level energy consumption from EIF is not helpful for policy decision making. So it proposes to remove the EN directly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN and adding modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN on the notification to PCF is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.51.6	Policy control aspects

During AM policy association establishment or modification procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.51.5. Based on the Energy Saving indicator the PCF may make policy decisions reusing the existing parameters based on the operator policy.

END OF CHANGES