3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500927

Online, 20th – 25th Jan, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Procedure for Ambient IoT Inventory Service

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: The Ambient IoT Inventory Service is introduced to the specification. In addition, the procedure will consider both direct architecture and indirect architecture.

1. Introduction/Discussion

The pCR aims to provide the procedure call flow and description of Ambient IoT Inventory Service. The procedure uses direct architecture option as the baseline with additional reference to capture the difference in indirect architecture option.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts)***

6.1	General

Editor’s Note: The contents of this general clause is FFS and is expected to describe there are service procedures which are reader type/routing agnostic and there are procedures which relate to the transports to the readers.

6.2	Ambient IoT Service Procedures

6.2.1	General

Clause 6.2.2 provides the procedure for Ambient IoT Inventory. Clause 6.2.3 provides the procedure for Ambient IoT Command.

6.2.2	Inventory Procedure

Figure 6.2.2-1 described the inventory procedure.

The inventory procedure uses procedures for AIOTF and AIoT Reader communication for communication between an AIOTF and AIoT RAN defined in clause 6.3.

Figure 6.2.2-1: Inventory Procedure

1.	The AF sends AIoT Inventory service operation request to the NEF, carrying the AF ID, information used to select AIoT Readers (e.g. Reader Set ID) and optionally device filter information as specified in clause 8.3.

The information used to select AIoT Readers includes a Reader Set ID which can either indicates an area for the Inventory Service Operation or one or more specific Reader(s) to perform the Inventory Service Operation.

NOTE 1:	The coordination between the operator and 3rd Party/AF to determine which readers are in a Reader Set is out of SA WG2 scope and can be configured via OAM.

The device information may include one or more Ambient IoT Device ID(s) or a prefix of Ambient IOT Device ID.

2.	The NEF selects AIOTF(s) based on the information used to select AIoT Readers and the OAM configuration.

3.	NEF sends the AIoT Inventory service operation request to the selected AIOTF as specified in clause 8.2.

4.	The AIOTF may obtain AF authorization information from the ADM. The AF authorization information are configured in the ADM based on operator policy and the service agreement between the AF and the operator. The AF authorization information may include the service area where the AF is allowed to request the service operation or the service operation(s) that the AF is allowed to invoke.

The AIOTF determines whether the AIoT service operation request is allowed based on the AF authorization information obtained from the ADM. If the AIoT service operation request is not authorized, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the associated identifier to be included in the paging message sent by an AIoT Reader based on the AF request, information obtained from the ADM (e.g., allowed target AIoT devices for the AF) and/or local configuration. The identifier is a full or partial Ambient IoT Device ID associated with this AF service operation request to enable identification of a single Ambient IoT Device, multiple Ambient IoT Devices or all Ambient IoT Devices.

AIOTF performs Reader Selection based on the information provided by the AF. The AIOTF determines the associated AIOT RAN based on the mapping between BS Reader and AIOT RAN. If AMF is present, the AIOTF shall also determine the associated AMF of the determined AIOT RAN.

The AIOTF may also use the last serving Reader in the subscription-like information to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4 as specified in clause 8.2.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	For each of the selected Reader or AIoT RAN, the AIOTF sends the Inventory Request message including the correlation ID, the identifier to be included in the paging message and the selected AIoT Reader(s) using the procedures defined in clause 6.3.

8.	The AIoT Reader sends an Inventory Response to the AIOTF with the correlation ID and the Reader (s) indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message, the AIoT Reader(s) will execute inventory operation by triggering the Paging and random access procedures with the Ambient IoT Devices using the procedures included in the request.

10.	If an AIoT device matches the identifier in the paging message, the AIoT Device performs random access procedures.

11.	The AIoT Device sends D2R message to the Reader which includes AIOT Device NAS. The AIOT Device NAS includes the Ambient IoT Device ID.

The AIOT Device NAS message may contain the complete Ambient IoT Device ID or a truncated Ambient IoT Device ID which is a partial Ambient IoT Device ID without the identifier in the paging message.

Steps 10 and 11 are performed for each AIoT Device matching the identifier in the paging message.

NOTE 2:	Whether additional information will be further included in the AIOT Device NAS for security mechanism will be determined and aligned with SA WG3.

12. The Reader sends an Inventory Report message to AIOTF including the correlation ID, Reader ID and the AIOT Device NAS message(s) from the AIoT Device(s). If AIOT RAN does not perform aggregation, step 12 will be performed for each AIoT Device sending the D2R message in step 11.

13.	The AIOTF performs Ambient IoT Device ID validation based on the Ambient Device Subscription information received from the ADM/AAA. If the Ambient IoT Device ID is a truncated Device ID, the AIOTF constructs the entire AIoT Device ID by adding the identifier included in the paging message associated with the correlation ID to the truncated AIoT Device ID.

The AIOTF updates the device status for this AIoT Device in the ADM, e.g. last serving AIOTF.

14.	The AIOTF completes the requested inventory. The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of Ambient IoT Permanent Device ID(s).

15.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the Ambient IoT Permanent Device ID(s).

# * * * * End of changes * * * *