3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500938

E-meeting, 20 – 24 January 2025	(revision of S2-250xxxx)

Title:	[DRAFT] LS reply on Further LS on Assistance information from CN to the reader

Response to:	LS (S2-2500078/R2-2411232) on “Further LS on Assistance information from CN to the reader” from RAN2

Release:	Rel-19

Work Item:	FS_AmbientIoT, FS_Ambient_IoT_solutions

Source:	[Huawei to be] SA2

To:	RAN2

Cc:	RAN3

Contact Person:

Name:	Runze Zhou

Tel. Number:

E-mail Address:	zhourunze AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks RAN2 for sending R2-2411232 “Further LS on Assistance information from CN to the reader”. SA2 has discussed the questions listed in the LS and would like to provide the following answers:

Q1: Is it feasible for CN to provide the following information to the reader?

- the service type of A-IoT (e.g. inventory, command)

- whether the service is targeted for a single or multiple devices;

- the approximate number of target devices (if available).

[SA2 answer]

- AIOTF in the CN can provide the service type of A-IoT (e.g. inventory, command) to the BS Reader in all cases.

- AIOTF always knows whether the service is targeted for a single or multiple devices, based on the AF provided AIoT Device ID or filtering information.

- AIOTF can provide the approximate number of target devices (if provided by AF) to the BS Reader.

Q2: Can there be a case where a D2R response is not expected for a R2D message for command service type? If so, can the CN indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type (e.g., either explicitly or implicitly by indicating the response message size if available)?

[SA2 answer]

SA2 assumes there is always a D2R response for every R2D message for command service type. In case of the “read” operation for command, the AIoT Device will send the AIoT data in the D2R response. In case of the “write/disable” operation for command, the AIoT Device will send “ACK” in the D2R response.

Further, for the latency/completion time of a given A-IoT procedure, RAN2 would also like to ask SA2 to kindly answer the following question:

Q3: Will there be any latency related requirements associated with completion time of a given A-IoT service, provided/visible to the reader?

[SA2 answer]

SA2 has no QoS requirement or key issue in the rel-19 study.

2. Actions:

To RAN2 group.

ACTION: 	SA2 kindly asks RAN2 to take the above information into future work.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #167		17th Feb – 21st Feb, 2025			Athens, Greece

TSG-SA2 Meeting #168		7th Apr – 11th Apr, 2025			Goteborg, Sweden