3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500949

# 20 – 24, January, 2025, E-meeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1575 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Third Party identity verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.228 Clause AF.1 it is mentioed the meaning of third party network which is a kind of an Enterprise or private network. This annex describes support for authorization, signing, and verification of third-party user identity information in IMS. A third party user in this context is a user belonging to a third party network which can be e.g. an Enterprise or private network. So the Clause AF.2.2.3 and AF.3 it is mentioned again the third party network can provide RCD information in the outgoing INVITE request which is technically wrong and confusing. It is actually UE or IP-PBX which sends INVITE and can send RCD information. Hence it needs to be clarified Also the authorization of the calling party to use third party identity by IMS network is little confusing as there are many nodes in IMS but they don’t have any role to play. Hence NOTE also has been simplified and the corresponding call flow as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The third party network has been replaced with UE or IP-PBX which actually sends INVITE to the network - IMS network has been clarified in the NOTE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity is stage 2 specification may lead to wrong understnading for stage 3 and implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AF.2.2.3, AF.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

### AF.2.2.3	UE and IP-PBX

Based on agreements and trust relationship between the third-party network and the IMS network, the originating UE or IP-PBX  can provide RCD information or RCD URL in outgoing initial SIP INVITE requests. The IMS network, based on operator policy, may sign the provided RCD information or RCD URL.

NOTE 1:	Based on operator policies and implementation means, the IMS network (i.e. S-CSCF or IMS AS) can authorize the UE or IP-PBX to use and  provide third party user identity information in the outgoing initial SIP INVITE request.

NOTE 2:	For the both UE and IP-PBX trunking case, the IMS network can remove third party user identity information from the SIP INVITE request, add or replace third party user identity information in the SIP INVITE request based on implementation and operator policies.

The terminating UE may present third party user identity information of the calling-party to the called party, either received from RCD information in the SIP INVITE request or fetched from the RCD server based on the received RCD URL.

# AF.3	Procedure for signing and verification of third party user identity information in IMS

Figure AF.3-1 depicts the procedure for signing and verification of third party user identity information in IMS.

Figure AF.3-1: Procedure for signing and verification of third party user identity information in IMS

1-2.	If the UE or IP-PBX  is providing third party user identity information in the outgoing initial SIP INVITE request and is authorized to use third party user identity information, and based on operator policies, S-CSCF, IMS AS or IBCF invoke signing of the provided third party user identity information with a signing AS.

NOTE 1:	Based on operator policy the INVITE may be rejected by IMS network if it determines that UE or IP-PBX is not authorized to use third party user identity.

3.	The originating IMS AS may retrieve RCD server address, RCD URL, or RCD information from the HSS, based on the originating IMPU or wildcarded IMPU.

4.	The originating IMS AS may receive RCD server address, RCD URL, or RCD information from the HSS.

5.	If the IMS AS has retrieved an RCD URL, the IMS AS, based on configuration, may retrieve the RCD information from the RCD server using the RCD URL. The IMS AS includes either the RCD URL or, if retrieved from the RCD server, the RCD information in the outgoing SIP INVITE request.

If the IMS AS has retrieved an RCD server address, the IMS AS provides the originating IMPU or wildcarded IMPU to the RCD server, to retrieve the RCD information. The IMS AS includes the retrieved RCD information in the outgoing SIP INVITE request.

NOTE 2:	How RCD URL or RCD information are carried in SIP signalling is defined in TS 24.229 [10a].

6-8.	The originating IMS network invokes signing of the RCD information or RCD URL included in the outgoing SIP INVITE request with a signing AS.

NOTE 3:	Based on operator policies, either the S-CSCF, IMS AS or IBCF in the originating network invoke signing of the RCD information or RCD URL.

9-10.	Based on operator policies, the S-CSCF, IMS AS or IBCF in the terminating IMS network invoke signature verification of the RCD information or RCD URL included in the incoming SIP INVITE request with a verification AS. The S-CSCF, IMS AS or IBCF in the terminating IMS network may reject the call when the signature verification is unsuccessful.

11-12.	The terminating UE may present third party user identity information of the calling-party to the called party either received retrieved from RCD information, if present, in the SIP INVITE request or fetched from the RCD server based on the received RCD URL.

* * * End of Changes * * * *