3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500979

Electronic Meeting, 20-24 January 2025

Source: 	Sateliot, Thales, Novamint

Title: 	On the purpose and criticality of the “List of Satellite IDs” and “S&F Monitored List” and proposal to specify separate lists.

Document for: 	Discussion

Agenda Item: 	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC / Rel-19

Abstract of the contribution: This paper discusses the purpose and criticality of the “List of Satellite IDs” and “List of Satellite IDs” and “S&F Monitored List” and makes a proposal for specification of two separate list, noting that the “List of Satellite IDs” is considered as critical to have to not impair network and UE operation while the “S&F Monitored List” is considered as nice to have to contribute to UE power optimization.

# 1	Discussion

The completion of the functional specification of the “List of satelliteID(s)” and “S&F Monitoring List” features as per conclusions in TR 23.700-29 is one of the objectives agreed as part of the Exception Sheet for 5GSAT_Ph3-ARC WI. Table 1 below provides an except of the Exception List for WT2, highlighting in yellow the part of the work that is not complete, consequences if not included and contentious issues related to the specification of the S&F satellite lists.

Table 1. Excerpt from Exception List (SP-241918)

Task(s) within work which are not complete: WT2: "Support of Store and Forward satellite operation". Points not completed: - (2a) – Functional specification of the “List of satelliteID(s)” and “S&F Monitoring List” features as per conclusions in TR 23.700-29 - (2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode - (2c) – Document in Informative annex for split-MME deployment that; support of Attach with PDN connection is handled based on implementation between the MME on board and ground CN entities. Consequences if not included in Release 19: S&F satellite operation feature not completed. In particular: - If (2a) is not included, the UE context synchronisation across multiple satellites is challenging for split-MME based deployments. - If (2b) is not specified, the UE and network expected behaviour when transitioning between operation in S&F mode and non-S&F remains undefined and may lead to mis-operation. - If (2c) is not addressed, the specification will remain unclear/ambiguous. No normative impacts are expected Contentious issues: WT; Points pending completion; Contentious issue. WT2 - Support of Store and Forward satellite operation; (2a); Different options have been discussed whether using one or two separate lists, adding “advisory”/”mandatory” indication in the list, specifying “shall or ”should” or may” UE behaviours). Further discussion is needed. Risk to not complete this during the extension is low.. (2b); None expected. However, further discussion is needed once the different features (S&F Wait Timer, S&F Satellite List(s)) get consolidated. Risk to not complete this during the extension is low.. (2c); The specification will remain unclear/ambiguous. No normative impacts are expected.

In our view, to progress on that issue, it’s fundamental to distinguish between the purpose and criticality of the two lists, which indeed were proposed separately for each of the two architectural options considered in the work (i.e. “List of Satellite IDs” information element was proposed for the split-MME architecture and the “S&F Monitored List” was proposed for the full EPC onboard architecture) but that both ended up being referred to using the single term of “S&F Monitored List”, which in our opinion is what has created the inconsistencies and controversy when trying to specify a single list to obey to the two original distinct purposes.

A first discussion on the differences between the two lists was conducted in a previous paper in SA2#166 (S2-2412119). An extension and consolidation of such discussion is provided in Table 2, highlighting the distinct purpose and criticality of the two lists.

Table 2. Assessment of the purpose and criticality of the (1) “List of Satellite IDs” information element proposed for the split-MME architecture and (2) the “S&F Monitored List” proposed for the full EPC onboard architecture.

| Type of List | Purpose | Criticality assessment |
| --- | --- | --- |
| “List of Satellite IDs” proposed for the split-MME | The need for this list is rooted in the fact that, under a deployment option based on the split-MME architecture, not necessarily all the satellites of the network shall be synchronised with the UE context. This is properly indicated the description of the split-MME architecture documented in Annex O.2 in 3GPP TS 23.401 v19.1.0, where the following note was added: NOTE: Based on implementation, the UE context can be uploaded on a subset of MME-onboard(s) or all MME-onboard(s) associated with MME-ground. To be able to limit service to a subset of satellites is critical for the scalability of the solution. The purpose of this list is to prevent UE to access to satellites operating in S&F Mode in which no UE context is found/updated. For example, one operator with e.g. 100 satellites operating in S&F mode may decide to serve each particular UE using only e.g. 10 of those satellites, which could already achieve a trade-off between the complexity of keeping the UE context synchronised in the serving satellites and the performance of the service in terms of revisit time. Therefore, if this list is provided to the UE, it means that UE context will only be uploaded in the satellites listed in the list, and the UE shall not attempt access in any satellite operating in S&F Mode which is not included in the list. Otherwise, if the UE disregards the list, the UE may end up accessing satellites with no valid UE context, getting rejections and triggering parallel attaches. | Critical for proper network and UE operation. If not supported, there is no way to inform the UE that UE context is available only in a subset of satellites. Consequently, the NW will need to keep the UE context synchronised in ALL the satellites, posing serious scalability problems. This, if not supported, UE context synchronisation across multiple satellites is challenging for split-MME based deployments and scalability of such deployments is seriously restricted. |
| “S&F Monitored List” proposed for the full EPC onboard | The purpose of this list is mainly to assist the UE in retrieving MT data. This list is mainly conceived from a UE power saving perspective. If supported, the network can restrict the delivery of MT traffic to a subset of satellites so that a UE may listen only to those satellites of the list for getting any MT traffic, which can reduce UE power consumption given that the UE can skip listening/accessing to other satellites unless they have MO traffic. | Nice to have for UE power optimization If not supported, the UE may need to listen to all the satellites of the selected PLMN to check whether there is MT traffic or not. |

# 2	Conclusion and Proposal

Based on the analysis in Table 2, we think that it’s critical to support at least the “List of Satellite IDs” in this release of the specification.

However, given the complementarity of the purposes of the two lists, we think that supporting the “S&F Monitored List” is also beneficial to provide assistance for MT traffic delivery and contribute to UE power reduction which is an important figure of merit for many IoT NTN applications.

Accordingly, a CR is submitted in S2-2500972 which proposes to include the two lists ““List of Satellite IDs” and ““S&F Monitored List”, which have been re-named as, respectively, “S&F Serving Satellite List” and “S&F Monitoring Satellite List” seeking to improve clarity and consistency of the terminology.

While contribution S2-2500972 proposes to have two separate lists, a solution converging the two lists into a common list is also seen as a feasible option. In particular, such a solution could consist of keeping the “S&F Serving Satellite List” as the single list and adding, for each satellite identifier included in the list, whether such satellite will be used to deliver MT traffic or not.