3GPP SA WG2 Meeting #166-AdHoc-e                                                        	S2-2501030

Electronic Meeting, 20-24 January 2025

Source:	Samsung

Title:	Discussion on location determination for Full EPC onboard.

Document for:	Discussion

Agenda item:	5GSAT_Ph3-ARC

# 1. Introduction

For Full EPC onboard the satellite it is not clear as to how the location verification is performed by ground network entities. This paper discusses a potential issue and proposes the way forward.

# 2. Discussion

At time T1:

Feeder link is not available and only service link is available. Original UE at the location-1 interacts with the Full EPC onboard satellite. Sends MO data to the onboard entities. Onboard entities store the data to deliver it to the target entities when connected to the ground network.

At time T2:

When feeder link is established, the proxy UE interacts with the ground network i.e. RAN node and MME, attempts to send the stored MO data to its target entities. But for RAN node and MME this proxy UE simulates as original UE. Thus the RAN node will determine the location of the proxy UE as the location of the UE i.e. the location-2. The RAN node will report the location as location-2 to the MME as part of the ULI information.

Observation: Original UE location is location-1 but RAN node will report as location-2 to MME as part of the ULI information. This is not an accurate location information for the MME.

Proposal: In S&F mode, the MME should query the coarse location information from the UE, the proxy UE should report the location of the original UE(i.e. location-1) to the MME even though proxy UE is in location-2. Additionally, if MME finds the location information reported by the UE and the RAN node are different then for accurate estimation the MME should use the location information received from the UE.

# 3. Conclusion & proposal

We propose to discuss above issue and the proposal as part of S2-2501031.

# 4. Reference

NA.