3GPP TSG SA WG2#166AH-e	            S2-2501061

Electronic Meeting, 20 – 24 January 2025	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0296 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the procedures defined for local offloading management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 6.10, there is an ambiguity on the terms “Local Offloading Management allowed indication” and “Local Offloading Management support indication”. The support indication should be SMF capability and independent of UE PDU session whereas the allowed indication should be PDU session related and be provided to SMF by I-SMF. As IP address of UE is not available at I-SMF as part of EASDF selection, EASDF IP address retrieval should follow the similar procudere as defined for HR-SBO – utilizing Neasdf_DNSContext_Create Request. In order to handle the DNS requests at the EASDF selected by the I-SMF for queries that do NOT match the FQDN/IP address allowed for local offloading management, DNS server and UPF N6 interface addresses should be provided from the SMF to I-SMF. The cases defined for UE mobility for Local Offloading Management should cover the case where UE location is not within the anchor SMF service area and either UE location is within/outside Local Offloading Management service area. Also, I-SMF insertion/change/removal require further details to clarify interaction between target and source AMFs, I-SMFs and old/new I-SMF and SMF. Similar to HR-SBO, the retrieval of Local Offloading Management Allowed indication can be part of UE context – see corresponding CR for further details S2-2500abc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changes “support indication” to “allowed indication” in clause 6.10.2.2. - EASDF IP address retrieval is aligned with HR-SBO procedure in clause 6.10.2.2. - SMF provides DNS server address and UPF N6 interface address to I-SMF to handle the traffic that is not allowed for Local Offloading Management in clause 6.10.2.2. - Further cases are defined as part of UE mobility in clause 6.10.2.4. - I-SMF insertion/change/removal is further clarified in clause 6.10.2.4 by referring to HR-SBO procedures defined for the same scenarios. - Local Offloading Management Allowed indication retrieval is updated to include UE context transfer between the target and source AMFs as in HR-SBO. - An editorial change in clause 6.10.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and incomplete specification related to Local Offloading Management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.2.2, 6.10.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area.

Local management service area is configured in AMF as a list of Local management service area ID(s).

For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

SECOND CHANGE

#### 6.10.2.2	I-SMF insertion during the PDU Session establishment

Figure 6.10.2.2-1: Procedure for PDU Session establishment supporting Local Offloading Management

1.	During the Registration procedure, the AMF receives the Local Offloading Management allowed indication per DNN/S-NSSAI as part of SMF selection subscription data from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

If Local Offloading Management is allowed for the requested DNN/S-NSSAI in the step 1 of clause 4.3.2.2.1 of TS 23.502 [3], during the PDU session establishment procedure the AMF selects a SMF that supports Local Offloading Management. For the selected SMF, the AMF executes as follows:

-	If the UE is not within the local offloading management service area , then the I-SMF for local offloading management is not needed and step 2a is executed.

-	If the UE is within the local offloading management service area, then the I-SMF for local offloading management is needed and step 2b is executed.

2a.	Same as the step 1-6 defined in Figure 6.2.3.2.2, the SMF selects an EASDF and instructs the EASDF to create DNS context for PDU session. Step 3-4 are skipped.

2b.	The AMF selects an I-SMF that serves the UE location and local offloading management service area and supports Local Offloading Management. The PDU Session Establishment procedure as described in clause 6.7.2.2 is used with the following differences and additions:

-	This procedure applies for non-roaming PDU session and all NFs are located in HPLMN.

-	The V-SMF and V-UPF are replaced by I-SMF and I-UPF, respectively.

-	The H-SMF and H-UPF are replaced by SMF and UPF(PSA), respectively.

-	The AMF sends a Local Offloading Management allowed indication to the I-SMF in the step 3a of clause 4.3.2.2.2 of TS 23.502 [3] with V-SMF replaced by I-SMF.

If the I-SMF receives the Local Offloading Management allowed indication from AMF, the I-SMF:

-	Selects UL CL/BP UPF and L-PSA UPF based on UE location information.

-	Selects an EASDF.

-	Obtains the EASDF IP address based on local configuration, or invoke Neasdf_DNSContext_Create Request including the DNN, S-NSSAI and the UE IP address set to unspecified address or to a mapped address as specified in clause 7.1.2.2 to obtain the EASDF IP address.

-	May obtain EASDF DNS security information based on local configuration or via interaction with EASDF.

-	Sends the request for the establishment of the PDU Session, Local Offloading Management allowed indication, IP address and (if exists) DNS security information of EASDF and I-SMF supported DNAI(s) to the SMF in the Nsmf_PDUSession_Create Request in the step 6 of the procedure in figure 4.3.2.2.2-1 of TS 23.502 [3] with V-SMF, H-SMF replaced by I-SMF, SMF respectively.

When the SMF receives the Local Offloading Management allowed indication and allows I-SMF to perform Local Offloading Management based on SM subscription data, the SMF:

-	Skips the EASDF Discovery and Selection procedure and DNS message handling.

-	Retrieves the Local Offloading Management Policy (i.e. FQDN/IP range allowed to be routed to the local part of DN as described in clause 6.4 of TS 23.503 [4]) from PCF by providing local offloading support indication. Each Local Offloading Management Policy is provided with an Offload Identifier which is assigned by PCF.

-	Sends the retrieved Local Offloading Management Policy or Offload Identifier to the I-SMF.

-	provides the DNS server address to be used for DNS requests related with traffic not subject to Local Offloading Management.

-	provides the PSA UPF address information (e.g. PSA UPF IP address on N6) to be used by EASDF selected by I-SMF to build EDNS Client Subnet option for target FQDN(s) of the DNS queries not subject to Local Offloading Management.

-	Sends IP address and optional DNS security information of the EASDF received from the I-SMF to the UE via PCO.

-	Sends Local Offloading Management Allowed indication to the I-SMF.

NOTE:	Local Offloading Management Allowed indication can be part of SMF selection subscription data and Session Management Subscription data. UDM implementation are responsible for keeping the consistency of this information.

3.	The I-SMF configures the EASDF with the DNS handling rules using the Local Offloading Control Information received from SMF.

If the I-SMF obtains the EASDF IP address based on local configuration, the I-SMF invokes Neasdf_DNSContext_Create Request to configure the EASDF.

The I-SMF configures the UL CL UPF and local PSA UPF selected in the step 2 to forward DNS messages to EASDF.

4.	EAS Discovery procedure with EASDF is performed as described in clause 6.10.2.3.

THIRD CHANGE

#### 6.10.2.4	UE mobility for PDU session supporting Local Offloading Management

For a PDU session supporting Local Offloading Management, during mobility events, e.g. Hand-Over or AMF change, the mobility handling is executed same as the respective mobility handling defined in clause 4.23 of TS 23.502 [3] and clause 6.7.2 with the following difference:

-	The retrieval of the Local Offloading Management Allowed indication either as part of UE context where source AMF provides the target AMF with whether Local Offloading Management is allowed for each PDU session of the UE or the SMF selection subscription data from UDM, the new AMF determines the new I-SMF as follows:

-	If the service area of SMF includes the new UE location and the UE location is out of local offloading management service area, the AMF removes the old I-SMF (if available).

-	If the service area of SMF does not include the new UE location and the UE location is out of local offloading management service area, the AMF follows the procedure defined in clause 4.23 of TS 23.502 [3] without supporting Local Offloading Management. In this case, if the old I-SMF supported Local Offloading Management and configured EASDF, the AMF sends Nsmf_PDUSession_CreateSMContext request to the SMF as in step 9 in Figure 4.23.7.3.2-1 of TS 23.502 [3], and the SMF performs EASDF selection and configuration.

-	If the service area of SMF either includes or does not include the new UE location and the UE location is within local offloading management service area, the AMF selects a new I-SMF that serves the local offloading management service area unless the existing I-SMF can serve the new UE location and local offloading management service area. If there is an old I-SMF which cannot serve the new UE location, the AMF also removes the old I-SMF.

-	To support the local offloading management, the selected I-SMF need to support Local Offloading Management, serves the new UE location and local offloading management service area; and support the DNN and S-NSSAI associated with the PDU session.

NOTE:	When the UE moves from an AMF which is not upgraded to act based on the Local Offloading Management support indication to an AMF which is upgraded to act based on the Local Offloading Management support indication, the above procedure also applies.

-	For I-SMF insertion/change/removal, the procedure described for intra (V-)PLMN HO in clauses 6.7.2.6, 6.7.2.7, 6.7.2.8, 6.7.2.9 and 6.7.2.10 is performed with the following differences:

-	removing V-PLMN, H-PLMN, and steps defined for serving PLMN change case,

-	replacing:

-	V-SMF with I-SMF,

-	H-SMF with SMF,

-	V-EASDF with EASDF,

-	V-UPF with I-UPF,

-	HR-SBO with Local Offloading Management (including allowed indication, policy and offload identifiers),

-	enabling the anchor SMF to send PCO to the UE, and

-	not using PLMN ID in Neasdf_DNSContext_Create Request from I-SMF to retrieve EASDF IP address.

End of CHANGES