SA WG2 Meeting #166 Ad hoc	S2-2501069

# 20 - 24 January 2025, e-meeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | - | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 – update to VFL training procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solving multiple Editor’s Notes. - Editor's note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS. VFL training requires many iterations of processes including local training, collecting intermediate result, and ML model update. In order to differentiate starting of VFL training from ongoing VFL training process, VFL Training Start Flag is needed. Similarly, after completing VFL training, VFL training termination flag is needed to differentiate successful completion and unsuccessful interruption in VFL training (VFL Training Completion and VFL Training Failure.) - Editor's note: It is FFS whether sample/feature information is required to be provided or updated in each training. Sample/feature information is negotiated during VFL preparation phase and sample and feature information are finally decided by VFL server. During VFL preparation phase, there is no step of notification of decision by VFL server to VFL client. Therefore, the determined sample set, feature information and list of VFL clients needs to be shared at initiation of VFL training procedure. VFL clients may support multiple Feature IDs for an interoperable VFL operation of an Analytic ID, VFL server may need to inform the decided Feature ID to each VFL client. - Editor's note: Whether and how to include interoperability information in the VFL training procedure is FFS. For an Analytic ID, there may be multiple VFL operation methods and for each VFL operation method, there may be multiple interoperable set possible. And a NWDAF as VFL client may support multiple interoperable set and multiple VFL operation. Therefore, for clarify of operation interoperability information may need to be shared at initial step of VFL training process. - Editor's note: Whether and how to define the trigger of VFL training is FFS. In registration and discovery phase, it is clarified that it may be triggered by service consumer or NWDAF with MTLF itself. - Editor's note: Whether and how to transfer the confirmation in VFL Preparation Phase at the beginning of VFL Training Phase is FFS. As explained above, the determined sample set, feature information and list of VFL clients needs to be shared at initiation of VFL training procedure. - Editor's note: Whether the parameters negotiated in the preparation phase are provided at the end of the preparation phase or at the start of the training is FFS. During VFL preparation phase, there is no step of notification of decision by VFL server to VFL client. Therefore, the determined sample set, feature information and list of VFL clients needs to be shared at initiation of VFL training procedure. - Editor's note: Additional Parameters to be provided in the request are FFS. For initial VFL operation, VFL server may send initialization parameter for local VFL training to VFL clients. Additional Parameter may include initialization parameters for VFL operation(e.g., initial model parameters, hyperparameters, etc.) - Editor's note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process. For each interoperability set, supported Featured IDs are configured in each VFL client. Therefore, it is reasonable to assume that each VFL client is preconfigured with local ML Model for a supported Feature IDs for each interoperability set of an Analytic ID. - Editor's note: It is FFS and may depend on the service design: When the clients report the client intermediate training result, it also includes the corresponding VFL correlation ID. It will be implementation specific when to report client’s intermediate training result once it is performed within the delay threshold suggested by VFL server. - Editor's note: Whether and how to exchange intermediate training result among VFL Clients is FFS. It is proposed for this release intermediate training result is shared only between VFL server and VFL client. - Editor's note: Whether weight of the VFL Client is computed by VFL server is FFS. Whether weight of the VFL client is computed by VFL server is upto the designed VFL operation and it is implementation specific issue. And it does not need to be shown or shared to VFL clients. Therefore, 3GPP does not need to consider weight of the VFL Client during VFL training procedure between VFL server and VFL client. - Editor's note: Whether VFL server and VFL clients share feature information is FFS. Feature information only needs to be shared at initial step. Ongoing VFL training process associate to a VFL correlation ID needs to use same feature IDs. Therefore, it is not needed to share feature information during ongoing VFL training process. - Editor's note: The content of the VFL status report is FFS. - Editor's note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS. As ongoing VFL training procedure and VFL termination procedure are different, during ongoing VFL training procedure, VFL server only needs to inform the updated local ML Model information for next round of VFL training to VFL clients. Therefore VFL status report or VFL convergence report is not needed but it is needed to share information for update VFL training such as model parameter, loss information, etc. - Editor's note: Whether the ML model metric (e.g. ML model accuracy) defined for HFL can be re-applied to VFL is FFS. The difference of HFL and VFL is whether same features may be utilized for ML model training or not, the evaluation metrics may be reused. - Editor's note: Whether VFL Training termination Flag in the termination message is required is determined after settling down the service operation. VFL Training termination procedure needs to include successful or failure. Based on the indication, step 9 can be performed or skipped. In addition, NEF may store the mapping of NWDAF temporary ID when successful completion only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR addresses a number of Editor’s Notes related to missing parameters and incomplete procedures, for the correct execution of VFL training. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training procedure will not implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.1, 6.2H.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.2H.2.3	Training Procedure for Vertical Federated Learning

##### 6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF/trusted AF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1: Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor's note:	How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE 1: 	VFL Server can be triggered the VFL training by request of consumer or by itself and determine to start the training based on local configuration and agreement among vendors and/or application providers participating in the same group for specific VFL task(s).

Steps 2-6 are repeated until the training termination condition is reached.

2.	To start the VFL training, the VFL server sends a request to start the training to all selected VFL clients. The request includes VFL correlation ID, VFL Training Start Flag, the parameters negotiated during the preparation phase (Analytic ID, selected VFL interoperability information, selected sample set, determined feature ID for each VFL client associated to the selected VFL interoperability information, etc). Optionally, the VFL Server includes: Analytic filter information, maximum response time (i.e. the maximum time between VFL clients receive intermediate model training information and send back intermediate training result). VFL server may include Additional parameter as initialization parameter for local VFL training to VFL clients (e.g., initial model parameters, hyperparameters, the label for local VFL training (if needed), etc.).

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the VFL correlation ID and Additional parameter as intermediate model training information to each of the VFL clients for next round of VFL training. Intermediate model training information may include the updated model parameters and loss information or other performance metrics for local ML model.

2a.	The VFL server sends a Nnwdaf_VFLTraining_Subscribe to the selected NWDAF VFL clients(s).

2b.	The VFL server sends a Naf_VFLTraining_Subscribe to the selected trusted AF VFL clients(s).

2c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Subscribe to the NEF handling that AF.

2d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Subscribe to that AF. The NEF may also translate the analytic filter information if needed, e.g. TAIs into geographical area.

NOTE 2:	In this release, the same NF associated with a VFL Server or VFL Client capability during the VFL training for a VFL correlation ID is also the same NF during the VFL inference.

NOTE 3: In this release, it is assumed that each VFL client is preconfigured with local ML model for a supported feature ID for each interoperability set of an Analytic ID.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g. after the first round of training) and possible intermediate model training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of the local ML model to the VFL server.

NOTE 3:	The intermediate model training information and intermediate training result are constructed in per sample granularity.

NOTE 4: It is implementation specific when to report client’s intermediate model training result once it is reported within the maximum response time received in step 2.

NOTE 5: In this release, it is assumed that intermediate model training result is shared between VFL server and VFL client.

5.	Each VFL client reports the computed client intermediate training result of the local ML model to the VFL server.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	A trusted AF VFL client sends a Naf_VFLTraining_Notify to the VFL server.

5c.	An untrusted AF VFL client sends a Naf_VFLTraining_Notify to the NEF.

5d.	For each untrusted AF VFL client , the NEF converts any external identifiers to internal identifiers and sends a Nnef_VFLTraining_AFClient_Notify to the VFL server.

6.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the intermediate model training information (e.g. gradient information or loss information) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The intermediate model training information is used for updating the models of VFL clients. Different intermediate model training information may be computed for different VFL clients and for the VFL Server itself.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

7.	[Optional] The NWDAF acting as VFL server evaluates (e.g. based on the convergence of a loss function or loss value and/or if the pre-set iteration number is reached) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6.

If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 8.

For various reason (e.g., VFL process does not converge after further iteration, by request of the consumer), VFL server may determine the VFL training process fails. In this case, the VFL Server terminates the current VFL training process with indication of VFL training failure via step 8.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include model metric (e.g. ML model accuracy).

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

8.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process, the termination message contains VFL Correlation ID and VFL Termination flag which indicate VFL training completion or VFL training failure.

8a.	The VFL server sends a Nnwdaf_VFLTraining_Unsubscribe t to the selected NWDAF VFL clients(s).

8b.	The VFL server sends a Naf_VFLTraining_Unsubscribe to the selected trusted AF VFL clients(s).

8c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Unubscribe to the NEF handling that AF.

8d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Unsubscribe to that AF.

9.	When VFL training successfully completes, the VFL Server, stores VFL correlation ID, the local trained ML Model, the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, locally trained Model. Additionally, the VFL server stores the VFL client information, which may be used to determine associated VFL client in the VFL inference.

When VFL training successfully completes, each VFL client stores VFL correlation ID, the locally trained ML Model, the mapping information of the VFL correlation ID to locally trained Model.

When VFL training failure is indicated, VFL correlation ID is removed at VFL server and VFL client.

NOTE 4:	The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

NOTE 5:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

##### 6.2H.2.3.2	Training Procedure for Vertical Federated Learning untrusted AF is acting as VFL server

Figure 6.2H.2.3.2-1: Training procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

Editor's note:	The ENs listed in clause 6.2H.2.3.1 are also applied to this clause.

1.	Same as step 1 in Figure 6.2H.2.3.1-1.

2.	To start VFL training, the VFL server do same as in step 1 in Figure 6.2H.2.3.1-1, using Nnef_VFLTraining_Subcribe.

Steps 3-7 are repeated until the training termination condition is reached.

3.	[Optional] Same as step 3 in Figure 6.2H.2.3.1-1.

4.	Same as step 4 in Figure 6.2H.2.3.1-1.

5.	Same as step 5 in Figure 6.2H.2.3.1-1.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	For an untrusted AF acting as VFL server, the NEF converts any internal identifiers to external identifiers, provides the external NWDAF ID and sends a Nnef_VFLTrainingNotify to the VFL server.

6.	[Optional] Same as step 6 in Figure 6.2H.2.3.1-1.

7.	Same as step 7 in Figure 6.2H.2.3.1-1.

8.	Same as step 8 of Figure 6.2H.2.3.1-1. However, sub steps in that figure are not applicable.

8a.	For each NWDAF VFL client, the untrusted AF as VFL server sends a Nnef_VFLTraining Unsubscribe to the NEF handling that AF. The untrusted AF identifies the VFL client using the external NWDAF ID assigned in the discovery procedure (see clause 6.2H.2.1.1).

8b.	The NEF sends an Nnwdaf_VFLTraining_Unsubscribe to the NWDAF VFL client indicated by the received external NWDAF ID.

9.	Same as step 9 of Figure 6.2H.2.3.1-1. When VFL training failure is indicated, NEF removes VFL correlation ID and external NWDAF IDs associated.

* * * End of Changes * * *