3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2501217

E-meeting, 20 – 24 January 2025

Title:	LS on energy information collection from OAM to EIF

Response to:	-

Release:	Release 19

Work Item:	EnergySys

Source:	SA2

To:	SA5

Cc: 	-

# Contact Person:

# Name:	Zhuoyun Zhang

# Email:		zhangzhuoyun AT vivo DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

As part of the Rel-19 EnergySys work, SA2 has defined the new network function EIF (Energy Information Function) for UE related energy consumption information collection, calculation and exposure. SA2 specification is based on the following assumptions:

- The EIF requests to the OAM for the gNB energy consumption, gNB data volume, UPF energy consumption, and UPF data volume, by providing the corresponding gNB ID(s) and (I-)UPF ID(s).

- Currently SA2 assumed that the above data is measured on a period of time T which is configured PLMN-wide, i.e., for the measurement performed by OAM at NF level and by UPF at UE, PDU session and QoS flow level. Furthermore, it is assumed that the measurement time (i.e., the starting of measurement periods) is synchronized for the whole PLMN (i.e., for OAM for measurement at NF level and SMF for measurement at UE, PDU session and Application level).

- The data provided by OAM is associated with the time which the measurement refers to (i.e., a timestamp or an equivalent information).

- In the current version of specification, only the Energy Consumption is considered. SA2 has also discussed the renewable energy information collection and calculation per UE, PDU Session and/or QoS flow granularity. However, whether it would be supported in Rel-19 depends on whether the Node-level renewable energy information is exposed by OAM system.

In addition, SA2 has the following questions regarding OAM:

Question 1: SA2 would like SA5 to provide feedback on whether and what the Node-level renewable energy information can be collected from OAM.

Question 2: In order to realize the aforementioned reporting mechanism, which SA5 specifications should the SA2 specs refer to?

Question 3: Could OAM provide information on how the energy related information reporting is assumed to be performed from SA5 point of view, e.g., periodically, on demand, via management object, via file, etc?

# 2. Actions:

# To SA5:

ACTION: SA2 kindly asks SA5 to provide feedback as requested above.

# 3. Date of Next SA2 Meetings:

TSG-SA2#167	  17th Feb – 21st Feb 2025				Athens, Greece

TSG-SA2#168	  7th  Apr – 11th  Apr 2025		         	Goteborg, Sweden