3GPP TSG-WG2 Meeting #166-AdHoc-e	S2-2501297

E-meeting, 20 – 24 January 2025	 was S2-2411480

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0490 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Path Failure Detection for Relay Reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Qualcomm Inc., AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As described in TS 23.304 clause 6.4.3.8.3, relay reselection may be triggered by 5G ProSe Remote UE due to the connection is lost or the link quality drop. The detail content about path failure detection is missing. The path failure (connection lost or link quality drop) may be detected by lower layer (e.g. PC5-RRC layer). The lower layer (e.g. PC5-RRC layer) indicates to upper layer (e.g. PC5-S signaling layer) about the dectected failure. This paper proposes to add content about path failure detection to complete the relay reselection for multi-hop UE-to-Network relays, and complete the term "5G ProSe Remote UE" in figure 4.2.7.1A-1 to align with the corresponing content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added descriptions for path failure detection to clause 6.4.3.8.3, in detail lower layer (e.g. PC5-RRC layer) detects and indicates to upper layer (PC5-S signaling layer) about the detected failure. - Added "Remote UE" after "5G ProSe" in figure 4.2.7.1A-1 to compete the term. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The path failure detection is missing, the content of relay reselection for multi-hop UE-to-Network relays is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.1A, 6.4.3.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

#### 4.2.7.1A	5G ProSe Multi-hop Layer-3 UE-to-Network Relay reference architecture

Figure 4.2.7.1A-1 shows the high level reference architecture for 5G ProSe multi-hop Layer-3 UE-to-Network Relay. There may be one or multiple 5G ProSe Intermediate UE-to-Network Relays between the 5G Prose Remote UE and the 5G ProSe UE-to-Network Relay. In this figure, the 5G ProSe Layer-3 UE-to-Network Relay may be in the HPLMN or a VPLMN.

Figure 4.2.7.1A-1: Reference architecture for 5G ProSe multi-hop Layer-3 UE-to-Network Relay

>>>> Start of Changes <<<<

##### 6.4.3.8.3	Relay reselection and mobility support

There are two types of situations for parent relay reselection, i.e. for a single 5G ProSe Remote UE and for a 5G ProSe Intermediate UE-to-Network Relay with multiple child relays or 5G ProSe Remote UEs.

For a single 5G ProSe Remote UE, when the connection with an old parent relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold, the 5G ProSe Remote UE triggers the reselection of the relays that offers the same service, identified by the same RSC. The path failure (connection lost or link quality drop) may be detected by a lower layer (e.g. PC5-RRC layer). The lower layer (e.g. PC5-RRC layer) indicates to an upper layer (e.g. PC5-S signalling layer) about the detected failure, together with the information on the detected failure/event, and the PC5 Link Identifier for the unicast link (which can be used to retrieve the corresponding UE ID(s) from the link context).

Figure 6.4.3.8.3-1: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays

Figure 6.4.3.8.3-1 depicts the Relay reselection operation for this situation. When the 5G ProSe Remote UE has lost the connection or has an imminent connection loss/deteriorated connection with the 5G ProSe Intermediate UE-to-Network Relay-1, the 5G ProSe Remote UE performs relay reselection by using the relay discovery procedure in clause 6.3.2.5. The 5G ProSe Remote UE discovered the 5G ProSe Intermediate UE-to-Network Relay-2 that offers the same RSC. In case there are multiple 5G ProSe Intermediate UE-to-Network Relays discovered, the 5G ProSe Intermediate UE-to-Network Relay that offers connection to the same 5G ProSe UE-to-Network Relay is prioritized.

5G ProSe Remote UE establishes the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2 as specified in clause 6.4.3.8.1. After the connection establishment, the Intermediate UE-to-Network Relay-2 uses the Layer-2 Link Modification procedure to update the 5G ProSe UE-to-Network Relay regarding the 5G ProSe Remote UE. If the same 5G ProSe UE-to-Network Relay is used, it switches the connection/routing context for the 5G ProSe Remote UE from the Layer-2 link with the 5G Intermediate UE-to-Network Relay-1 to that of the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2.

The 5G ProSe Remote UE tears down the link with the 5G ProSe Intermediate UE-to-Network Relay-1 (or the link failure is detected), which will trigger the 5G ProSe Intermediate UE-to-Network Relay-1 to update the 5G ProSe UE-to-Network Relay to remove its association with 5G ProSe Remote UE.

If the same 5G ProSe UE-to-Network Relay is selected after the relay reselection, the 5G ProSe Remote UE can keep its IP address/prefix (in case of Layer-3 UE-to-Network relay).

If a new 5G ProSe UE-to-Network Relay is used after the relay reselection, the procedures as that described in clause 6.4.3.8.1 will be executed. In this case, IP address/prefix changes may happen (in case of Layer-3 UE-to-Network relay).

The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure.

>>>> End of Changes <<<<