3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500132

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0637 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Considering RAN load when collecting data to train model for positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Open issues and ENs related to whether the LMF can consider RAN load when performing data collection to train models. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The LMF may subscribe at an NWDAF for Network Performance Analytics to select times for data collection when NG-RAN is not experiencing extensive load. The LMF may also subscribe at the AMF to RAN overload information in the area where data are to be collected and refrain from data collection when being notified of a RAN overload. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Open issues remain. LMF may cause RAN congestion by collecting input data for model training at inappropriate times. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.2, 6.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.22.2	Data Collection at LMF for the LMF-based AI/ML positioning based on UE measurements

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training, performance monitoring and may also request the UE to provide the UE location. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.22.2-1: Data collection at LMF for the LMF-based AI/ML positioning based on UE measurements

- The LMF determines that data collection is required for the purpose to train an AI/ML Model for UE Positioning (this may be based on an internal trigger in the LMF or a request from the NWDAF). The LMF may subscribe at an NWDAF for Network Performance Analytics to select times for data collection when NG-RAN is not experiencing extensive load.

- The LMF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover AMF(s) that served for the area of interest.

3.	The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest and the UE Positioning Capability for each UE using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest" and "UE Positioning Capability". The LMF may also subscribe at the AMF to RAN overload information in the area where data are to be collected and refrain from data collection when being notified of a RAN overload.

Editor's note:	Whether and how to limit the number of UEs is FFS.

4.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest"). If "UE Positioning Capability" is also requested in step 1, AMF includes UE Positing Capability, UE User Plane Positioning Capabilities if available for each SUPI in the response message sent to LMF.

5.	The LMF further determines the UEs from the list of SUPIs that received from AMF in step 2 for data collection based on UE Positioning Capability, UE User Plane Positioning Capabilities, UE consent check result, the PRU information available in the LMF and operator's policy. The User consent check is not needed for data collection from PRU for performance monitoring.

Editor's note:	Further details on user consent for data collection for model training or model performance monitoring for LMF-based AI/ML Model positioning will be aligned with SA WG3.

6.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data. The UE may reject the data collection request from the LMF (e.g. considering UE status, user's input). If the UE accepts data collection request, the UE may cancel the data collection later as defined in clause 6.3.4.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE.

2nd change

### 6.22.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

- 1.	The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs or to monitor the AI/ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1. The LMF may subscribe at an NWDAF for Network Performance Analytics to select times for data collection when NG-RAN is not experiencing extensive load.

2.	The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model using a PRU, which are associated to this LMF that executes data collection, the LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF in order to identify the LMF(s) which has PRU associated to a list of TAIs. The LMF may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs in order to get the data collection and the known PRU location in this case step 6 follows.

If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest.

3.	For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest". The LMF may also subscribe at the AMF to RAN overload information in the area where data are to be collected and refrain from data collection when being notified of a RAN overload.

4.	The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed.

5.	The LMF checks whether the SUPI provided user consent for data collection for the purpose of training with UDM. If user consent is provided, then step 6 follows. if user consent is not provided, the LMF does not perform step 6 for this UE, that will not be part of the data collection, and its data is not used for the LMF-based AI/ML Model positioning.

Editor's note:	Further details on user consent for data collection for model training or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3.

6.	The LMF request input data for the target UE from NG-RAN.

7.	The LMF receives ground truth data from the UE.

The measurements and ground truth data are used for training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 8 and 9 may occur in parallel for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

End of changes