3GPP TSG-SA WG2#166-AdHoc-e	S2-2500133

Electronic Meeting, 20-24 January 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	Emergency call back and “delayed” paging

Document for:	Discussion/Approval

Agenda Item:	19.50

Work Item / Release:	TEI19/Rel-19

Abstract of the contribution: Analyses the interaction of emergency call back and delayed paging features.

# 1.	Discussion

SA2 in recent discussions to draft alignment CRs for the “LP-WUS” feature in SA2#166 captured the following Editor’s Note in TS 23.501:

Editor's note:	The use of LP-WUSPS for emergency sessions and emergency call back is FFS.

In SA2#166AHE SA2 received LS R2-2411041/S2-2500044 from RAN2 “LS on emergency services and eDRX” that is related also to how to handle emergency call back, in this case for UE in RRC_INACTIVE state with eDRX.

In this paper we are going to analyse the right approach for handling emergency callback for UE configured with features that may impact/delay the received paging.

# 2.	Background

# First some background for emergency callback. Emergency call back as defined in TS 23.167 and based on service requirements defined in TS 22.101 is treated as “normal call” based on “local regulation”.

Extract from TS 23.167 cl. 4.1:

12.	The architecture shall enable emergency centres and PSAPs to request a PSAP call back to a UE with which the Emergency centres or PSAPs had an emergency session. The serving network of the UE shall use the appropriate call termination procedures e.g. IMS if the UE is available for voice over PS, or ICS if the user is available over CS. PSAP call back is subject to local regulation.

NOTE 3:	PSAP call back sessions are treated as normal calls.

NOTE 4:	Subject to local regulation, any supported media can be used during a call back attempt from a PSAP.

Extract from TS 22.101 cl. 10.1.3:

### 10.1.3	Call-Back Requirements

Subject to local/regional regulations the network shall support a call-back from a PSAP.

It shall be possible to supply the user's Directory Number/MSISDN/SIP URI as the CLI to the PSAP to facilitate call-back. The CLI used on call-back shall allow the PSAP to contact the same terminal that originated the emergency call.

If the incoming call can be identified by the core network as a call-back to an emergency call (i.e. coming from a PSAP) then supplementary services at the terminating party shall be handled as described in TS 22.173 [40] for Multimedia Telephony (e.g. Communication Diversion, Communication Hold, Communication Barring).

NOTE: There is no specific callback requirement for CS supplementary services.

A call-back may be attempted for a period of time defined by local regulations after the emergency call release. In case of a UE in limited service state, call-back is not required.

Observation 1: The definition of “normal call” for the case of IMS we can assume is:

- there is no support for emergency call back when the UE is in limited service

- therefore the UE has to have active IMS PDU session and normal IMS registration to deliver the call

- in case of roaming such IMS PDU session in most cases will terminate in HPLMN of the UE

- there is no priority treatment e.g. special ARP, 5QI, RRC establishment cause that can allow the 5GS (CN and RAN) to distinguish between emergency call back and any other mobile terminated call

Since rel.12 though when 3GPP introduced for first time extended DRX for idle mode in EPS, text was added in TS 23.401 for the UE to disable extended DRX when it has active bearers for emergency services:

When the UE has bearers for emergency bearer services, the UE and MME follow regular discontinuous reception as defined in clause 5.13 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has bearers for emergency bearer services. When the bearers for emergency bearer services are released, the UE and MME shall reuse the negotiated extended idle mode DRX parameters in the last TAU/Attach procedure.

Similar text was added to TS 23.501 when extended DRX was introduced in 5GS

When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure.

It is a question what is the use case this text was intending to cover? Given what is mentioned in Observation 1, it will not cover emergency call back since it will not be delivered over the emergency PDU session and is handled as “normal call”. Nevertheless, when “Paging Early Indication with Paging Subgrouping Assistance” (PEIPS) was introduced similar text was added:

When the UE has an active emergency PDU Session:

-	The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

And since the design of LP-WUS follows PEIPS the aforementioned Editor’s Note was added.

Observation 2: Current specifications require the UE to “disable” idle mode extended DRX and not signal Paging Subgrouping Support Indication when it has active PDU session for emergency services.

# 3. 	Next steps

Given observations 1 and 2, whether it makes sense (adds any benefit) to disable features that may impact paging (eDRX, PEIPS, LP-WUS) will depend on whether there is possibility of sending MT signalling within the IMS PDU session for reasons other than emergency call back. For example it may be possible in certain cases where SIP timers expire and the UE to receive SIP signalling as part of the MO emergency session after the RRC inactivity timer expires and the UE turns into CM-IDLE.

Observation 3: the possible benefit to disable features that may impact paging (eDRX, PEIPS, LP-WUS) will depend on whether there is possibility of sending MT signalling within the IMS PDU session for reasons other than emergency call back.

It is therefore proposed that SA2 checks with CT1 whether they foresee any scenarios for the UE to receive SIP signalling as part of the MO emergency session after the RRC inactivity timer expires and the UE turns into CM-IDLE.

Proposal 1: SA2 checks with CT1 whether they foresee any scenarios for the UE to receive SIP signalling as part of the MO emergency session after the RRC inactivity timer expires and the UE turns into CM-IDLE.

Depending on the answer to such LS, SA2 can determine how to handle all features that impact paging (eDRX in CM-IDLE and RRC_INACTIVE, PEIPS, LP-WUS). If the CT1 answer is positive then SA2 can keep the existing text in TS 23.501 in clauses of eDRX and PEIPS and remove the EN and add similar text for the case of LP-WUS. If the CT1 answer is negative then there is no need for such normative text but since this text exists since many releases ago, in order to avoid backwards compatibility it can be converted to a NOTE or become optional normative text.

# 4.	How to handle LS R2-2411041“ LS on emergency services and eDRX”

The LS R2-2411041“ LS on emergency services and eDRX” relates also to the analysis in previous clauses of this paper. Namely, when the UE is RRC_INACTIVE state with extended DRX it can only receive emergency call back over IMS PDU session (see observation 1). It is therefore proposed to postpone the incoming LS R2-2411041 and answer it accordingly after receiving response from CT1 and deciding on the overall handling explained in clause 3.

Proposal 2: Answer to the RAN2 LS on emergency services and eDRX (R2-2411041) should be postponed until CT1 answers the question described in Proposal 1.

# 5.	Conclusions

The following proposals are requested to be agreed:

Proposal 1: SA2 checks with CT1 whether they foresee any scenarios for the UE to receive SIP signalling as part of the MO emergency session after the RRC inactivity timer expires and the UE turns into CM-IDLE.

Proposal 2: Answer to the RAN2 LS on emergency services and eDRX (R2-2411041) should be postponed until CT1 answers the question described in Proposal 1.

LS to CT1 as per Proposal 1 is drafted in S2-2500134.